PSI 
Graphic Transport Framework User Manual

http://aea.web.psi.ch/Urs_Rohrer/MyWeb/trans.htm

(Draft)

rewritten by S. Sinigardi and M. Camatti,

based on the original manual by Urs Rohrer

v0.8 – July 2012

If you would like to contribute any correction to this manual, please mail me

sinigardi@bo.infn.it

INTRODUCTION

TRANSPORT is a first— and second—order matrix multiplication computer

program intended for the design of static-magnetic beam transport systems.

It has been in existence in various evolutionary versions since 1963. The

present version, described in this manual, includes both first— and second

order fitting capabilities.

Many people from various laboratories around the world have contributed either directly or indirectly to the development of TRANSPORT. The first—order matrix methods were developed by the AGS machine theorists followed by a paper by Penner. The extension of the first—order matrix methods to include second and higher orders was conceived and developed by Brown, Belbeoch and Bounin in Orsay, France, in 1958-59. The original first—order TRANSPORT computer program was written in BALGOL by C.H. Moore at SLAC in collaboration with H.S . Butler and S.K. Howry in

1963. The second—order portion of the program was developed and debugged by Howry and Brown , also in BALGOL. The resulting BALGOL version was

translated into FORTRAN by S . Kowalski at MIT and later debugged and improved by Kear, Howry and Brown at SLAC. In 1971-72, D. Carey at FNAL

completely rewrote the program and developed an efficient second—order

fitting routine using the coupling coefficient (partial derivatives) of multipole components to the optics as derived by Browns). This version

was implemented at SLAC by F. Rothacker in the early spring of 1972 and

subsequently carried to CERN in April, 1972, by K.L. Brown.

C. Iselin of CERN made further contributions to the program structure and improved the convergence capabilities of the first—order fitting routines.

A standard version of the resulting program has now been adopted at

SLAC, FNAL, and CERN. This manual describes the use of this standard

version and is not necessarily applicable to other versions of TRANSPORT.

Copies of this manual may be obtained from

1) Scientific Information Service, CERN, 1211 Geneva 23, Switzerland (Ref. CERN 80-04)

2) The Reports Office, Stanford Linear Accelerator Center, P.O. Box 4349,

Stanford, CA 94305, USA (Ref. SLAC—91, without the Appendix).

3) The Reports Office, Fermi National Accelerator Laboratory, P.O. Box

500, Batavia, IL 60510, USA (Ref. NAL—91, "TRANSPORT Appendix" available under separate cover)

The program may be obtained from:

1) IBM Version:

Frank Rothacker

TRANSPORT Program Librarian

Mail Bin 88

Stanford Linear Accelerator Center

P.O. Box 4349

Stanford, CA 94305, USA

2) IBM, CDC or PDPlO Versions:

David C . Carey

Fermi National Accelerator Laboratory

P.O. Box 500

Batavia, IL 60510, USA

3) IBM or CDC Vers ions :

Program Library

Division DD

CERN

CH 1211 Genéve 23

Switzerland

The present authors assume responsibility for the contents of this

manual, but in no way imply that they are solely responsible for the

entire evolution of the program.

In order to make this report available without delay, the Appendix

has been reproduced directly as published by FNAL.

MATHEMATICAL FORMULATION OF TRANSPORT

General conventions

A beam line is comprised of a set of magnetic elements placed sequentially at intervals along an assumed reference trajectory. The reference trajectory is here taken to be a path of a charged particle passing through idealized magnets (no fringing fields) and having the central design momentum of the beam line.

In TRANSPORT, a beam line is described as a sequence of elements.

Such elements may consist not only of magnets and the intervals between

them, but also of specifications of the input beam, calculations to be done ,

or special configurations of the magnets. A certain relation, described

below, of the magnets and their fields to the assumed reference trajectory

is considered normal. Alternative configurations can be described by

means of elements provided for such purposes.

The two coordinates transverse to the initial reference trajectory

are labeled as horizontal and vertical. A bending magnet will normally

bend in the horizontal plane. To allow for other possibilities a coordinate rotation element is provided. Because of such other possibilities,

when describing bending magnets we shall often speak of the bend and non

bend planes. The transverse coordinates will also often be labeled x

and y, while the longitudinal coordinate will be labeled z.

All magnets are normally considered "aligned" on the central trajectory. A particle following the central trajectory through a magnet experiences a uniform field which begins and ends abruptly at the entrance and exit faces of the idealized magnet. Therefore, through a bending magnet the reference trajectory is the arc of a circle , while through all other magnetic elements it is a straight line . To accommodate a more gradual variation of field at the ends of a bending magnet a fringing field element is provided. In order to represent an orientation with respect to the reference trajectory other than normal of a magnet or

section of a beam line, a misalignment element also exists.

The magnetic field of any magnet, except a solenoid, is assumed to

have midplane symmetry. This means that the scalar potential expanded

in transverse coordinates about the reference trajectory is taken to be

an odd function of the vertical coordinate. If a coordinate rotation is

included, then the potential is odd in the coordinate to which the vertical

has been rotated. For a bending magnet this will always be in the non-bend plane.

The program TRANSPORT will step through the beam line, element by

element, calculating the properties of the beam or other quantities, described below, where requested. Therefore one of the first elements is a

specification of the phase space region occupied by the beam entering the

system. Magnets and intervening spaces and other elements then follow

in the sequence in which they occur in the beam line . Specifications of

calculations to be done or of configurations other than normal are placed

in the same sequence, at the point where their effect is to be made.

The transfer matrix R

The following of a charged particle through a system of magnetic

lenses may be reduced to a process of matrix multiplication. At any specified position in the system an arbitrary charged particle is represented

by a vector (single column matrix) X, whose components are the positions,

angles, and momentum of the particle with respect to the reference trajectory, i.e.

X=xθyϕlδ

Definitions:

x = the horizontal displacement of the arbitrary ray with respect

to the assumed central trajectory.

θ = the angle this ray makes in the horizontal plane with respect

to the assumed central trajectory.

y = the vertical displacement of the ray with respect to the

assumed central trajectory.

ϕ = the vertical angle of the ray with respect to the assumed

central trajectory.

ℓ = the path length difference between the arbitrary ray and the

central trajectory.

δ = Δp/p is the fractional momentum deviation of the ray from the

assumed central trajectory.

This vector, for a given particle, will henceforth be referred to as

a ray. The magnetic lens is represented to first order by a square

matrix R, which describes the action of the magnet on the particle coordinates. Thus the passage of a charged particle through the system may be

represented by the equation

| X1=RX0 | (1) |
| --- | --- |

where X(O) is the initial coordinate vector and X(l) is the final coordinate vector of the particle under consideration. The same transformation

matrix R is used for all such particles traversing a given magnet [one

particle differing from another only by its initial coordinate vector X(O)].

The traversing of several magnets and interspersing drift spaces is

described by the same basic equation, but with R now being replaced by the

product matrix R(t) = R(n) . . . R(3)R(2)R(l) of the individual matrices of

the system elements. This cumulative transfer matrix is automatically

calculated by the program and is called TRANSFORM 1. It may be printed

where desired, as described in later sections.

This formalism may be extended to second order by the addition of

another term. The components of the final coordinate vector, in terms

of the original, are now given as

| Xi1=jRijXj0+ jkTijkXj0Xk(0) | (2) |
| --- | --- |

where T is the second—order transfer matrix. It too is accumulated by the

program as one traverses a series of elements. At each point the series

is again truncated to second order. Normally the program will calculate

only the first—order terms and their effect. If it is desired to include

second—order effects in a beam line , an element is provided which specifies

that a second—order calculation is to be done. For more information on

the T matrix, see the references at the end of the manual.

The following of a charged particle via TRANSPORT through a system

of magnets is thus analogous to tracing rays through a system of optical

lenses. The difference is that TRANSPORT is a matrix calculation which

truncates the problem to either first— or second—order in a Taylor's

expansion about a central trajectory. For studying beam optics to greater

precision than a second-order TRANSPORT calculation permits, ray-tracing programs which directly integrate the basic differential equation of motion

are recommended.

The beam matrix σ

In accelerator and beam transport systems , the behavior of an individual

particle is often of less concern than is the behavior of a bundle

of particles (the beam), of which an individual particle is a member. An extension of the matrix algebra of Eq. (1) provides a convenient means for

defining and manipulating this beam. TRANSPORT assumes that the beam may

be correctly represented in phase space by an ellipsoid in the six

dimensional coordinate system described above. Particles in a beam are

assumed to occupy the volume enclosed by the ellipsoid, each point re

presenting a possible ray. The sum total of all phase points , the phase

space volume , is commonly referred to as the "phase space" occupied by

the beam.

The validity and interpretation of this phase ellipse formalism

must be ascertained for each system being designed. However, in general ,

for charged particle beams in, or emanating from accelerators , the firs t

order phase ellipse formalism of TRANSPORT is a reasonable representation

of physical reality. For other applications, such as charged particle

spectrometers, caution is in order in its use and interpretation.

The equation of an n—dimensional ellipsoid may be written in matrix

form as follows:

| X0Tσ0-1X0=1 | (2) |
| --- | --- |

where X(O)T is the transpose of the coordinate vector X(O), and σ(0) is

a real, positive definite, symmetric matrix.

The volume of the n-dimensional ellipsoid defined by sigma is

πn2Γn2+1detσ12.

The area of the projection in one plane is

A=πdetσ112, where σ1 is the submatrix corresponding to the given

plane. This is the "phase space" occupied by the beam.

As a particle passes through a system of magnets, it undergoes the

matrix transformation of Eq. (1). Combining this transformation with the equation of the initial ellipsoid, and using the identity RR-1=I (the unity matrix) , it follows that

from which we derive

The equation of the new ellipsoid after the transformation becomes

where

It can readily be shown that the square roots of the diagonal terms

of the sigma matrix are a measure of the "beam size" in each coordinate.

The off-diagonal terms determine the orientation of the ellipsoid in n—dimensiona1 space (for TRANSPORT n = 6). Thus , we may specify the

beam at any point in the system via Eq. (5) , given the initial "phase

space" represented by the matrix elements of σ(0).

The initial beam is specified by the user as one of the first elements of the beam line. Normally it is taken to be an upright ellipse

centered on the reference trajectory; that is, there are no correlations

between coordinates. Both correlations and centroid displacements may

be introduced via additional elements.

The phase ellipse may be printed wherever desired. For an interpretation of the parameters printed see the section under type code 1.0.

When a second—order calculation is specified, the second-order matrix

elements are included in the beam matrix. For details on how this is done

see the Appendix to this manual.

Fitting

Several types of physical elements have been incorporated in the program to facilitate the design of very general beam transport systems. Included are an arbitrary drift distance, bending magnets, quadrupoles, sextupoles, solenoids, and an accelerator section (to first-order only).

Provision is made in the program to vary some of the physical parameters

of the elements comprising the system and to impose various constraints on the beam design. In a first—order run one may fit either the

TRANSFORM (R) matrix representing the transformation of an arbitrary ray through the system and/or the phase ellipse (sigma) matrix representing

a bundle of rays by the system as transformed. In a second-order run one

may fit either the second—order TRANSFORM (T) matrix or minimize the net

contribution of second-order terms to the beam (sigma) matrix.

The program will normally make a run through the beam line using

values for the physical parameters as specified by the user and printing

the results . If constraints and parameters to be varied are indicated,

it will attempt to fit. To do this it will make an additional series of

runs through the beam line. Each time it will calculate corrections to

be made from the previous step to the varied parameters to try to satisfy

the indicated constraints. When the constraints are satisfied (or the

fitting procedure has failed) the program will make a final run through

the beam line again printing the results. In this final run the values

of the physical parameters used are those which are the result of the

fitting procedure.

Thus, in principle, the program is capable of searching for and

finding the first- or second—order solution to any physically realizable

problem. In practice, life is not quite so simple. The user will find

that an adequate knowledge of geometric magnetic optics principles is a

necessary prerequisite to the successful use of TRANSPORT. He (or she)

should possess a thorough understanding of the first—order matrix algebra

of beam transport optics and of the physical interpretation of the various

matrix elements .

In other words, the program is superb at doing the numerical calculations for the problem but not the physics. The user must provide a

reasonable physical input if he (or she) expects complete satisfaction

from the program. For this reason a list of pertinent reprints and

references are included at the end of this manual. They should provide

assistance to the inexperienced as well as the experienced user.

INPUT FORMAT FOR TRANSPORT (Pag.10)

By the TRANSPORT input DATA SET is meant the totality of data read

by the program in a single job. A DATA SET may consist of one or more

problems placed sequentially. A problem specifies a calculation or set

of calculations to be done on a given beam line.

A problem, in turn, may consist of one or more problem steps. The data in the first step of a problem specify the beam line and the calculations to be made. The data in succeeding steps of the same problem specify only changes to the data given in the first step.

A common example of a problem with several steps is sequential fitting. In the first step one may specify that certain parameters are

to be varied to satisfy certain constraints. Once the desired fit has

been achieved the program will then proceed to the next step. The data in this step now need specify only which new parameters to vary, or old

ones no longer to vary, or which constraints to add or delete. The values

of the varied parameters that are passed from one step to the next one

are those that are the result of the fitting procedure.

A problem step contains three kinds of DATA cards: the TITLE card,

the INDICATOR card, and the ELEMENT cards.

The TITLE card contains a string of characters and blanks enclosed

by single quotes. Whatever is between the quotes will be used as a

heading in the output of a TRANSPORT run.

The second card of the input is the INDICATOR card. If the data

which follow describe a new problem, a zero (0) is punched in any column

on the card. If the data which follow describe changes to be made in

the previous problem step, a one (1) or two (2) is punched in any column

on the card. For further explanation read the Indicator Card section of

this manual.

The remaining cards in the deck for a given problem step contain the

DATA describing the beam line and the calculations to be done. The DATA

consist of a sequence of elements whose order is the same as encountered

as one proceeds down the beam line. Each element specifies a magnet or

portion thereof or other piece of equipment, a drift space, the initial

beam phase space , a calculation to be done , or a print instruction.

Calculation specifications , such as misalignments and constraints, are

placed in sequence with the other beam line elements where their effect

is to take place. The input format of the cards is "free—field", which

is described below. The data for a given problem step are terminated by

the word SENTINEL, which need not be punched on a separate card.

Each element, in turn, is given by a sequence of items (mostly

numbers) , separated by spaces and terminated by a semicolon. The items,

in order, are a type code number, a vary field, the physical parameters,

and an optional label.

The type code number identifies the element, indicating what sort

of entity (such as a magnet, drift space, constraint, etc.) is represented.

It is an integer (number) followed by a decimal point. The interpretation

of the physical parameters which follow is therefore dependent on the type

code number. The type code numbers and their meanings are summarized in

Table 1. If the type code number is negative, the element will be ignored

in the given problem step. However, storage for that element will be

allocated by the program, so that the element may be introduced in a later

step of the same problem. Storage space for any element in any problem

step must be allocated in the first step of the problem.

The vary field indicates which physical parameters of the element

are to be adjusted if there is to be any fitting. It is punched immediately (no intervening blanks) to the right of the decimal point of the type code number. See the section under type code 10.0 for an explanation of the use of vary codes.

The physical parameters are the quantities which describe the

physical element represented. Such parameters may be lengths , magnetic

fields, apertures, rotation angles, beam dimensions, or other quantities,

depending on the type code number. The meanings for the physical parameters for each type code are described thoroughly in the section for that type code . A summary , indicating the order in which the physical parameters should be punched, is given in Table 1 . For any element the first physical parameter is the second entry in Table 1 or the second parameter in the section describing a given element. In some cases the parameters of an element do not really refer to physical quantities, but will nevertheless be referred to as such in this manual.

The label, if present, contains one to four characters and is enclosed by single quotes, slashes or equal signs. During the calculation the elements will be printed in sequence and the label for a given element will be printed with that element. Labels are useful in problems with many elements and/or when sequential fitting is used. They must be used to identify any element to be changed in succeeding steps of a given problem.

Provision has been made in the program to allow the user to introduce comments before any type code entry in the data deck. This is accomplished

by enclosing the comments made on each card within single parentheses.

Each element must be terminated by a semicolon (;). Optionally a

semicolon may be replaced by an asterisk (*) or a dollar sign ($). Spaces before and after the semicolon are allowed but not required. If the program encounters a semicolon, dollar sign or asterisk before the expected number of parameters has been read in and if the indicator card was a

zero (0) , the remaining parameters are set to zero. If the indicator

card was a one (1) or two (2) , then the numbers indicated on the card are substituted for the numbers remaining from the previous solution; all

other numbers are unchanged.

The "free—fie1d" input format of the data cards makes it considerably easier to prepare input than the standard fixed-field formats of FORTRAN.

Numbers may be punched anywhere on the card and must simply be in the

proper order. They must be separated by one or more blanks . Several

elements may be included on the same card and a single element may continue from one card to the next. A single number must be all on one card , it may not continue from one card to the next . The program storage is limited to a total of 2000 locations (including type codes and those parameters not punched but implied equal to zero) and 500 elements.

A decimal number (e.g. 2.47) may be represented in any of the following ways:

2.47

.00247+3

.0247E+02

247E-2

247000-5

The sample problem below contains two problem steps , each beginning

with title and indicator cards and terminating with a SENTINEL. The first

step causes TRANSPORT to do a first-order calculation with fitting. The

second initiates a second-order calculation with the data that is a result

of that fitting. Corresponding elements between the two steps are identified by having the same label.

The type ten element which specifies the fitting condition is labeled FITl. It is active for the first—order calculation, but is turned off for

the second-order calculation. The vary codes for elements DR1 are set to

zero for the second—order problem. The second—order element, SECl, is

ineffective during the fitting, but causes the program to compute the

second—order matrices in the second calculation.

As many problems and problem steps as one wishes may be stacked in one job.

Note that in previous versions of TRANSPORT a decimal point was

required with every numerical entry except the indicator card (which

must: not have a decimal point in any version of TRANSPORT).

The use of labels (pag. 15)

The use of labels is available for identification of individual

elements. When inserted for the user's convenience, the association of

a label with a given element is optional . I f the parameters of an element are to be changed between steps of a given problem, a label is required. The label identifies the element in the earlier step to which the changes specified in the later step are to apply.

The label may be placed anywhere among the parameters of a given

element. It should be enclosed in quotes, slashes, or equal signs.

Blanks within a label are ignored. The maximum length of a label is

four non—blank characters.

As an example, the following all denote the same drift space:

‘DRF’	3.	1.5	;

3/DRF/15-1*

3.	.15E1	= D R F = $

On a 15.0 type code element the label may not be the third item.

This is to avoid ambiguities with the unit name. Thus the following are

not equivalent:

If the parameters describing an element are to differ in succeeding

steps of a given problem the element must be included in both steps,

having the same label each time. All elements which appear in a problem

must be included in the first step (indicator card 0) of that problem.

Only those to be changed in later steps need to be labeled. In later steps (indicator card 1) of a problem only those elements to be changed

are specified. The elements to be changed are identified by their labels.

If the type code number of an element is negative in a given step of a problem, that element will be ignored when the calculation is performed. However, storage space in the computer will be allocated for the element for possible activation in later steps of the problem. In the later step, only those parameters to be changed need to be specified.

The storage space allocated for the parameters of a given element is determined only by the type code. The sole exceptions are the continuation codes for type codes 1.0 and 14.0.

For example, if a fitting constraint is to be ignored in the first

step of a problem, but activated in a later step, it should be indicated

in both steps . In the first step such an element might appear as

-10.	'FIT' ;

In the later step one would then insert

10.	1.	2.	0.0	.001	'FIT'	;

causing a waist constraint to be imposed on the beam. Alternatively one can specify the physical parameters in the first step and then, in the

later step, merely indicate that the element is now to be activated. The

above procedure is therefore equivalent to placing the element :

-10.	1.	2.	0.0	.001	‘FIT’	;

in the first problem step, and the element

10.	'FIT'	;

in the later step.

Vary codes may also be inserted or removed in passing from one

problem step to the next . For instance, one might wish to vary the field

of a quadrupole in one step of a problem and then use the fitted value

as data in the following step . The first step might then contain the

element:

5.01	5.0	10.0	5.0	'QUAD'	;

and the following step would contain the element

5.	'QUAD’	;

Since, in the second step, the first item on the card contains no vary

code the vary code is deleted. All other parameters, not being re-specified, are left unchanged.

Several elements may have the same label. If, as in the above

example, one wished to vary the field of several quadrupoles in one step,

then pass the final values to the next step, one could give all such

elements the same label. There might be four quadrupoles, all labeled

'QUAD', being varied simultaneously. If the data for the next step contain the single element

5.	'QUAD'	;

the vary code on all elements labeled 'QUAD' will be deleted.

The physical parameters of an element may be changed between steps

of a problem. In the first step a bending magnet may be given a length

of 5 meters.

4.	5.0	10.0	0.0	'BEND'	;

In a succeeding step, its length could be increased to 10 meters by in

setting the element

4.	10.0	'BEND'	;

All parameters, up to and including the one to be changed, must be specified. The remaining, if omitted, will be left unchanged from the previous step.

MISSING: Table 1, important & interesting! Pag. 18)

Table 1: Summary of TRANSPORT type codes

| PHYSICAL ELEMENT | TYPE CODE | 2nd ENTRY | 3rd ENTRY | 4th ENTRY | 5th ENTRY | 6th ENTRY | 7th ENTRY | 8th ENTRY | 9th ENTRY |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BEAN | 1.VVVVVV0 | x(cm) | 0 (mr) | Y(cm) | ϕ (mr) | L(cm) | δ (percent) | P0 |  |
| r.m.s. ADDITION TO BEAN ENVELOPE | 1.VVVVVV00 | δx(cm) | Δ0 (mr) | Δy (cm) | Δϕ (mr) | Δl(cm) | Δδ (percent) | ΔP (GeV/c) | 0 |
| POLE FACE ROTATION | 2.V | ANGLE OF ROTA TION (degrees) |  |  |  |  |  |  |  |
| DRIFT | 3.V | LENGTH (metres) |  |  |  |  |  |  |  |
| BENDING MAGNET | 4.VVV | LENGTH (metres) | FIELD (kG) | FIELD GRADIENT (n-value) |  |  |  |  |  |
| QUADRIPOLE | 5.VV0 | LENGTH (metres) | FIELD (kG) | HALF-APERTURE (cm) |  |  |  |  |  |
| TRANSFORM 1 UPDATE | 6.0 | 0.0 | 1.0 |  |  |  |  |  |  |
| TRANFORM 2 UPDATE | 6.0 | 0.0 | 2.0 |  |  |  |  |  |  |
| BEAM CENTROID SHIFT | 7.0 | SHIFT (x)(cm) | SHIFT (0) (mr) | SHIFT (y) (cm) | SHIFT (ϕ)(mr) | SHIFT (L)(CM) | SHIFT(δ percent) |  |  |
| ALIGNMENT TOLERANCE | 8.VVVVVV0 | DISPLACEMENT (x) (cm) | ROTATION (θ) (mr) | DISPLACEMENT (y) (cm) |  | DISPLACEMENT (z)(cm) | ROTATION (α)(mr) | CODE NUMBER |  |
| REPEAT CONTROL | 9.0 | NUMBER OF REPEATS |  |  |  |  |  |  |  |
| FITTING CONSTRAINTS | 10.0 | ± I? | J | DESIRED VALUE OF (I, J) MATRIX ELEMENTS | ACCURACY OF FIT |  |  |  |  |
| Note: + I is used for fitting a beam (σ)matrix element. –I is used for fitting an RI matrix element. -(I + 20) is used for fitting an R2 matrix element. |  |  |  |  |  |  |  |  |  |
| ACCELERATOR | 11.0 | LENGTH (metres) | E (energy gain) (GeV) | ♦? (phase lag) (degrees) | (WAVELENGTH) (cm) |  |  |  |  |
| BEAM (Rotated Ellipse) | 12.0 | THE FIFTEEN CORRELATIONS AMONG THE SIX ELEMENTS (This entry must be proceded by a type code 1.0 entry) |  |  |  |  |  |  |  |
| INPUT/OUTPUT OPTIONS | 13.0 | CONTROL CODE NUMBER |  |  |  |  |  |  |  |
| ARBITRARY R MATRIX | 14.vvvvvv0 | R(J,I) | R(J,2) | R (J,3) | R (J,4) | R (J,5) | R (J,6) | J |  |
| UNITS CONTROL (Transport Dimensions) | 15.0 | CODE | UNIT SYMBOL | SCALE FACTOR (if required) |  |  |  |  |  |
| QUADRATIC TERM OF BENDING FIELD | 16.0v | 1.0 | ϵ (l) = Β (lρ0)2 | Ρ0 in units of tranverse length (cm) |  |  |  |  |  |
| MASS OF PARTICLES IN BEAM | 16.0 | 3.0 | M/m (dimensionless) | M= mass of electron |  |  |  |  |  |
| HALF-APERTURE OF BENDING MAGNET IN x-PLANE | 16.0 | 4.0 | w/2 (cm) |  |  |  |  |  |  |
| HALF-APERTURE OF BENDING MAGNET IN y-PLANE (gsp) | 16.0 | 5.0 | g/2 (cm) |  |  |  |  |  |  |
| LENGTH OF SYSTEM | 16.0 | 6.0 | L (metres) |  |  |  |  |  |  |
| FRINGE FIELD CORRECTION COEFFICIENT | 16.0 | 7.0 | K1 (dimensionless) |  |  |  |  |  |  |
| FRINGE FIELD CORRECTION COEFFICIENT | 16.0 | 8.0 | K2 (dimensionless) |  |  |  |  |  |  |
| CURVATURE OF ENTRANCE FACE OF BENDING MAGNET | 16.0v | 12.0 | (l/R1) (l/metres) |  |  |  |  |  |  |
| CURVATURE OF EXIT FACE OF BENDING MAGNET | 16.0V | 13.0 | (l/R2) (l/metres |  |  |  |  |  |  |
| FOCAL PLANE ROTATION | 16.0 | 15.0 | Angle of focal plane rotation. See type code 16.0 for details. |  |  |  |  |  |  |
| INITIAL BEAM LIME X-COORDINATE | 16.0 | 16.0 | X0 |  |  |  |  |  |  |
| INITIAL BEAM LIME y-COORDINATE | 16.0 | 17.0 | Y0 |  |  |  |  |  |  |
| INITIAL BEAM LIME z-COORDINATE | 16.0 | 18.0 | Z0 |  |  |  |  |  |  |
| INITIAL BEAM LIME HORIZONTAL ANGLE | 16.0 | 19.0 | θ 0 |  |  |  |  |  |  |
| INITIAL BEAM LIME VERTICAL ANGLE | 16.0 | 20.0 | ψ 0 |  |  |  |  |  |  |
| SECOND-ORDER CALCULATIONS | 17.0 |  |  |  |  |  |  |  |  |
| SEXTUPOLE | 18.0V | LENGTH (metres) | FIELD (kG) |  |  |  |  |  |  |
| SOLENOID | 19.VV | LENGTH (metres) | FIELD (kG) |  |  |  |  |  |  |
| BEAM ROTATION | 20.V | ANGLE OF ROTATION(degrees) |  |  |  |  |  |  |  |
| STRAY FIELD | 21.0 | See later section of report |  |  |  |  |  |  |  |
| Note: The v’s following the type codes indicate the parameters which may be varied. See section under type code 10.0 for a detailed explanation of vary |  |  |  |  |  |  |  |  |  |

(da controllare perché ho faticato molto nella lettura e interpretazione segni!)

OUTPUT FORMAT

General appearance

Here we give a brief description of the general appearance of the

output and its meaning. The user may refer to the sample output shown on

pages 22 through 27. It is the printed output resulting from the sample

data shown in the section on input format. In a simple example it is not

possible to show each of the different type codes. Several of the type

codes produce output which is not characteristic of all other type codes.

We therefore refer the user to the sections on the various type codes for

an explanation of any features peculiar to a given type code.

The output for each step of a given problem is printed separately.

The printing for one step is completed before that for the next step is

begun. Therefore we will describe the output for a single problem step.

The output shown below is from a problem with two steps.

Initial listing

For each problem step, the program begins by printing out the user's

input records.

Listing during the calculation (pag.20)

The program now begins the calculation. If there is no fitting, one

listing of the beam line will be made. If there is fitting there will

normally be two listings . The first will represent the beam line before

any fitting has occurred. The second will be based on the new values of

the physical parameters which were altered by the fitting process. If

sequential fitting is employed and an indicator card of two (2) is used

the first run will be omitted. The user should read the section describing the indicator card for further explanation.

In any listing the elements are printed in order with their labels

and physical parameters . Elements with negative type code numbers are

ignored. Each listed element is preceded by the name of that type of

element, enclosed in asterisks . All physical elements are listed in this

way. Some of the other elements are not explicitly listed but produce

their effect in either the calculated quantities or the listing of the

beam line. For descriptions of individual cases, the reader should

consult the sections on the type codes.

Calculated quantities appear in the listing as requested in the input

data. Important cases will be described in greater detail below. The

physical parameters for each element are printed with the appropriate

units. For some elements a calculated quantity, not in the input data,

will appear, enclosed in parenthesis. Such quantities are explained in

the sections under the individual type codes.

Calculated quantities

The important cases of calculated quantities which appear in the

output are the transfer matrices, the beam matrix, the layout coordinates,

and the results of the fitting procedure . The transfer and beam matrices

and layout coordinates appear as requested in the listing of the beam

line. The results of the fitting procedure appear between the two

listings. All these quantities are explained in greater detail below.

The transfer and beam matrices appear only where requested. A re-

quest for printing of layout coordinates should be made at the beginning

of the beam line. The coordinates will then be printed after each physi-

cal element. In all cases the quantities printed are the values at the

interface between two elements. They are evaluated at a point after the

element listed above them and before the element listed below. For further explanation of calculated quantities the user should read the section on the mathematical formulation of TRANSPORT, the appendix to the

manual, and the section on the appropriate type code. For the transfer

matrix the appropriate type code is thirteen; for the beam matrix it is

one, and for the coordinate layout it is again thirteen.

Quantities relevant to the fitting appear between the two listings

of the beam line. At each iteration of the fitting procedure a line is

printed containing the value of the relaxation factor used, the value of

chi—squared before the iteration was made, and the corrections made to

each of the varied parameters. Once the fitting is complete the final

chi-squared and the covariance matrix are printed. For further details

the user should read the section on type code 10.0, and the section on

fitting in the appendix.

Mancano tabelle pagg. 22,23,24, 25,26,27

TITLE CARD

The title card is the first card in every problem step of a TRANSPORT

data set. The title card is always required and must be followed by a

O, a l or a 2 card (see next section) to indicate whether the data to

follow is new (0 card) or a continuation of a previous data set (a 1 card

or a 2 card) .

The title must be enclosed within either quotation marks ('), slashes (/), or equal signs (=) on a single card. The string may begin and end in any column (free field format) , for example

'SLAC 20 GEV/C SPECTROMETER'

or

/SLAC 20 GEV SPECTROMETER/

Note that whichever character is used to enclose the title must not

be used again within the title itself.

Example of a DATA SET for a single problem step

INDICATOR CARD (pag.29)

The second card of the input for each step of a problem is the indicator card.

If the data which follow describe a new problem, a zero (0)

is punched in any column on the card. If the data which follow describe

changes to be made in the previous step of a given problem, a one (1) or

two (2) is punched in any column on the card.

If a given problem step involves fitting, the program will normally

list the beam line twice, printing each time the sequence of elements

along with transfer or beam matrices where specified. The first listing

uses the parameters of each element before any fitting has taken place.

The second shows the results of the fitting. If a problem involving

fitting has several steps, the second run of a given step often differs

little from the first run of the following step.

If the second or subsequent step of a problem involves fitting and

one wishes to print both runs through the beam line, a one (1) is punched

on the indicator card. If the first listing is to be suppressed a two (2)

is punched. If no fitting is involved, the program will ignore the two

and will do one single run through the system.

If the initial listing is to be deleted, 10 is added to the indicator

to give 10, 11, or 12. In order to be consistent with earlier versions

of TRANSPORT, an indicator of minus one (-1) is interpreted as a two (2) ,

but nine (9) is not interpreted as twelve (12) .

The sample problem input shown on page 14 (?) causes TRANSPORT to do a

first—order calculation with fitting (0 indicator card) and then to do a

second—order calculation (1 indicator card) with the data that is the

result of the fitting.

COMMENT CARDS

Comment cards may be introduced anywhere in the deck where an

element would be allowed by enclosing the comments made on each card

within single parentheses. No parentheses are allowed within the

parentheses of any comment card. The comments are not stored, but appear

only in the initial listing of the given problem step.

Example of the use of comment cards in a data set

‘Title Card’

0

(THIS IS A TEST PROBLEM TO ILLUSTRATE THE)

(USE OF COMMENT CARDS)

elements

(COMMENTS MAY ALSO BE MADE BETWEEN)

(TYPE CODE ENTRIES)

elements

SENTINEL

LISTING OF AVAILABLE TRANSPORT TYPE CODE ENTRIES

INPUT BEAM: Type code 1.0

The phase space and the average momentum of the input beam for a

TRANSPORT calculation are specified by this element. The input is given

in terms of the semi-axes of a six—dimensional erect*) beam ellipsoid

representing the phase space variables x, θ , y, ϕ  , λ , and δ.

Each of these six parameters is entered as a positive quantity, but should be

thought of as ±x, ±θ, etc; hence , the total beam width is 2x, the total

horizontal beam divergence is 2 θ and so forth.

------------------------------------------------------

*) For a rotated (non—erect) phase ellipsoid input, see type code 12.0.

------------------------------------------------------

Usually the BEAM card is the third card in the deck. If other than

standard TRANSPORT units are to be used, the units specification cards

(type code 15 .0) should precede the BEAM card. Standard TRANSPORT units

for x, θ, y, ϕ, λ  and δ are cm, mr, cm, mr, cm and percent. The standard

unit for the momentum p(O) is GeV/c. Also if a beam line coordinate

layout is desired, the card specifying that a layout is to be made (a 13.0

12.0 element), and any initial coordinates (see type code 16.0) all precede

the BEAM card.

There are eight entries (all positive) to be made on the BEAM card.

- The type code 1.0 (specifies a BEAM entry follows).

- One-half the horizontal beam extent (x) (cmv in standard units).

- One-half the horizontal beam divergence (θ) (mr).

- One-half the vertical beam extent (y) (cm).

- One—ha1f the vertical beam divergence (ϕ) (mr) .

- One—half the longitudinal beam extent  (λ)(cm).

- One—ha1f the momentum spread (δ) (in units of percent Δp/p).

- The momentum of the central trajectory [p(O)] (GeV/c) .

All eight entries must be made even if they are zero (0). As for all

other type codes, the last entry must be followed by a semicolon, dollar

sign, or asterisk. Thus a typical BEAM entry might be

Label if desired.

- 0.5    2.    1.3   2.S    0.   1.5   10 .   '   '   ;

meaning, x = ±0,5 cm, θ = ± 2,0 mr, y = ± 1,3 cm, φ = ± 2.5 mr, λ ± 0.0 cm,

δ = ± i1.5 percent Δp/p, and the central momentum p(O) = 10.0 GeV/c.

The units of the tabulated matrix elements in either the first

order R or sigma matrix or second order T matrix of a TRANSPORT print—out will correspond to the units chosen for the BEAM card. For the above

example , the R(12) = (X/θ ) matrix element will have the dimensions of

cm/mr; and the T(236) = (θ/yδ) matrix element will have the dimensions

mr/(cm ' percent) and so forth.

The longitudinal extent λ is useful for pulsed beams. It indicates

the spread in length of particles in a pulse. It does not interact with

any other component and may be set to zero if the pulse length is not

important.

The phase ellipse (sigma matrix) beam parameters may be printed as

output after every physical element if activated by a (13. 3. ;) element.

Alternatively, individual printouts may be activated by a (13. 1. ;)

element. The projection of the semi—axes of the ellipsoid upon each of

its six coordinates axes is printed in a vertical array, and the correlations among these components indicating the phase ellipse orientations

are printed in a triangular array (see the following pages).

The phase ellipse beam matrix

The beam matrix carried in the computer has the following construction:

|  | x | θ | y | ϕ | λ | δ |
| --- | --- | --- | --- | --- | --- | --- |
| x | σ (11) |  |  |  |  |  |
| θ | σ (21) | σ (22) |  |  |  |  |
| y | σ (31) | σ (32) | σ (33) |  |  |  |
| θ | σ (41) | σ (42) | σ (43) | σ (44) |  |  |
| ℓ | σ (51) | σ (52) | σ (53) | σ (54) | σ (55) |  |
| δ | σ (61) | σ (62) | σ (63) | σ (64) | σ (65) | σ (66) |

The matrix is symmetric so that only a triangle of elements is

needed.

In the printed output this matrix has a somewhat different format

for ease of interpretation:

|  |  |  | x | θ | y | ϕ | λ |
| --- | --- | --- | --- | --- | --- | --- | --- |
| x | σ(11) | CM |  |  |  |  |  |
| θ | σ(22) | MR | r (21) |  |  |  |  |
| y | σ(33) | CM | r (31) | r (32) |  |  |  |
| ϕ | σ(44) | MR | r (41) | r (42) | r (43) |  |  |
| ℓ | σ(55) | CM | r (51) | r (52) | r (53) | r (54) | ) |
| δ | σ(66) | PC | r (61) | r (62) | r (63) | r (64) | r (65) |

where: 		r(ij) = σ(ij)σiiσ(jj)½

As a result of the fact that the σ  matrix is positive definite, the

r(ij) satisfy the relation

r(ij) ≤ 1.

The full significance of the σ (ij) and the r(ij) are discussed in

detail in the Appendix ("Description of Beam Matrix") . The units are

always printed with the matrix.

In brief, the meaning of the √σ (ii) is as follows:

√σ(ll) = xmax = the maximum (half) -width of the beam envelope in the

x(bend)—plane at the point of the print-out.

√σ (22) = θmax = the maximum (half) —angular divergence of the beam

envelope in the x(bend) plane.

√σ(33) = ymax = the maximum (half)—height of the beam envelope.

√σ(44) = ϕmax = the maximum (half)—angular divergence of the beam envelope in the 		y(non—bend)-plane.

√σ(55) = ℓ max = one—half the longitudinal extent of the bunch of particles.

√σ(66) = δ = the half—width (1/2 Δp/p) of the momentum interval

being transmitted by the system.

The units appearing next to the √σ(ii) in the TRANSPORT print—out

are the units chosen for coordinates x, θ , y, ϕ , λ and δ  = Δp/p, respectively.

To the immediate left of the listing of the beam envelope size in a

TRANSPORT print—out , there appears a column of numbers whose values will normally be zero. These numbers are the coordinates of the centroid of

the beam phase ellipse (with respect to the initially assumed central

trajectory of the system) . They may become non-zero under one of three

circumstances:

1) when the misalignment (type code 8.0) is used,

2) when a beam centroid shift (type code 7.0) is used, or

3) when a second-order calculation (type code 17.0) is used.

To aid in the interpretation of the phase ellipse parameters listed

above, an example of an (x,θ) plane ellipse is illustrated below. For

further details the reader should refer to the Appendix of this report.

The area of the ellipse is given by:

A = π (det σ)½ = πxmax θ xint = πxint θmax

The equation of the ellipse is:

Yx2 + 2αxθ  + βθ2  = ϵ

Where:

σ = σ11σ21σ21σ22 = ϵ β-α-αγ

and

βγ – α2 = 1 , r21 = r12 = -α 1+α2=  -αβγ

r.m.s. addition to the BEAM

To allow for physical phenomena such as multiple scattering, provision has been made in the program to permit an r.m. s. addition to the beam envelope. There are nine entries to be included:

- Type code 1.0 (specifying a BEAM entry follows).

- The r.m.s. addition to the horizontal beam extent (Δx) (cm).

- The r.m.S. addition to the horizontal beam divergence (Δθ) (mr).

- The r.m.s. addition to the vertical beam extent (Δy) (cm).

- The r.m.s. addition to the vertical beam divergence (Δθ) (mr)

- The r.m.s. longitudinal beam extent (Δλ) (cm).

- The r.m.s. momentum spread (Δδ) (in percent Δp/p).

- The momentum change in the central trajectory [Δp(O)] in (GeV/c).

- The code digit 0. indicating an r.m.s. addition to the BEAM is

being made .

The units for the r.m. s. addition are the same as those selected

for a regular BEAM type code 1.0 entry. Thus a typical r.m.s. addition

to the BEAM would appear as follows:

- .1   .2   .15   .3   0.   .13   -0.1   0.   ;

where the last entry (0.) preceding the semicolon signifies an r.m.s.

addition to the BEAM is being made and the next to the last entry indi

cates a central momentum change of -0.1 GeV/c.

FRINGING FIELDS and POLE-FACE ROTATIONS for bending magnets:

Type code 2.0 (pag.37)

To provide for fringing fields and/or po1e—face rotations on bending

magnets, the type code 2.0 element is used.

There are two parameters:

1 - Type code 2.0.

2 — Angle of po1e—face rotation (degrees).

The type code 2.0 element must either immediately precede a bending

magnet (type code 4.0) element (in which case it indicates an entrance

fringing field and pole—face rotation) or immediately follow a type

code 4. 0 element (exit fringing field and pole—face rotation) with no

other data entries between *), A positive sign of the angle on either

entrance or exit po1e—faces corresponds to a non-bend plane focusing

action and bend plane defocusing action.

For example , a symmetrically oriented rectangular bending magnet

whose total bend is 10 degrees would be represented by the three entries

2.   5. ;     4. ——- ;    2.     5. ;

The angle of rotation may be varied. For example , the element

2.1    5.   ; would allow the angle to vary from an initial guess of

5 degrees to a final value which would, say, satisfy a vertical focus

constraint imposed upon the system. See the type code 10.0 section for

a complete discussion of vary codes.

Even if the pole—face rotation angle is zero, 2.   O.   ; entries

must be included in the data set before and after a type code 4. 0 entry

if fringing—fie1d effects are to be calculated.

A single type code 2.0 entry that follows one bending magnet and

precedes another will be associated with the latter.

___________________________________

*) It is extremely important that no data entries be made between a type code 2 .0 and a type code 4. 0 entry. If this occurs, it may result in an incorrect matrix multiplication in the program and hence an incorrect physical answer. If this rule is violated, an error message will be printed.

_____________________________________

Should it be desired to misalign such a magnet, an update element

must be inserted immediately before the first type 2 .0 code entry and the

convention appropriate to misalignment of a set of elements applied, since, indeed, three separate transformations are involved. See section under

type code 8.O for a discussion of misalignment calculations and the section under type code 6.0 for a discussion of updates.

The type code signifying a rotated pole-face is 2.0. The input format is:

Label (if desired)

2.    β .   ‘RO’   ;

The units for β are degrees.

Pole—face rotation matrix

The first—order R matrix for a pole-face rotation used in a

TRANSPORT calculation is as follows:

R = 100000tanβρ01000000100000-tan⁡(β-ψ)ρ0100000010000001

Definitions:

β  = angle of rotation of pole face (see figure on following page for sign convention of β)

ρ0  = bending radius of central trajectory

g = total gap of magnet

ψ = correction term resulting from spatial extent of

fringing fields **)

where 			ψ = K1 gρ0 1+sin2βcosβ  1-K1K2 gρ0tan β*

-------------------------------------------------------

*)See type code 16.0 for input formats for g, K1 , and K2 TRANSPORT entries.

**)See SLAC—754) (page 74)?? for a discussion of ψ .

didascalia

FIELD BOUNDARIES FOR BENDING MAGNETS

The TRANSPORT sign conventions for X, β, R and h are all positive as shown in the figure. The positive y direction is out of the paper. Positive β's imply transverse focusing. Positive R's (convex curvatures) represent negative sextupole components of strength

S = (- h/2R) sec3 β. (See SLAC-75, page 71.)??

DRIFT: Type code 3.0

A drift space is a field-free region through which the beam passes.

There are two parameters:

1 — Type code 3.0 (specifying a drift length).

2 - (Effective) drift length (metres) . The length of a drift

space may be varied in either first:- or second—order fitting.

Typical input format for a DRIFT:

Label (if desired) (not to

exceed 4 spaces between  quotes).

3.   6.   ‘D1’   ;

DRIFT space matrix

The first—order R matrix for a drift space is as follows:

1L0000010000001L00000100000010000001

where

L = the length of the drift space.

The dimensions of L. are those chosen for longitudinal length via a

units symbol

scale factor (if needed)

15. 8. '   ' ; type code entry (if used) preceding the BEAM (type code 1.0)

card. If no 15. 8. entry is made, the units of L. will automatically be

in metres (standard TRANSPORT units)

WEDGE BENDING MAGNET: Type Code 4.0

A wedge bending magnet implies that the central trajectory of the

beam enters and exits perpendicularly to the pole-face boundaries (to

include fringing-field effects and non—perpendicu1ar entrance or exit

boundaries —— see type codes 2.0 and 16.0).

There are four first-order parameters to be specified for the wedge

magnet via type code 4.0:

1 — Type code 4.0 (specifying a wedge bending magnet).

2 — The (effective) length L of the central trajectory in metres .

3 — The central field strength B(O) in kG,

B(0) = 33.356 (p/ρo),

where p is the momentum in GeV/c and ρ0 is the bending radius of

the central trajectory in metres .

4 — The field gradient (n—value , dimensionless); where n is defined

by the equation

By(x,O,t) = By(0,0,t) (1 - nhx + ...) ,

where

H= 1/ρ0. See SLAC-75 (page 31) 4).

The quantities L, B(O) , and n may be varied for first—order fitting

(see type code 10.0 for a discussion of vary codes).

The bend angle in degrees and the bend radius in metres are printed

in the output.

A typical first—order TRANSPORT input for a wedge magnet is

label (not to exceed 4 spaces)

4.    1.    B.    n.    ‘  ‘   ;

If fringing field effects are to be included, a type code 2.0 entry

must immediately precede and follow the pertinent type code 4. 0 entry (even

if there are no pole—face rotations) . Thus a typical TRANSPORT input for

a bending magnet including fringing fields might be:

Labels (not to exceed I 4 spaces) if desired

2.   0.   ‘  ‘   ;

4.   L.   B(0).   n.   ‘  ‘   ;

2.   0.   ‘  ‘   ;

For non-zero pole-face rotations a typical data input might be

2.   10.   ;   4.  L. B(0).   n.   ;   2.   20.   ;

Note that the use of labels is optional and that all data entries may be

made on one line if desired.

The sign conventions for bending magnet entries are illustrated in

the following figure. For TRANSPORT a positive bend is to the right

looking in the direction of particle travel. To represent a bend in

another sense, the coordinate rotation matrix (type code 20.0) should

be used as follows:

A bend up is represented by rotating the (x, y) coordinates by

-90.0 degrees about the (z) beam axis as follows:

Labels (not to exceed 4 spaces)

if desired

20. -90.   ' '   ;

2. β(1).   ' '   ;

4. L.   B.   n.   ' '   ;

2. β(2).   ' '   ;

20. +90.   ' '   ; (returns coordinates to their initial

orientation)

A bend down is accomplished via:

20.   +90.   ‘  ‘   ;

2.

4.

2.

20,   -90.   ‘  ‘   ;

A bend to the left (looking in the direction of beam travel) is

accomplished by rotating the x, y coordinates by 180 degrees, e.g.

20.    180. ' '   ;

2.

4.

2.

20,   -180.   ‘  ‘   ;

(didascalia del disegno)

FIELD BOUNDARIES FOR BENDING MAGNETS

The TRANSPORT sign conventions for X, β, R and h are all positive as shown in the figure, The positive y direction is out of the paper. Positive β's imply transverse focusing. Positive R's (convex curvatures) represent negative sextupole components of strength

s = (- h/ZR) sec3 5. (See SLAC-75, page 71.)

The field expansion for the midplane of a bending magnet is taken

from Eq. (18) page 31 of SLAC—75, thereby defining the dimensionless

quantities n and β as follows:

By (x,0,t) = By (0,0,t) [1 – nhx + βh2x2 + γh3x3 + …]

The type code signifying a BEND is 4.0. The input format for a

TRANSPORT calculation is:

Label(not to exceed 4 spaces)

4.  L.   B.   n.   ' '   ;

If n is not included in the data entry, the program assumes it to

be zero. A β  entry for a second—order calculation is made via the

16.0 1.0 element. (Do not confuse this β with a po1e—face rotation.)

The standard units for L and B are metres and kG. If desired, these

units may be changed by 15.0 8.0 and 15.0 9.0 type code entries preceding

the BEAM Card.

QUADRUPOLE: Type code 5.0 (pag.48)

A quadrupole provides focusing in one transverse plane and defocusing in the other.

There are four parameters to be specified for a TRANSPORT calculation:

1 - Type code 5.0 (specifying a quadrupole).

2 -(Effective) magnet length L (in metres) .

3 - Field at pole tip B (in kG) . A positive field implies horizontal

focusing; a negative field, vertical focusing.

4 - Half-aperture a (in cm) . Radius of the circle tangent to the

pole tips.

The length and field of a quadrupole may be varied in first—order

fitting. The aperture may not be.

The strength of the quadrupole is computed from its field, aperture

and length. The horizontal focal length is printed in parentheses as

output. A positive focal length indicates horizontal focusing and a negative focal length indicates horizontal defocusing. The quantity

actually printed is the reciprocal of the (θ/x) transfer matrix element

(1/R2 1) for the quadrupoles . Thus two identical quadrupoles of opposite

polarity will have different horizontal focal lengths due to the difference

between the sine and the hyperbolic sine.

The type code for a QUAD is 5.0. The input format for a typical

data set is:

Label (if desired) not to exceed

4 spaces between quotes

5.   L.   B.   a.   ‘  ‘   ;

The standard TRANSPORT units for L, B, and a are metres , kG, and cm,

respectively. If other units are desired they must be chosen via the

appropriate 15.0 type code entries preceding the BEAM (type code 1.0)

card.

First-order quadrupole matrix

cos kqL1kqsinkqL0000-kqsinkqLcos kqL000000cosh kqL1kqsinhkqL0000kqsinhkqLcosh kqL00000010000001

These elements are for a quadrupole which focuses in the horizontal

(x) plane (B positive). A vertically (y-plane) focusing quadrupole

(B negative) has the first two diagonal submatrices interchanged.

Definitions: 	L = the effective length of the quadrupole

a = the radius of the aperture

B0 = the field at radius a

K2q= (B0/a) (1/Bρ0), where (Bρ0) = the magnetic rigidity

(momentum) of the central trajectory.

TRANSFORM 1 update*): Type code 6. 0. 1.

To re—initia1ize the matrix TRANSFORM 1 (the product of the R

matrices, Rl) use type code 6.0. A (6. O. 1.  ;) card effects an update

of the R1 matrix and initiates the accumulation of a new product matrix

at the point of the update. This facility is often useful for misaligning

a set of magnets or fitting only a portion of a system.

The matrix R1 is updated by no other element. It is not used in the

calculation of the beam matrix. The beam matrix is calculated from the

auxiliary transfer matrix R2 described on the next page.

A TRANSFORM 1 matrix will be printed at any position in the data

set: where a (13. 4.  ;) entry is inserted.

See the following section for the introduction of an auxiliary trans-

formation matrix R2 (TRANSFORM 2) to avoid the need for TRANSFORM 1 up-

dates.

The (6. O. 1.  ;) card also causes an update of the R2 matrix.

---------------------------------------

*) By "updating" we mean initiating a new starting point for the accumulation (multiplication) of the R matrix. At the point of update the previous accumulation is discontinued. When the next element possessing a transfer matrix is encountered, the accumulated transfer matrix R1 is set equal to the individual transfer matrix R for that element. Accumulation is resumed thereafter.

---------------------------------------

AUXILIARY TRANSFORMATION MATRIX (R2): Type code 6. O. 2.

The R1 matrix represents the accumulated transfer matrix from either

the beginning of the beam line or the last explicit R1 update (6. O. 1. ;). However several elements in TRANSPORT which affect the beam matrix cannot

be represented in any transfer matrix. To avoid update complications with R1 an auxiliary transfer matrix R2 exists. The beam matrix is then

calculated from the R2 matrix and the beam matrix at the last R2 update.

Both the R1 and R2 matrices are normally available for printing. However there is no redundancy in computer use, since, internally to the

program only R2 is calculated at each element. The matrix R1 is calculated

from R2 only as needed.

The R2 matrix is updated explicitly via a (6. 0. 2.  ;) entry. It may be printed by a (13. 24.  ;) entry. Constraints on R2 are imposed similarly to those on R1. For details see the section describing type code 10.0.

The complete list of elements which update TRANSFORM 2 is:

1) a beam type code 1.0 entry

2) the (6. 0. 1.  ;) entry

3) the (6. O. 2.  ;) entry

4) a centroid shift type code 7.0 entry

5) a misalignment type code 8.0 entry

6) a stray field type code 21.0 entry.

Please note that automatic updates of TRANSFORM 2 occur when an

align element (type code 8.) is inserted specifying the misalignment of all subsequent bending magnets. These TRANSFORM 2 updates take place immediately before and immediately after any bending magnet which has

either the entrance or exit fringe fields specified via a type code 2

entry.

SHIFT IN THE BEAM CENTROID: Type code 7.0

Sometimes it is convenient to redefine the beam centroid **) such

that it does not coincide with the TRANSPORT reference trajectory. Provision has been made for this possibility via type code 7.0. Seven parameters are required:

1 — Type code 7.0.

(2 to 7) — the coordinates x, θ, y, ϕ, λ, and δ  defining the shift in the

location of the beam centroid with respect to its previous

position. The units for x, θ, y, ϕ, λ, δ  are the same as

those chosen for the BEAM (type code 1.0 entry), normally cm,

mr, cm, mr, cm, and percent.

Any or all of the six beam centroid shift parameters may be varied in first-order fitting. The centroid position may then be constrained at any later point in the beam line by this procedure.

The transformation matrix R2 is updated by this element.

In order for this code to function properly, the initial BEAM entry

(type code 1.0) must have a non—zero phase space volume, for example a

1.  0   0   0   0   0   0 p(O). ;

BEAM entry is not permissible when calculating a shift in the beam

centroid; whereas a

1. 1. 1. 1. 1. 1- 1- p(0).   ;

entry (non-zero phase volume) is acceptable.

-------------------------------------------------------

**) By "beam centroid" we mean the centre of the beam ellipsoid.

-------------------------------------------------------

MAGNET ALIGNMENT TOLERANCES: Type code 8.0

The first—order effects of the misalignment of a magnet or group of

magnets are a shift in the centroid of the beam and a change in the beam

focusing characteristics. Two varieties of misalignment are commonly

encountered: 1) the magnet is displaced and/or rotated by a known amount;

or 2) the actual position of the magnet is uncertain within a given tolerance. TRANSPORT has the capability of simulating the misalignment of

either single magnets or entire sections of a beam line. Any combination

of the above alternatives may be simulated through the use of the "align"

element. The results may be displayed in either the printed output of

the beam (sigma) matrix or tabulated in a special misalignment table

(described below).

There are eight parameters to be specified:

- Type code 8.0 (specifying a misalignment).

- The magnet displacement in the horizontal direction (cm).

- A rotation about the horizontal axis (mt).

- A displacement in the vertical direction (cm).

- A rotation about the vertical axis (mr).

- A displacement in the beam direction (cm).

- A rotation about the beam direction (mr).

- A three-digit code number (defined below) specifying the type

of misalignment.

The three displacements and three rotations comprise the six degrees

of freedom of a rigid body and are used as the six misalignment coordinates. The coordinate system employed is that to which the beam is referred at the point it enters the magnet.  For example, a rotation of a

bending magnet about the beam direction (parameter 7 above) is referred

to the direction of the beam where it enters the magnet. The units employed are the standard TRANSPORT units shown above, unless redefined by

type code 15. entries. If the units are changed, the units of the misalignment displacements are those determined‘ by the 15. 1. type code

entry; the units for the misalignment rotations are those determined by

the 15. 2. type code entry.

The misalignment of any physical element or section of a beam line

may be simulated. Misaligned sections of a beam line may be nested. A

beam line rotation (type code 20.) may be included in a misaligned section.

Thus, for example, one can simulate the misalignment of magnets

that bend vertically. The arbitrary matrix (type code 14.) may not be

included in a misaligned section. A misalignment must never be included

in a second—order run (type code 17.) .

A misalignment element may indicate that a single magnet or section

of the beam line is to be misaligned, or it may indicate that all subsequent magnets of a given type (quadrupoles and/or bending magnets) are to

be misaligned. The type of misalignment is specified in the three—digit

code number, and the location of the type code 8. align element depends

on the type of misalignment.

If a misalignment pertains to a single magnet or a single section of

the beam line, then the misalignment element (type code 8.) must directly

follow that magnet or section of the beam line. If a misalignment element indicates that all subsequent magnets of a given type are to be

misaligned, it must precede the first of such magnets. Further description

of the available types of misalignment is given in the table below.

The results of the misalignment may be displayed in either the beam

(sigma) matrix or in a misalignment table. If the results are displayed

in the beam (sigma) matrix, then that matrix is altered by the effects of

the misalignment. The effects of additional misalignments cause further alterations, so that at any point along the beam line, the beam (sigma)

matrix will contain the combined effects of all previous misalignments.

The misalignment table can be used to show independently the effects

on the beam matrix of a misalignment in each degree of freedom of each

misaligned magnet. Each new misalignment to be entered in the table

creates a new set of six duplicates of the beam matrix. Printed for each

duplicate beam matrix are the centroid displacement and the beam half

width in each of the six beam coordinates. Each of the six matrices shows

the combined result of the undisturbed beam matrix and the effect of the

misalignment in a single coordinate of a single magnet or section of the

beam line. In a single TRANSPORT run the results of misaligning up to

ten magnets or sections of the beam line may be included in the misalign

ment table. Further requests for entry in the misalignment table will be

ignored. Examples of such a table and the input which generated it are

shown below.

When the user specifies that the actual position of the magnet(s) is

uncertain within a given tolerance, the printout will show a change in

the beam (sigma) matrix resulting from the effects of the misa1ignment(s) .

Thus, if one wishes to determine the uncertainty in the beam centroid resulting from uncertainties in the positioning of the magnets, the initial

beam dimensions should be set to zero, i.e. the beam card entry at the

beginning of the system should appear as follows:

1. O. O. O. O. O. O. p(O).

If it is desired to know the effect of an uncertainty in position on

the beam focusing characteristics, then a non—zero initial phase space

must be specified. The printout will then show the envelope of all pos

sible rays, including both the original beam and the effects of the misalignment .

If the misalignment is a known amount, it may affect the beam centroid as well as the beam dimensions. Therefore one should place on the

BEAM card the actual dimensions of the beam entering the system. For a

known misalignment, the program requires that the initial beam specified

by type code 1 must be given a non-zero phase volume, to insure a correct printout.

An align element pertaining to a single magnet or section of the

beam line updates the BEAM (sigma) matrix and the R2 matrix, but not the

R1 matrix. A misalignment element which indicates misalignment of all

subsequent magnets of a given type will update the BEAM (sigma) matrix

and the R2 matrix before each bending magnet with fringe fields and after

each misaligned magnet of any type.

The tolerances may be varied. Thus, type—vary code 8.111111 permits

any of the six parameters (2 through 7 above) to be adjusted to satisfy

whatever BEAM constraints may follow. For fitting , a misalignment must

pertain to a single magnet or single section of the beam line, and the

results must be displayed in the beam (sigma) matrix. (See the section

under type code 10. for a discussion of the use of vary codes.)

The meaning of the options for each digit of the three—digit code

number is given in the following table.

- The units position specifies the magnet(s) or section of the

beam line to be misaligned.

CODE 					INTERPRETATION (pag.57)

NUMBER

----------------------------------------------------------------------

XX0	The single magnet (type code element) immediately preceding

the align card it to be misaligned. A bending magnet with

fringe fields should be misaligned using one of the options

described below.

XX1	The last R1 matrix update (the start of the beam line or a

6. O. 1. type code entry) marks the beginning of the section to be 	misaligned. The misalignment element itself marks the end. The section is 	treated as a unit and misaligned as a whole. The misalignments of 	quadrupole triplets and other combinations involving more than two quadru-

poles may be studied using this code digit.

XX2	The last R2 matrix update (see type code 6. for a list of elements which 	update R2) marks the beginning of the misaligned section. The misalignment 	element marks the end. This option makes use of the fact that R2 matrix 	updates do not affect the R1 matrix.

A bending magnet with fringing fields or pole face rotations

(type code 2.) should be misaligned using this option. See

examples 1 and 2 below for an illustration of this.

An array of quadrupoles provides another example of the use

of this option. By successive application of align elements, the elements 	of a quadrupole triplet could be misaligned relative to each other, and 	then the triplet as a whole could be misaligned. See example 3 below for an 	illustration of this.

XX3	All subsequent bending magnets and quadrupoles are independently misaligned 	by the amount specified. This option is useful in conjunction with the 	tabular display of the misalignment results (see below) . A bending magnet, 	with fring ing fields included, is treated as a single unit and misaligned 	accordingly.

XX4	All subsequent bending magnets, including fringing fields, are 	independently misaligned by the amount specified. See XX3 above for further 	comments.

XX5	All subsequent quadrupoles are independently misaligned by

the amount specified. See example 4 below for an illustration of this. See 	XX3 above for further comments.

- The tens position defines the mode of display of the results of

the misalignment .

X0X. 	The beam matrix contains the results of the misalignment.

The beam matrix is printed wherever a 13. 1. card is

encountered. The beam matrix will then contain contributions

from all previous misalignments.

X1X	A table is used to store the results of misalignments. The

effect of up to ten independently misaligned magnets may be

shown in the table in a single run . The table is printed via

a 13 . 8. card, and may be compared with the undisturbed

beam matrix (printed by a 13. 1.  ;  card) at any point. An

example of such a table is shown below.

- The hundreds position distinguishes between an uncertainty in

position (OXX.) or a known displacement (1XX.) .

Any combination of digits may be used to define the exact circumstances intended. Thus, code 111. indicates the deliberate displacement

of a set of magnets (referred to the point where the beam enters the set) ,

with the results to be stored in a table.

Example N0. 1: A bending magnet with a known misalignment

A bending magnet (including fringe fields) misaligned by a known

amount might be represented as follows:

3.	L(1).   ;

6.	0.   2.   ;

2.	0.   ;   4.   1.   B.   n.   ;   2.   0.   ;

8.	0.   0.   0.   0.   0.   2.   102.   ;

3.	L(2).   ;

This represents a known rotation of the bending magnet about the incoming beam direction (z axis) by 2.0 mr. The result of this misalignment will be a definite shift in the beam centroid, and a mixing of the horizontal and vertical coordinates. The use of the 6. O. 2. Transform 2 update and the misalignment code number XX2 is necessary because the magnetic array (bending magnet + fringing fields) consists of three type code elements instead of one.

Example N0. 2: A bending magnet with an uncertain position

A bending magnet having an uncertainty of 2 mr in its angular positioning about the incoming beam (z axis) would be represented as follows:

3.	L(1).   ;

6.	0.   2.   ;

2.	0.   ;   4.   L.   B.   n.   ;   2.   0.   ;

8.	0.   0.   0.   0.   0.   2.0   002.   ;

3.	L(2).   ;

To observe the uncertainty in the location of the outcoming beam

centroid, the input BEAM card should have zero phase space dimensions as

follows:

- O. O. O. O. 0. 0. p(0).   ;

If the beam dimensions on the input BEAM card are non—zero, the resultant beam (sigma) matrix will show the envelope of possible rays, including both the input beam and the effect of the misalignment.

Example No. 3: A misaligned quadrupole triplet

One typical use of both the R1 and R2 matrices is to permit the misalignment of a triplet. For example, an uncertainty in the positions

within the following triplet

5. 1. -8. 10. ;

5. 2. 7. 10. ;

5. 1. -8. 10. ;

may be induced by appropriate 8. elements as noted:

6. O. 1.  ;

5. 1. -8. 10. ;

6. O.  2. ;

5. 2.  7. 10. ;

5. 1 . -8. 10. ;

8. ——— ——— ——— ——— ——— ——— OOO. ;

8. ——— ——— ——— ——— ——— ——— O02. ;

8. ——— ——— ——— ——— ——— ——— O01. ;

The first 8. card in the list refers to the misalignment of the

third magnet only. The second 8 . card refers to the misalignment of the

second and third magnets as a single unit via the R2 matrix update (the

6. O. 2.  ; entry). The last 8. card refers to the misalignment of the

whole triplet as a single unit via the R1 matrix update (the 6 . O. 1 . ;

entry) -

The comments about the BEAM card (type code 1. entry) in example 2

above are applicable here also.

Example N0. 4: Misaligned quadrupoles in a triplet (pag. 62)

Individual uncertainties in the positions of the quadrupoles in the

triplet in example no. 3 above may be induced by a single misalignment as follows:

8. --- --- --- --- --- --- 015.  ;

5. 1.  -8.  10.  ;

5. 2.   7.  10.  ;

5. 1.  -8.  10.  ;

The effect of each misalignment coordinate on each quadrupole will

be stored separately in a table. This table is printed wherever a

13. 8. type code is inserted.

Mancano tabelle pag. 63, 64, 65, 66 del manuale

REPETITION: Type code 9.0 (pag.67)

Many systems include a set of elements that are repeated several

times . To minimize the chore of input preparation, a 'repeat ' facility

is provided.

There are two parameters:

1 - Type code 9.0

2 — Code digit. If non-zero, it states the number of repetitions

desired from the point it appears . If zero it marks the end

of a repeating unit.

For example, a total bend of 12 degrees composed of four 3-degree

bending magnets each separated by 0.5 metres could be represented by

9. 4. ; 4. --- ; 3. .5  ;  9. O. ; Those elements (in this case a bend

and drift) between the 9. 4. ; and 9. O. ; would be employed four times.

There is no indication of the 9.0 cards in the printed TRANSPORT

output when calculating except for the repeated listing of the elements

they control .

Vary codes may be used within a repeating unit in the usual fashion.

However all repetitions of a given varied element will be coupled.

Repeat cards may be nested four deep. By "nesting" we mean a repeat

within a repeat. An example is given below.

Example of Nesting

The total length of this sequence is:

2*(10. + 3*(29 + 4*50)+1.5) =1343

VARY CODES and FITTING CONSTRAINTS: Type code 10.0

Some (not all) of the physical parameters of the elements comprising

a beam line may be varied in order to fit selected matrix elements. In

a first—order calculation one might fit elements of the R1 or R2 transformation matrices or the beam (sigma) matrix. In second order one might constrain an element of the second—order matrix T1 or minimize the net contribution of aberrations to a given beam coordinate. Special constraints

are also available.

One may not mix orders in fitting. First order vary codes and constraints must be inserted only in a firs t—-order calculation, and similarly for second order.

The physical parameters to be varied are selected via 'Vary Codes'

attached to the type codes of the elements comprising the system. The

fitting constraints on matrix elements are selected via type code 10.0

entries placed in the system where the constraint is to be imposed.

Vary codes

Associated with each physical element in a system is a vary code

which specifies which physical parameters of the element may be varied.

This code occupies the fraction portion of the type code specifying the

element. It has one digit for each parameter, the digits having the

same order in the code as the physical parameters have on the card. A

'0' indicates the parameter may not be varied; a '1' that it may be.

For instance, 3.0 is the combined type (3) and vary code (0) for a drift

length which is to remain fixed; 3. 1 indicates a drift length that may

be varied (by the virtue of the . 1). The type code 4,010 indicates a

bending magnet with a variable magnetic field. In punching the code 3. O,

the zero need not be punched. In punching the 4.010 code, the first zero

must be punched but the second zero need not be.

First-order vary codes

In a first—order run the following parameters marked v may be varied,

those marked 0 may not be varied.

BEAM …… 		l.vvvvvvO — All components of the input beam may be

varied, except the momentum.

R.M.S. ADD1TION.. 	l.vvvvvvOO — All components of an r.m.s. addition may

be varied except the momentum change Ap.

ROTAT. . . . . . 	2.v - The pole face angle of a bending magnet may be

varied.

DRIFT... 		3.v - The drift length may be varied.

BEND. . . . . . .	4 .vvv - The length, the field, and/or the n—value may

be varied.

QUAD....... 		5.vvO - The length may be varied; the field may be,

the aperture may not be.

AXIS SHIFT.. 	7.vvvvvv — Any of the axis shift parameters may be

varied.

ALIGN.. . . . . 	8.vvvvvvO — Any of the alignment parameters may be

varied.

INITIAL 		l6.0v — Any of the three initial position floor coordi-

COORDINATES" 	nates or two angle coordinates may be varied.

MATRIX.... . 	14.vvvvvvO — Any of the first order matrix elements

may be varied.

SOLENOID. . . 	19.vv — The length and/or field may be varied.

BEAM ROTATION. . . 	20.v — The angle of rotation may be varied.

The use of the permissive 'may' rather than the imperative 'will'

in discussing variables is meaningful. The program will choose the parameters

it will vary from among those that it may vary. In general it chooses to vary those parameters that have the greatest influence upon the conditions to be fit.

Second-02"de1~ vary codes

In a second—order run the following parameters may be varied:

DRIFT. . . . . 	3.v — The drift length may be varied. Variation of a drift

length should be done with caution as it may affect the

first-order properties of the beam line . But inversely 				coupled drift spaces straddling a sextupole will, for example, 			show only second-order effects.

ϵ (1). . . . . . 	16.0v 1. — The normalized quadratic term (sextupole component)

in the midplane expansion for the field of a bending magnet

may be varied.

1/R1. . . . .. 	16.0v 12. - The pole face curvature of a bending magnet

entrance may be varied.

1/R2. . . 		16.0v 13. — The pole face curvature of a bending magnet

exit may be varied.

SEXTUPOLE 		18. Ov — The field strength may be varied.

The special parameter cards (type code 16.0) once introduced apply

to all subsequent magnets in a beam line until another type code 16. O

specifying the same parameter is encountered. Thus if such a parameter

is varied, the variation will apply simultaneously to all subsequent

magnets to which it pertains. The variation will persist until the parameter or vary code attached to the parameter is changed by the introduction of another type code 16.0 card specifying the same parameter.

Coupled vary codes

It is possible to apply the same correction to each of several

variables. This may be done by replacing the digit 1 in the vary code with one of the digits 2 through 9, or a letter A through Z. All such variables whose vary digits are the same , regardless of position will receive the same correction. For example , the three type—vary codes (5. 0A, 5. O1 , 5. 0A) might represent a symmetric triplet. The same correction will be made to the first and third quadrupoles, guaranteeing that the triplet will remain symmetric.

If a vary digit is immediately preceded by a minus sign, the computed correction will be subtracted from, rather than added to, this variable. Thus parameters with the same vary digit, one of them being preceded by a minus sign, will be inversely coupled. For example the type-vary code sequence (3.B, 5.01, 3.-B) will allow the quadrupole to move without altering the total system length.

Vary digits may also be immediately preceded by a plus sign without changing their meaning. Thus 5.0A is the same as 5.0+-A. For historical

reasons, the vary digits (9 and 4), (8 and 3), and (7 and 2) are also

inversely coupled. Inverse coupling may not be used with type codes

1.0 or 8.0.

The total number of independent variables in a first-order run is

limited to 20 by reasons of the mathematical method of fitting and to

10 for a second—order run. So far as this limit is concerned, variables

that are tied together count as one. Variables within repeat elements

(type code 9.0) also count only one.

Possible fitting constraints

A variety of possible constraints is available. Fitting may be done

in either first— or second—order, but not in both simultaneously. The

order of the constraint must be appropriate to the order of the run. A

list of constraints available is given below. They are explained more

fully on later pages.

First—order constraints

1) An element of the first—order transfer matrix R1.

2) An element of the auxiliary first—order matrix R2.

3) A σ (BEAM) matrix element.

4) The correlations r in the beam coordinates.

5) The first moments of the beam.

6) The total system length.

7) An AGS machine constraint.

8) The reference trajectory floor coordinates.

Second-order constraints

1) An element of the second—order transfer matrix T1.

2) An element of the second-order auxiliary transfer matrix T2.

3) The net contributions of aberrations to a given coordinate of the beam

matrix σ.

4) The strength of sextupoles used in the system.

The second—order matrices are actually computed using the auxiliary

matrix T2. Therefore, when activating second—order fitting, one must not

include any element which causes an update of the R2 matrix. For a complete list of such elements see type code 6.0.

The present value of the constrained quantity, as well as the desired

value , is printed in the output. In the case of transfer matrix elements

this value may be checked by printing the transfer matrix itself . Certain

other constrained quantities may be checked similarly . Exceptions are

noted in the explanations following.

R1 matrix fitting constraints

There are five parameters to be specified when imposing a constraint

upon the (i, j) element of an R1 matrix.

- Type code 10. n (specifying that a fitting constraint follows).

- Code digit (-i).

- Code digit (j).

- Desired value of the (i, j) matrix element.

- Desired accuracy of fit (standard deviation).

Note that any fitting constraint on an R1 matrix element is from

the preceding update of the R1 matrix. An R1 matrix is updated only

by a (6. O. 1.  ;)entry.

The symbol (n) is normally zero or blank. If n = 1, then entry 4 is taken to be a lower limit on the matrix element. If n = 2, entry 4 is taken to be an upper limit.

Some typical R1 matrix constraints are as follows:

| Desired optical condition | Typical fitting constraint |
| --- | --- |
| Point to point imaging: Horizontal plane R(12) = 0 Vertical plane R(34) = 0 | 10. -1. 2. 0. .0001 ‘F1’; 10. -3. 4. 0. .0001 ‘F2’; |
| Parallel to point focus: Horizontal plane R(11) = 0 Vertical plane R(33) = 0 | 10. -1. 1. 0. .0001 ‘F3’; 10. -3. 3. 0. .0001 ‘F4’; |
| Point to parallel transformation: Horizontal plane R(22) = 0 Vertical plane R(44) = 0 | 10. -2. 2. 0. .0001 ‘F5’; 10. -4. 4. 0. .0001 ‘F6’; |
| Achromatic beam: Horizontal plane R(16) = R(26) = 0 | 10. -1. 6. 0. .0001 ‘F7’; 10. -2. 6. 0. .0001 ‘F8’; |
| Zero dispersion beam: Horizontal plane R(16) = 0 | 10. -1. 6. 0. .0001 ‘F9’; |
| Simultaneous point to point and waist to waist imaging: Horizontal plane R(12) = R(21) = 0 Vertical plane R(34) = R(43) = 0 | 10. -1. 2. 0. .0001 ‘F10’; 10. -2. 1. 0. .0001 ‘F11’; 10. -3. 4. 0. .0001 ‘F12’; 10. -4. 3. 0. .0001 ‘F13’; |
| Simultaneous parallel to point and to waist transformation: Horizontal plane R(11) = R(22) = 0 Vertical plane R(33) = R(44) = 0 | 10. -1. 1. 0. .0001 ‘F14’; 10. -2. 2. 0. .0001 ‘F15’; 10. -3. 3. 0. .0001 ‘F16’; 10. -4. 4. 0. .0001 ‘F17’; |

R2 matrix fitting constraints (pag.76)

There are five parameters to be specified when imposing a constraint upon the (I, j) element of an R2 matrix.

- Type code 10.n

- Code digit –(20 + i).

- Code digit (j).

- Desired value of the (I, j) matrix element.

- Desired accuracy of fit (standard deviation).

Some typical R2 matrix constraints are as follows:

The symbol (n) is normally zero or blank. If n = 1, then entry 4 is taken to be a lower limit on the matrix element. If n = 2, entry 4 is taken to be an upper limit.

| Desired optical condition | Typical fitting constraint |
| --- | --- |
| Point to point imaging: Horizontal plane R (12) = 0 Vertical plane R (34) = 0 Parallel to point focus: Horizontal plane R (11) = 0 Vertical plane R (33) = 0 Achromatic beam: Horizontal plane R(16) – R(26) = 0 | 10. -21. 2. 0. .001 ‘F1’ ; 10. -23. 4. 0. .001 ‘F2’ ; 10. -21. 1. 0. .001 ‘F2’ ; 10. -23. 3. 0. .001 ‘F2’ ; 10. -21. 6. 0. .001 ‘F3’ ; 10. -22. 6. 0. .001 ‘F4’ ; |

See type code 6.0 for a complete list of elements which update the

R2 matrix.

σ (BEAM) matrix fitting constraints

There are five parameters to be specified when imposing a constraint

upon the (i, j) element of a σ(BEAM) matrix.

- Type code 10.n

- Code digit (i). (i ≥ j)

- Code digit (j).

- Desired value of the (i, j) matrix element.

- Desired accuracy of fit (standard deviation).

The symbol (n) is normally zero or blank. If n = 1, then entry 4 is

taken to be a lower limit on the matrix element. If n = 2, entry 4 is

taken to be an upper limit. If i = j, then the value inserted in entry  4 is

the desired bean size (σ (ii))½ e.g. x(max) = (σ (11))½ etc.

Some typical σ matrix constraints are as follows:

| Desired optical condition | Typical fitting constraint |
| --- | --- |
| Horizontal waist σ (2l) = O Vertical waist σ (43) = O Fit beam size to x(max) = 1 cm Fit beam size to y(max) = 2 cm Limit max beam size to x = 2 cm Limit min beam size to y = 1 cm | 10. 2. l. O. .001 ‘F1’ ; 10. 4. 3. O. .001 ‘F2’ ; 10. 1. 1. 1. .001 ‘F3’ ; 10. 3. 3. 2. .001 ‘F4’ ; 10.2 1. 1. 2. .01 ‘F5’ ; 10.1 3. 3. 1. .01 ‘F6’ ; |

In general, it will be found that achieving a satisfactory 'beam' fit

with TRANSPORT is more difficult than achieving an R matrix fit. When

difficulties are encountered, it is suggested that the user 'help' the

program by employing sequential (step by step) fitting procedures when

setting up the data for his problem. More often than not a "failure to

fit" is caused by the user requesting the program to find a physically

unrealizable solution. An often encountered example is a violation of

Liouville-'s theorem.

Beam correlation matrix (r) fitting constraints

Five parameters are needed for a constraint on the (i, j) element

of the beam correlation matrix.

- Type code 10.n

- Code digit (10 + i) .

- Code digit (j ) .

- Desired value of the (i, j) matrix element.

- Desired accuracy of fit (standard deviation).

TRANSPORT does not print the beam (σ) matrix directly . Instead it prints the beam half widths and represents the off-diagonal elements by the correlation matrix. If one wishes to fit an element of this matrix to a non-zero value it is convenient to be able to constrain the matrix element directly.

Some typical matrix constraints are as follows:

| Desired optical condition | Typical fitting constraint |
| --- | --- |
| Horizontal waist r(2l) = 0 yy' correlation = r(34) = 0.2 | 10. 12. 1. O. .001 ‘F1’ ; 10. 13. 4. 0.2 .001’F2’ ; |

First moment constraint

In first order, known misalignments and centroid shifts cause the

centre (centroid) of the phase ellipsoid to be shifted from the reference

trajectory, i . e. , they cause the beam to have a non—zero first moment.

The first moments appear in a vertical array to the left of the vertical

array giving the σ (ii). The units of the corresponding quantities are

the same .

It is perhaps helpful to emphasize that the origin always lies on

the reference trajectory. First moments refer to this origin. However ,

the ellipsoid is defined with respect to its centre, so the covariance

matrix, as printed, defines the second moment about the mean.

First moments may be fitted. The code digits are i = O and j ,

where j is the index of the quantity being fit. Thus 10. O. 1. .1 .01;

constrains the horizontal (1. ) displacement of the ellipsoid to be

0.1 ± 0.01 cm.

This constraint is useful in deriving the alignment tolerances of

a system or in warning the system designer to offset the element in order

to accommodate a centroid shift.

System length constraint

A running total of the lengths of the various elements encountered

is kept by the program and may be fit. The code digits are i. 0.,

j = O.

Thus the element (10. 0. O. 150. 5. ;) would make the length of the

system prior to this element equal to 150 ± 5 metres . Presumably there

would be a variable drift length somewhere in the system. By redefining

the cumulative length via the (16. 6. L. ;) element, partial system

lengths may be accumulated and fit.

AGS machine constraint*)

Provision has been made in the program for fitting the betatron phase

shift angle μ, associated with the usual AGS treatment of magnet systems.

In the horizontal plane: use code digits i = -11., j = 2., and

specify:

Δ = 12π cos-1 0.5 R11+R22 = μ2π (vert.)

= freq./(No. of periods).

In the vertical plane: i.= -13., j = 4., and

Δ = 12π cos-1 0.5 R33+R44 = μ2π (vert.)

For example, if there are 16 identical sectors to a proposed AGS

machine and the betatron frequencies per revolution are to be 3.04 and

2.14 for the horizontal and vertical planes respectively, then the last

element of the sector should be followed by the constraints:

10. -11. 2. .190 .001  ;

10. -13. 4. .134 .001  ;

i.e  3.0416 = 0.190 and 2.1416 = 0.134.

For example: A typical data listing might be:

5.01 ---   ;

3.   ---   ;

5.01 ---   ;

3.   ---   ;

10. -11. 2. 0.190 .001  ;

10. -13. 4. 0.134 .001  ;

______________________________________________________________________

*) See Courant and Snyderl). Also note that this constraint is valid

only when the unit cell structure and the corresponding beta functions are both periodic.

________________________________________________________________________

Floor coordinate fitting constraint

Five parameters are needed to specify a floor coordinate constraint:

- Type code 10.

- Code digit 8.

- Code digit (j ) .

- Desired value of floor coordinate.

- Desired accuracy of fit (standard deviation) .

The code digit (j) indicates the floor coordinate to be constrained. Its possible values are 1 to 6 indicating the floor x, y, z, theta, phi, and psi, respectively. Theta is the angle which the floor projection of the reference trajectory makes with the floor z axis. Phi is the vertical pitch. Psi is a rotation about the reference trajectory. This is also the order in which coordinates are printed in the floor layout activated by the 13. 12. element. Initial coordinates are given on type codes 16. 16. ; through 16. 20. ; and type code 20.

The floor coordinates are actually zero—-th rather than first order

properties of a beam line. However, in TRANSPORT, they may be constrained

in a first—order fitting run, and therefore are included here.

T1 matrix fitting constraints

Five parameters are needed for a constraint on the (i, j, k) element

of the second—order transfer matrix T1.

- Type code 10.0

- Code digit (-i).

- Code digit (10j + k).

- Desired value of the (i, j , k) matrix element.

- Desired accuracy of the fit (standard deviation).

Note that upper and lower limit constraints are not available for

second order fitting.

Some typical T1 matrix constraints are as follows:

| Desired optical condition | Typical fitting constraint |
| --- | --- |
| Geometric aberration T122= O Chromatic aberration T346 =.5 | 10. -1. 22. .0 .001 'Fl' ; 10. -3. 46. .5 .001 ‘F2’ ; |

There must be no updates of the R2 matrix when constraining an element

of the T1 matrix. There is no limit on the number of constraints which

may be imposed .

If no drift lengths are varied the problem will be linear and the

absolute size of the tolerances will be unimportant. Only their relative

magnitude will be significant. Sometimes only a subset of the elements

of the matrix Tijk which give significant contributions to beam dimensions

need be eliminated. In such cases one may wish to minimize the effect of

this subset, by weighing each matrix element according to its importance.

One does this by including a constraint for each such matrix element, and

setting its tolerance equal to the inverse of the phase space factor

which the matrix element multiplies. For a matrix element Tijk acting on

an uncorrelated initial phase space, the tolerance factor would be

1/(x0jx0k), where xOj and xok are the initial beam half widths specified

by the type code 1.0 card.

T2 matrix fitting constraints

Five parameters are needed for a constraint on the (i, j, k) element

of the second order auxiliary transfer matrix T2.

- Type code 10.0

- Code digit — (20 + i) .

- Code digit (10j + k) .

- Desired value of the (i , j , k) matrix element .

- Desired accuracy of the fit (standard deviation)

Note that upper and lower limit constraints are not available for

second—order fitting.

Some typical T2 matrix constraints are as follows:

| Desired optical condition | Typical fitting constraint |
| --- | --- |
| Geometric aberration T122 = O Chromatic aberration = .5 | 10. -21. 22. .0 .001 ‘F1’ ; 10. -23. 46. .5 .001 ‘F2’ ; |

By using a T2 constraint the user may fit an element of the second

order transfer matrix which pertains to any section of the beam. One

causes an R2 update at the beginning of the section with a 6. O. 2.  ;

element. One then places the T2 constraint at the end of the section.

Any number of such constraints may be imposed . This is the only second

order constraint that may be used in conjunction with an R2 update.

If a printing of the T1 matrix is requested via a 13. 4.  ; element it will be the second—order transfer matrix from the last R1 update . The

comments about phase space weighting, made in connection with the T1 constraint, are equally valid for the T2 constraint, provided the phase space

factors are obtained from the beam matrix at the position of the R2 update.

Second—order U(BEAM) matrix fitting constraint

Five parameters must be specified for a constraint on the second

order contributions to a beam matrix diagonal element σii.

- Type code 10.0

- Code digit (i) .

- Code digit (i) .

- The number 0.

- Desired accuracy of the fit (standard deviation) .

If , for example, one wished to minimize the net contributions of

second—order aberrations to the horizontal divergence, one would insert

the following card:

10. 2. 2. .0 .01 ;

The quantity that is minimized is the net increase due to second-

order terms in the second moment of the beam about the origin. This

quantity is treated as the chi—squared of the problem, so the only meaningful desired value for the fit is zero. The square root of this quantity

is printed in the output. It is computed using the R2 matrix. Therefore,

once again, one must not include any element which updates the R2 matrix.

Centroid shifts must not be inserted when doing second—order fitting,

even immediately following the beam card.

The second—order image of the initial beam centroid at some later

point in the beam is not necessarily the beam centroid at the later point.

The parameters printed by TRANSPORT are the new centroid position and the

beam matrix about the new centroid. One must therefore look at both of

these to observe the effects of the fitting procedure. It may even

happen that an improvement in one parameter will be accompanied by a

slight deterioration in the other.

The beam profile at any point is a function of the initial beam

parameters. One may therefore impose weights on the effect of the various

aberrations by the choice of parameters on the BEAM card. One might, for

example, adjust the strength of the correction of the chromatic aberrations

by the choice of the Δp/p parameter. In particular, when using a BEAM

constraint, one should not attempt to minimize or eliminate chromatic

aberrations if Δp/p is set equal to zero on the beam card (type code 1.0).

Correlations (the 12.0 card) may also be included in the initial

beam specification.

Sextupole strength constraints

Five parameters must be specified for a constraint on sextupole

strength.

- Type code 10.0

- Code digit 18.

- Code digit 0.

- The number 0.

- Desired maximum sextupole field strength.

A single sextupole constraint card applies to all sextupoles which

follow. The maximum field strength is treated as a standard deviation

and may be exceeded on an optimal fit.

One can employ this constraint to find the optimal locations for sextupoles. By placing inversely coupled drift lengths before and after the

sextupole its longitudinal position may be varied. By constraining the

field strength the sextupole can be slid to a position where the coupling

coefficients to the aberrations will be largest. One will need to experiment with adjusting the maximum field strength to achieve the best configuration.

Internal constraints

A set of upper and lower bounds on the value of each type of parameter

is in the memory of the program.

If a correction is computed for a parameter which would take its value outside this range, it is reset to the limit of the range. The current limits are:

| Type code | Limits |
| --- | --- |
| 1.0 2.0 3.0 4.0 5.0 20.0 | 0 < input; beam -60 < pole-face rotation < 60 (deg) 0 < drift 0 < magnet length 0 < quad length -360< beam rotation <360 (deg). |

These limits apply only when a parameter is being varied. Fixed

values that exceed this range may be used as desired.

These constraints were included to avoid physically meaningless

solutions.

Corrections and covariance matrix

When the program is fitting, it makes a series of runs through the

beam line. From each run it calculates the chi-squared and the corrections to be made to the varied parameters. For each iteration a single line is printed containing these quantities.

The program calculates the corrections to be made using a matrix

inversion procedure . However, because some problems are difficult , it

proceeds with caution. The corrections actually made are sometimes a

fixed fraction of those calculated. This fraction, used as a scaling

factor, is the first item appearing on the line of printed output . The

second factor is the chi-squared before the calculated corrections are

made . Following are the corrections to be made to the varied parameters .

They are in the order in which they appear in the beam line . If several

parameters are coupled, they are considered as one and their position is

determined by the first to appear.

When convergence has occurred, the final value of the chi-squared

and the covariance matrix are printed. The covariance matrix is symmetric ,

so only a triangular matrix is shown. The diagonal elements give the

change in each varied parameter needed to produce a unit increase in the

chi-squared. The off-diagonal elements give the correlations between

the varied parameters.

The appearance of the chi-squared and covariance matrix is:

*COVARIANCE (FIT X2)

C11

r12  C22

.		.

.		 .

.		  .

r1n  . . . rn,n-1 Cnn

For more details on the mathematics of the fitting, the user should

consult the Appendix. For an example of the output of the program he

(or she) should refer to the section on output format.

ACCELERATION: Type code 11.0

An energy gain is reflected in both the divergence and the width of

the beam. This element provides a simulation of a travelling wave linear

accelerator energy gain over a field free drift; length (i.e. no externally

applied magnetic field).

There are five parameters:

- Type code 11.0

- Accelerator length (metres).

- Energy gain (GeV).

- ϕ (phase lag in degrees).

- λ (wavelength in cm).

The new beam energy is printed as output.

The energy of the reference trajectory is assumed to increase linearly over the entire accelerator length. If this is not the case, an appropriate model may be constructed by combining separate 11.0 elements. An 11.0 element with a zero energy gain is identical to a drift length.

None of the parameters may be varied.

Second-order matrix elements have not been incorporated in the

program for the accelerator section.

The units of parameters 2, 3, and 5 are changed by 15. 8., 15. 11.,

and 15. 5. type code entries respectively.

Accelerator section matrix

didascalia

Definitions: L = effective length of accelerator sector

E0 = particle energy at start of sector

ΔE = energy gain over sector length

ϕ  = phase lag of the reference particle behind  the crest of the 		accelerating wave, i.e. if ϕ is positive then for some l?>0 the 			particles having this value are riding the crest of the wave; the 		units of ϕ are degrees

λ = wavelength of accelerating wave; the units of λ are those of l?, 		(normally cm).

This matrix element assumes that E0 >> m0c2(fully relativistic) .

BEAM (rotated ellipse): Type code 12.0

To allow the output beam from some point in a system to become the

input beam of some succeeding system, provision has been made for reentering the correlation matrix which appears as a triangular matrix in the beam output. (See section under type code 1,0 and/or the Appendix for definitions.)

There are 16 parameters:

1 — Type code 12.0

2 to 16 - The 15 correlations (r(ij)) among the 6 beam components -

in the order printed (by rows).

Several cards may be used to insert the 15 correlations, if necessary.

Since this element is solely an extension of the beam input, a 12.0

element; must immediately be preceded by a 1.0 (BEAM) element entry.

The effect of this element in the printed output is shown only in

the beam matrix. If the beam matrix is printed automatically, it is not

printed directly after the BEAM element but only after the correlation

matrix has been inserted.

Output PRINT CONTROL instructions: Type code 13.0

A number of control codes which transmit output print instructions

to the program have been consolidated into a single type code:

There are two parameters:

1 — Type code 13.0

2 - Code number.

The effects of the various code numbers will be described below (not in

numerical order).

Several codes are available to control various aspects of the printed

output. Most type codes produce a line of output that advertises their

existence. Those that do not, usually have an obvious effect upon the

remainder of the output and thus make their presence clear.

Beam matrix print controls 1., 2., 3.

(13. 1. ;): The current beam (σ) matrix is printed by this code.

(13. 3. ;): The beam (σ) matrix will be printed after every physical element which follows this code.

(13. 2. ;): The effect of a previous (13. 3.  ;) code is cancelled

and the beam (σ) matrix is printed only when a (13. 1. ;) code is encountered or when another (13. 3. code is inserted. The suppression of the beam matrix is the normal default.

Transformation matrix print controls 4., 5., 6., 24.

(13. 4. ;): The current transformation matrix R1 (TRANSFORM 1) is

printed by this code. If the program is computing a second—order matrix,

this second—order transformation matrix will be included in the print—out.

This matrix is cumulative from the last R1 (TRANSFORM 1) update . The

units of the elements of the printed matrix are consistent with the input

units associated with the type code 1.0 (BEAM) entry.

(l3. 6. ;): The transformation matrix R1 will be printed after

every physical element which follows this code. The second—order matrix

will be printed automatically only if the one—line form (code 13. 19. ;)

of the transformation is selected. The second-order matrix will, however,

be printed at each location of a (13. 4. ;) element. The first-order

matrix will not be repeated.

(13. 5. ;): The automatic printing of R1 will be suppressed and R1

will be printed only when subsequently requested.

(13 . 24. ;): The TRANSFORM 2 matrix, R2, will be printed by this

code. The format and units of R2 are identical with those of R1, which

is printed by the (13. 4. ;). code. For a list of elements which update

the R2 matrix, see type code 6.

The units of the tabulated matrix elements in either the first—order

R or sigma matrix or second—order T matrix of a TRANSPORT print—out will

correspond to the units chosen for the BEAM card. For example, the

R(l2) = (x/θ) matrix element will normally have the dimensions of cm/mr;

and the T(236) = (θ/yδ) matrix element will have the dimensions mr/(cm percent

Δp/p) and so forth.

Misalignment table print control 8.

The misalignment summary table is printed wherever a (13. 8. ;)

element is inserted. Its contents are the effects of all previously

specified misalignments whose results were to be stored in a table . A

full description of the table and its contents is to be found in the

section on the align element (type code 8.) .

Coordinate layout control 12.

One can produce a layout of a beam line in any Cartesian coordinate

system one chooses. The coordinates printed represent the x, y, and z position, and the angles theta, phi, and psi, respectively, of the reference trajectory at the interface between two elements. Theta is the angle which the floor projection of the reference trajectory makes with the floor 2 axis. Phi is the vertical pitch. Psi is a rotation about the reference trajectory. In the printed output the values given are those at the exit of the element listed above and at the entrance of the element listed immediately below.

A request for a layout is specified by placing a (13. 12. ;) card

before the beam card. If no additional cards are inserted the reference

trajectory of the beam line will be assumed to start at the origin and proceed along the positive z—axis. The y—axis will point up and the x-axis to the left. One can also specify other starting coordinates and orientations by placing certain other cards before the beam card. For a description of such cards see type code 16.0 (special parameters).

The calculation of the coordinates is done from the parameters of the physical elements as given in the data. Therefore, if effective lengths are given for magnetic elements, the coordinates printed will be those at the effective field boundary. The effects of fringing fields in bending magnets are not taken

into account.

General output format controls 17., 18., 19.

(13. 17. ;): The subsequent printing of the physical parameters of all physical elements will be suppressed. Only the type code and the label will remain. This element is useful in conjunction with the (13. 19. ;) element which restricts the beam (σ) matrix and the transformation (R) matrix each to a single row. The elements of these matrices then appear in uninterrupted columns in the output, similar to the TRAMP computer code used at the Rutherford Lab, CERN, and elsewhere.

(13. 18. ;): Only varied elements and constraints will be printed. This element, in conjunction with the various options on the indicator card, can produce a very abbreviated output. The entire output of a multistep problem can now easily be printed on a teletype or other terminal.

(13. 19. ;): The beam (σ) and transformation (R1 or R2) matrices, when printed, will occupy a single line. Only those elements are printed which will be non—zero if horizontal midplane symmetry is maintained. The second—order transformation matrix will obviously occupy several lines. This element, in conjunction with the l3. l7. ; element and either the 13. 3. ; element or the 13. 6. ; element, will produce output in which the printed matrix elements will occupy single uninterrupted columns.

For visual appearances it is recommended that, if both beam (σ) and

transformation matrices are desired, they be printed in separate steps of a given problem.

Punched output controls 29., 30., 31., 32., 33., 34., 35.,36.

If the control is equal to 29, all of the terms in the first—order

matrix and the x and y terms of the second-order matrix are punched.

If the control is equal to 30, all of the terms of the first—order

matrix and all second—order matrix elements are punched out.

If the control, n, is greater than 30, all of the first—order terms

are punched and the second-order matrix elements which correspond to

(n-30.), i.e. if n = 32, the second—order theta matrix elements are

punched out. If n = 31, the second—order x matrix elements are punched,

and so forth.

ARBITRARY TRANSFORMATION input: Type code 14.0

To allow for the use of empirically determined fringing fields and other specific (perhaps non-phase—space—conserving) transformations, provision has been made for reading in an arbitrary transformation matrix. The first—order 6 X 6 matrix is read in row by row.

There are eight parameters for each row of a first-order matrix

entry:

1 — Type code 14.0

2 to 7 — 	The six numbers comprising the row. The units must be those

used to print the transfer matrix; in other words, consistent

with the BEAM input/output.

8 - Row number (1. to 6.)

A complete matrix must be read and applied one row at a time. Rows

that do not differ from the unit transformation need not be read.

For example, (14. -.1.9 0. O. 0. O. 2. ;) introduces a transformation matrix whose second row is given but which is otherwise a unit matrix.

Note that this transformation does not conserve phase space because

R(22) = 0.9, i.e. the determinant of R ≠ 1.

Any of the components of a row may be varied; however, there are

several restrictions.

Type code 14.0 elements that immediately follow one another will all

be used to form a single transformation matrix. If distinct matrices are

desired, another element must be inserted to separate the type code 14.0

cards. Several do—nothing elements are available; for example, a zero

length drift (3. 0. ;) is a convenient one.

When the last of a sequence of type code 14.0 cards is read, the

assembled transformation matrix will be printed in the output.

Note that

10a21a22 a11a1201 ≠ a11a12a21a22

Hence, a matrix formed by successive 14. (3. 0. ;), 14. – elements is not always equal to the one formed by leaving out the (3. O. ;) element.

If components of a 14.0 card are to be varied it must be the last

14.0 card in its matrix. This will force a matrix to be split into

factors if more than one row has variable components.

If it is desired to read in the second-order matrix coefficients for

the 1th row, then the followlng 22 additional numbers may be read in**) .

-------------------------------------------------------------------------------- ** This feature frees the user from making repetitive, expensive, second-order runs through a fixed portion of his system while experimenting with other magnets. This is done by reading the full matrix of this portion (obtained from a previous run) back into the machine as a single "arbitrary matrix."

--------------------------------------------------------------------------------

9 – continuation code 0.

10 to 30 – the 21 coefficients:

| T(i11) | T(i12) | T(i13) | T(i14) | T(i15) | T(i16) |
| --- | --- | --- | --- | --- | --- |
| T(i22) | T(i23) | T(i24) | T(i25) | T(i26) | T(i33) |
| T(i34) | T(i35) | T(i36) | T(i44) | T8i45) | T(i46) |
| T(i55) | T(i56) | T(i66) |  |  |  |

in that order, where i is the row number. It is necessary to read in 		the first-order matrix row which corresponds to the second—order 		matrix row 	being read in.

As in the first—order case, full rows not different from the

identity matrix [i.e., R(ii) = 1, all other R(ij) = 0, and all T(ijk) = 0] need not be read in.

Input-output UNITS: Type code 15.0

TRANSPORT is designed with a standard set of units that have been

used throughout this manual. However, to accommodate other units conveniently, provision has been made for redefining the units to be employed.

This is accomplished by insertion of one or more of the following elements .

There are four parameters to be specified:

- Type code 15.0

- Code digit.

- The abbreviation of the unit (see examples below) .

This will be printed on the output listing. It must be enclosed in single quotes and is a maximum of three characters long (four for energy). The format for insertion is the same as for labels.

- The scale factor (if needed).

The scale factor is the size of the new unit relative to the standard TRANSPORT unit . For example , if the new unit is inches and the standard TRANSPORT unit cm, the scale factor is (2.54).

The various units that may be changed are:

| Code Digit | Quantity | Standard TRANSPORT Unit | Symbols used in SLAC-75 |
| --- | --- | --- | --- |
| 1.0 2.0 3.0 4.0 5.0 6.0 7.0 8.0 9.0 10.0 11.0 | horizontal and vertical transverse dimensions, magnet apertures and misalignment displacements. horizontal and vertical angles and misalignment rotation angles vertical beam extent cm y (only)*) and bending magnet gap height vertical beam divergence*) (only) pulsed beam length and wave length in accelerator momentum spread bend, pole face rotation, and coordinate layout angles length (longitudinal) of elements, layout coordinates and bending magnet pole face curvatures magnetic fields mass momentum and energy gain in accelerator section | cm mr cm mr cm percent (PC) degrees (DEG) metres (M) kG electron mass GeV/c GeV | x,y θ , ϕ y ϕ ℓ δ t Β m p(O) ΔΕ |

*) These codes should not be used if the coordinate rotation (20.0) type code is used anywhere in the system.

Units are not normally restored at the end of a problem step . Once changed, they remain the same for all succeeding problem steps in an input deck until a 0 indicator card is encountered, at which time they are re

set to standard TRANSPORT units. The units may be reset to standard units

by inserting a (15. ;) type code entry.

The 15.0 elements are the first cards in a deck (immediately following the title card and the O or l indicator card) and should not be inserted in any other location. They produce no printed output during the calculation, their effect being visible only in the output from other elements .

Exemple: To change length to feet , width to inches , and momentum

to MeV/c, add to the front of the deck the elements

| 15. | 8. | ‘ FT’ | 0.3048; |
| --- | --- | --- | --- |
| 15. | 1. | ‘ IN’ | 2.54; |
| 15. | 11. | ‘ MEV’ | 0.001; |

The scale factor, 0.3048, multiplies a length expressed in the new unit,

feet, to convert it to the reference unit, metres, etc.

For the conventional units listed below, it is sufficient to stop with the unit name (the conversion factor is automatically inserted by the program) . If units other than those listed below are desired, then the unit name and the appropriate conversion factor must be included .

If the automatic feature is used with older versions of the program, there

must be no blank spaces between the quotes and the unit name.

Input-output units: Type code 15.0

(Conversion fact0rs for dimension changes versus code digit and label)

SPECIAL INPUT PARAMETERS: Type code 16.0

A number of constants are used by the program which do not appear as parameters in elements of any other type code. A special element has been provided to allow the designer to set their values. These special parameter entries must always precede the physical e1ement(s) to which they apply. Once introduced, they apply to all succeeding elements in the beam line unless reset to zero or to new values.

There are three parameters:

1 — Type code 16.0

2 — Code digit.

3 — Value of the constant.

A number of such constants have been defined in this manner.

All have a normal value that is initialized at the beginning of each run.

Code digits for special parameters

| 1. | ϵ(1) | a second—order measure of magnetic field inhomogeneity in bending magnets . If B(x) = B(0) 1-nxρ0+ βxρ02-… is the field expansion in the median (y = 0) plane, then ϵ(1) is defined as ϵ(1) = β 1ρ02 (where ρ0 is measured in units of horizontal beam width — normally cm). This parameter affects second—order calculations only. Normally the value if 0. It may be varied in second—order fitting. |
| --- | --- | --- |
| 3. | (M/m) | Mass of the particles comprising the beam, in units of the electron mass; normally 0. A non-zero mass introduces the dependence of pulse length on velocity, an important effect in low-energy pulsed beams. |
| 4. | W/2 | Horizontal half-aperture of bending magnet, in the same units as horizontal beam width, normally 0 (i.e. effect of horizontal half aperture is ignored). |
| 5. | g/2 | Vertical half—aperture of bending magnet, in the same units as vertical beam height; this parameter must be inserted if the effect of the spatial extent of the fringing fields upon transverse focusing is to be taken into account. (See type codes 2.0 and 4.0 as a cross reference) normally 0. |
| 6. | L | Cumulative length of system, in the same units as system length. It is set to zero initially, then increased by the length of each element, and finally printed at the end of the system. This element allows the cumulative length to be reset as desired. |
| 7. | K1 | An integral related to the extent of the fringing field of a bending magnet. See section under type code 2.0 and SLAC-75 page 74 for further explanation. If the (16. 5. g/2. ;) element has been inserted, the program inserts a default value of K1 = 12 unless a (16, 7, K1. ;) element is introduced, in which case the program uses the K1 value selected by the user. The table below shows typical values for various types of magnet designs. |
| 8. | K2 | A second integral related to the extent of the fringing field. Default value of K2 = 0 unless specified by a (16. 8. K2 . ;) entry. |

Typical values of K1 and K2 are given below for four types

of fringing field boundaries:

- a linear drop-off of the field,

- a clamped "Rogowski" fringing field,

- an unclamped "Rogowski" fringing field,

- a "square-edged" non-saturating magnet

| Model | K1 | K2 *) |
| --- | --- | --- |
| Linear Clamped Rogowski Unclamped Rogowski Square—edged magnet | 1/6 0.4 0.7 0.45 | 3.8 4.4 4.4 2.8 |

12.	1/R1	—Where R1 is the radius of curvature (in units of longitudinal

length, normally metres) of the entrance face of bending magnets.

(See figure on p.

13, 	1/R2 	-Where R2 is the radius of curvature (in units of longitudinal

length, normally metres) of the exit face of bending magnets.

(See figure on p.

The pole face curvatures (1/R1) and (1/R2) affect the system only in second-order, creating an effective sextupole component in the neighbourhood

of the magnet. If the parameters are not specified, they are assumed to be

zero, i .e. no curvature and hence no sextupole component. Either parameter

(or both) may be varied in second-order fitting.

--------------------------------------------------------------------------------

*)For most applications K2 is unimportant. If you find it is important to your result you should probably be making a more accurate calculation with a ray—tracing program (see References at the end of the manual.)

--------------------------------------------------------------------------------

FIELD BOUNDARIES FOR BENDING MAGNETS

The TRANSPORT sign conventions for x, 	β, R and h are all positive as shown in the figure. The positive y direction is out of the paper. Positive β's imply transverse focusing. Positive R's (convex curvatures) represent negative sextupole components of strength s = (- h/2R) sec3 β. (See SLAC—75, page 71.)

Tilt-to-focal plane (16.  15.  α. ;) element (pag.106)

Very often it is desired to have a listing of the second—order

aberrations along the focal plane of a system rather than perpendicular

to the optic axis, i.e. along the x coordinate. If the focal plane makes

an angle α with respect to the x axis (measured clockwise) then

provision has been made to rotate to this focal plane and print out

the second-order aberrations. This is achieved by the following procedures:

Alpha is the focal-plane tilt angle (in degrees) measured from the

perpendicular to the optic axis (α is normally zero) .

The programming procedure for a tilt: in the x(bend)-plane (rotation

about y axis) is:

16. 15. α. ;

3.  O. ; (a necessary do—nothing element)

13.  4. ;

16. 15. -α.  ; (rotate back to zero)

3.  O. ; (a necessary do—nothing element)

16. 15. O. ; (to turn off rotation element)

The programming procedure for a tilt in the y—plane (rotation about x-axis) is:

16. 15. α. ;

20. 90.  ;

3.  O.  ;

20. -90. ;

13.  4.  ;

16. 15. –α. ; (rotate back to zero)

3.  O. ;

16. 15. O. ; (to turn off rotation element)

Initial beam line coordinates and direction

When requesting a beam line coordinate layout via a (13. 12. ;)

element one can employ any coordinate system one desires. The position and direction of the beginning of the reference trajectory in this

coordinate system are given on elements 16. 16. through 16. 20. Such

cards should be placed before the beam card, but after any units changes.

Their meanings are as follows:

l6. 	16.	x0 , y0, and z0, respectively, the coordinates of

16. 	17. 	the initial point of the reference trajectory

16. 	18.	in the units chosen for longitudinal length.

16. 	l9.	Θ0 and ϕ0 the initial horizontal and vertical angles

16. 	20. 	of the reference trajectory in degrees.

When specifying the initial orientation of the reference trajectory

via the two angles, one must give the horizontal angle first. The

meaning of the two angles is given in the following figure. Any of the

above five parameters not explicitly specified will be taken to equal zero.

The initial coordinates may be varied in first—order fitting. Their

values will affect only the beam line floor coordinates and not any beam

or transfer matrix element.

REFERENCE TRAJECTORY

SPECIFICATION OF INITIAL ANGLES θ0 AND ϕ0 FOR BEAM LINE LAYOUT.

SECOND-ORDER CALCULATION: Type Code 17.0

A second—order calculation may be obtained provided no alignments are

employed. A special element instructs the program to calculate the second-

order matrix elements . It must be inserted immediately following the beam

(1. element).

Only one parameter should be specified:

1 - Type coder 17.0 (signifying a second—order calculation is to be

made).

To print out the second order T1 matrix terms at a given location in

the system, the (13. 4.  ;) print control card is used. For T2, the

(13. 24. ;) print control card is used. The update rules are the same

as those for the corresponding first-order R matrix. See SLAC—75 for

definitions of subscripts in the second order T(ijk) matrix elements.

The values of the BEAM (sigma) matrix components may be perturbed

from their first-order value by the second—order aberrations. In a

second-order TRANSPORT calculation, the initial beam is assumed to have

a Gaussian distribution. For exact details the reader should consult

the Appendix. For the beam matrix to be calculated correctly, there

should be no elements which update the R2 matrix. If a centroid shift is

present , it must immediately follow the beam (type code 1 .0) or beam

rotated ellipse (type code 12.0) card.

Only second-order fitting may be done in a second-order run. See

the section on type code 10 .0 for a list of quantities that may be constrained

in a second—order run. If a beam constraint is to be imposed in

second—order, there must be no centroid shifts present anywhere.

Second—order matrices are included in the program for quadrupoles ,

bending magnets (including fringing fields) , the arbitrary matrix, sextupoles, and solenoids. They have not been calculated for the acceleration (type code 11.0) element.

SEXTUPOLE: Type code 18.0

Sextupole (hexapole) magnets are used to modify second-order aberrations in beam transport systems . The action of a sextupole on beam particles is a second and higher order effect, so in first order runs (absence of the 17.0 card) this element will act as a drift space.

There are four parameters:

1 — Type code 18.0

2 - Effective length (metres).

3 — Field at pole tips (kG). Both positive and negative

fields are possible (see figures below).

4 — Half-aperture (cm). Radius of circle tangent to pole tips.

Other orientations of the sextupole may be obtained using the beam

rotation element: (type code 20.0).

The pole tip field may be varied in second—order fitting. It may

also be constrained not to exceed a certain specified maximum field.

(See the explanation of vary codes in the section on type code 10.0).

Such a constraint allows one to take into account the physical realities

of limitations on pole tip fields.

See SLAC—75 for a tabulation of sextupole matrix elements. The

TRANSPORT input format for a typical data set is:

Label if desired (not to exceed

4 spaces)

18.   L.  b.  a.  ‘ ‘  ;

DIPOLE QUADRUPOLE SEXTUPOLE

ILLUSTRATION OF THE MAGNETIC MIDPLANE (X AXIS) FOR DIPOLE, QUADRUPOLE AND SEXTUPOLE ELEMENTS. THE MAGNET POLARITIES INDICATE MULTIPOLE ELEMENTS THAT ARE POSITIVE WITH RESPECT TO EACH OTHER,

SOLENOID: Type code 19.0

The solenoid is most often used as a focusing element in systems

passing low-energy particles. Particles in a solenoidal field travel

along helical trajectories. The solenoid fringing field effects necessary to produce the focusing are included.

There are three parameters:

1 — Type code 19.0

2 - Effective length of the solenoid (metres) .

3 — The field (kG). A positive field, by convention, points in the

direction of positive z for positively charged particles.

The length and the field may be varied in first—order fitting. Both

first— and second-order matrix calculations are available for the solenoid.

A typical input format is:

Label if desired (not to exceed

4 spaces)

19.   L.  B.  ‘ ‘  ;

First-order solenoid matrix

Solenoid R matrix

Definitions:	L = effective length of solenoid

K = B(O)/(2Bρ0),where B(O) is the field inside the solenoid and (Bρ0) 		is the (momentum) of the central trajectory.

C = cos KL

S = sin KL

For a derivation of this transformation see report SLAC-4 by R. Helm.

Alternate forms of matrix representation of the solenoid:

C2 	 1kSC	 SC	  1kS2 	 0	 0

-KSC	 C2 	 -KS2 	 SC 	 0	 0

-SC	 - 1kS2	  C2 	  1kSC 	 0	 0

R(Solenoid) =

KS2 	 -SC 	 -KSC	 C2	 0	 0

0 	 0 	 0	 0 	 1 	 0

0	 0 	 0	 0 	 0 	 1

Rotating the transverse coordinates about the z axis by an angle

= -KL decouples the x and y first-order terms, i.e.

C 	 1kS	 0	 0	 0 	 0

-KS	 C	 0 	 0 	 0	 0

0	 0	  C 	  1kS 	 0	 0

R(-KL)R(Solenoid)

0	 0 	 -KS	 C	 0	 0

0 	 0 	 0	 0 	 1 	 0

0	 0 	 0	 0 	 0 	 1

COORDINATE ROTATION: Type code 20.0

The transverse coordinates x and y may be rotated through an angle α: about the z axis (the axis tangent to the central trajectory at the point

in question)** ). Thus a rotated bending magnet, quadrupole, or sextupole may be inserted into a beam transport system by preceding and following the element with the appropriate coordinate rotation. (See examples below.) The positive sense of rotation is clockwise about the positive z axis.

There are two parameters to be specified for a coordinate rotation:

1 - Type 20.0 (signifying a beam coordinate rotation).

2 — The angle of rotation α (degrees).

The angle of rotation may be varied in a first-order fitting (see

type code 10.0).

Note

This transformation assumes that the units of (x and y) and (θ and ϕ)

are the same. This is always true unless a 15.0 3.0 or a 15.0 4.0 type

code has been used.

-------------------------------------------------------------------------

**) See SLAC-'754), page 45 and Fig. 4, page 12 for definitions of x, y,

and z coordinates.

-------------------------------------------------------------------------

Examples

For a bending magnet, the beam rotation matrix may be used to

specify a rotated magnet.

Examples No. 1

A bend up is represented by rotating the x, y coordinates by -90.0

degrees as follows:

Labels (not to exceed 4 spaces) if desired

20.  -90.  ‘  ‘  ;

2.   β(1). ‘  ‘  ;

4.   L.  B.  n. ‘  ‘  ;

2.   β(2). ‘  ‘  ;

20.  +90.  ‘  ‘  ; (return coordinates to their initial orientation)

A bend down is accomplished via at +90 degree rotation.

20.  +90.  ‘  ‘  ;

2.

4.

2.

20.  -90.  ‘  ‘  ;

A bend to the left (looking in the direction of beam travel) is

accomplished by rotating the x, y coordinates by 180 degrees, e.g.

20.  180.  ‘  ‘  ;

2.

4.

2.

20. -180.  ‘  ‘  ;

Example No. 2

A quadrupole rotated clockwise by 60 degrees about the positive z

axis would be specified as follows:

20. 60. ' ' ;

5. L. B. a. ' ' ;

20. -60. ' ' ;

Beam rotation matrix

C		 S

C		 S

-S		  C

R=

-S 		 C

1

1

where 	C = cos α,

S = sin α,

α = angle of coordinate rotation about the beam axis,

blank spaces are zeros.

e.g. for α = +90 degrees, this matrix interchanges rows 1 and 2 with 3 and 4 of the accumulated R matrix as follows:

00100001-10000-100  R(11)R(12)R(13)R(14)R(21)R(22)R(23)R(24)R(31)R(32)R(33)R(34)R(41)R(42)R(43)R(44)  R(31)R(32)R(33)R(34)R(41)R(42)R(43)R(44)-R(11)-R(12)-R(13)-R(14)-R(21)-R(22)-R(23)-R(24)

(The rest of the matrix is unchanged.)

STRAY MAGNETIC FIELD: Type code 21.0

1 - Element No. 21.0

2 - Code No. n. 	n = 4: horizontal deflection

n = 2: vertical deflection.

3 — <BL>		mean value of Bdz.

4 — ± <σ BL> 	+: Gaussian random number generator;

affects beam first moment.

-: uncertainty in Bdz affects beam

second moment.

Uses the misalignment element (8.) to calculate an angular deflection

equal to Bdz(Bρ)

This type code is not functioning in the present version of the

program.

SENTINEL

Each step of every problem in a TRAWSPORT data set must be terminated

with the word SENTINEL. The word SENTINEL need not be on a separate card.

For a description of the form of a TRANSPORT data set see the section on

input format .

An entire run, consisting of one or several problems, is indicated

by an additional card containing the word SENTINEL. Thus, at the end of

the entire data set the word SENTINEL will appear twice.

Acknowledgements

R. Helm's suggestions and criticisms at SLAC have been invaluable

throughout the development of the program and the underlying theory.

R. Pordes has ably assisted D. Carey at FNAL during the more recent

developments of the program.

REFERENCES

- E.D. Courant and 11.8. Snyder, Theory of the Alternating Gradient

Synchrotron, Ann. Phys. (NY) 2, 1-48 (1958).

- S. Penner, Calculations of properties of magnetic deflection systems, Rev. Sci. Instrum. 150-160 (1961).

- K.L. Brown, R. Belbeoch, and P. Bounin, First— and second-order magnetic optics matrix equations for the midplane of uniformfield wedge magnets, Rev. Sci. Instrum. (1964).

- K.L. Brown, A first-- and second-order matrix theory for the design

of beam transport systems and charged particle spectrometers, SLAC Report 	No. 75, or Advances Particle Phys. L, 71—134 (1967).

- K.L. Brown and S.K. Howry , TRANSPORT/360 a computer program for

designing charged particle beam transport systems , SLAC Report No. 91 	(1970) . The present manual supersedes the above reference.

- K.L. Brown, A systematic procedure for designing high resolving

power beam transport systems or charged particle spectrometers, Proc. 3rd 	Int. Conf. on Magnet Technology, Hamburg, Germany, May 1970, p. 348, or 	SLAC—PUB-762 June 1970.

- Suggested Ray—Tracing Programs to supplement TRANSPORT:

David C . Carey, "TURTLE (Trace Unlimited Rays Through Lumped Elements)" , 	Fermilab Report No . NAL-64 (1971) . This is a computer program using 	TRANSPORT notation and designed to be run using the same data cards as for 	a previous TRANSPORT run.

K.L. Brown and Ch. Iselin, "DECAY TURTLE (Trace Unlimited Rays Through 	Lumped Elements)", CERN Report 74-2 (1974) . This is an extension of TURTLE 	to include particle decay calculations.

H. Enge and S . Kowalski have developed a Ray—Tracing program using 	essentially the same terminology as TRANSPORT. Any experienced user of 	TRANSPORT should find it easy to adapt to the M.I.T. program.

8. K.G. Steffen, High—energy beam optics, Interscience Monographs and

Texts in Physics and Astronomy, Vol. 17, John Wiley and Sons, New York 	(1965).

SUGGESTED BIBLIOGRAPHY

A.P. Banford, The transport of charged particle beams (E. and

F.N. Spon Ltd., London, 1966).

K.G. Steffen, High—energy beam optics, Interscience Monographs and

Texts in Physics and Astronomy, Vol. 17 (John Wiley and Sons, NY, 1965).

TRANSPORT APPENDIX

K. L. Brown and F. Rothacker

Stanford Linear Accelerator Center, Stanford, California

- C. Carey

- Fermi National Accelerator Laboratory, Batavia, Illinois

and

Ch. Iselin

CERN, Geneva, Switzerland

December 1977

APPENDIX

Table of Contents

Introduction

1 — Beam Transport Optics (a set of lectures given to the

SLAC technical staff)

Part I

Introduction

Geometric Light Optics vs. Magnetic Optics

Introduction of Momentum Dispersion into the Matrix Formalism  		Second-Order Matrix Formalism

Transformations Involving Many Trajectories

Part II

Introduction

First—Order Transformation Matrix

Beam Switchyard

TRANSPORT Notation

Second—Order Matrix

2 — First—Order Matrix Formalism for TRANSPORT

First—0rder R(Transfer) Matrix Formalism

First—Order Dispersion

First:—Order Path Length

Achromaticity

Isochronicity

First—-Order Imaging

Focal Lengths

Zero Dispersion

Magnification

First—0rder Momentum Resolution

First—Order Phase Ellipse Formalism for TRANSPORT

Description of the Sigma BEAM Matrix

The Phase Ellipse Matrix used by TRANSPORT

Physical Interpretation of Various Projections of the

2—dimensional BEAM Ellipse

The Upright Ellipse

Relationship between a Waist and a Para1lel—to—Point Image  			Relationship between a Waist and Point—to-Point Image  				Relationship between a Waist and the Smallest Spot Size

Achievable at a Fixed Target Position

Imaging from an Erect Ellipse to an Erect Ellipse

Relationship between a First-Order Point—to—Point Image and the

Minimum Spot Size Achievable at a Fixed Target Position

Orientation of the Major Axes of a Phase Space Ellipse

Table of Contents — (continued)

3 — Second—Order Aberrations

Second Order Contributions to Beam Dimensions

I Introduction

II The Ellipsoid Formalism

III The Effect of a Beam Line

IV Off—Axis Initial Distribution

References

A Systematic Procedure for Designing High Resolving Power Beam

Transport Systems or Charged Particle Spectrometers

I 	Introduction

II 	Theory

Multipole Strengths for Pure Multipole Fields	Multipole Strengths for a Non-Uniform Field Expansion

Multipole Strengths for a Contoured Entrance or Exit Boundary of a Magnet

The Description of the Trajectories as a Taylor's Expansion

III	Solution of the Equation of Motion

IV	Interpretation and use of the Coupling Coefficient

V	A Systematic Procedure for Designing High Resolution Systems

First-Order Considerations

First—Order Resolving Power

Dispersion

The Selection of the Optical Mode

Aberrations and their Correction

References

4 — The Effect of Beam Line Magnet Misalignments

I	Introduction

II	Particle Trajectory Coordinates

III	Magnet Misalignment Coordinates

IV 	Transformation of Particle Trajectory Coordinates

V	Evaluation of the Relevant Matrices

VI	Effect on the Beam Envelope

VII	Implementation

5 — First—Order Parameter Optimization and Covariance

APPENDIX

Introduction

This appendix has been included as an addition to the manual in an

attempt to better acquaint the user with ‘what TRANSPORT does‘, and with the

notation and mathematical formalism used in a TRANSPORT calculation.

The first section (Beam Transport Optics - Part I and Part II) is a

rewrite of two lectures given to members of the SLAC technical staff on the

elementary matrix algebra of optics. We include them here for the benefit of

the new user who may need a brief refresher course on charged particle optics

and/or has a need to become familiar with TRANSPORT notation. The new user

should also acquaint himself with the contents of the books and other publications

listed under 'references' at the end of the manual. References l and 2 are

essential if the user is to obtain the maximum value from TRANSPORT.

The second section of this appendix was written to introduce the mathematical formalism of the first-order R matrix and Sigma matrix (phase ellipsoid) beam optics used in a TRANSPORT calculation and to correlate this with the printed output.

Section three discusses second—order calculations and, in particular, a

procedure for calculating the "Sextupole" strengths required to minimize and/or

eliminate second—order aberrations in a beam transport system.

Section four is a brief derivation of the mathematical formalism used by

TRANSPORT for calculating magnet alignment tolerances.

Section five deals with the first-order parameter optimization code of

TRANSPORT and includes a brief explanation of the covariance matrix that is

printed after each first-order fit routine.

BEAM TRANSPORT OPTICS

Section I

Beam Transport Optics - Part I

(K. L. Brown)

1 . Introduction

A convenient starting point for this lecture is the equation relating

the magnetic rigidity of a particle (Bρ) to the particle momentum P.

Bρ= 1022.99793 P  or Bρ= 33.356 P

where

B 	is in kilogauss

ρ 	is the bending radius in meters

P 	is the particle 's momentum in BeV/c.

A note of caution: When using this equation for a TRANSPORT calculation, it is necessary to use at least 5 significant figures for the constant to

avoid round-off errors in the readout.

2. Geometric Light Optics vs.-Magnetic Optics

To relate geometrical light optics to charged particle optics, we

begin with the thin lens. Figure l shows a thin lens with a ray leaving

a focal point A at an angle θ0 , impinging on the lens at xo . As the ray leaves the lens, it is at x1 and going toward a focal point B at an

angle of θ1.

Figure l

Thin lens optics says that 1/p + 1/q = 1/f. Using this equation it is

readily verified that the matrix transformation for the lens action between

principal planes is

x1e1 = 10-1/f1 x0θ0 (nella parentesi centrale

manca croce divisoria)

The transformation for a drift distance L is

x1θ1 = 1001 x0θ0

Note that the determinant of the matrix in both examples is equal to

unity. This is always the case as will be proved formally later. That this

is so is a manifestation of Liouville's theorem of conservation of phase

spa ce area

Consider now a thick lens, as illustrated in Figure 3.

Figure 3

If L1 is the object distance to the face of the lens and L2 is the

corresponding image distance, then, in general,

1/L1 + 1/L2 ≠ 1/f.

If, however, we introduce two planes P1 and P2 located at a distance

z1 and z2 from the entrance and exit faces of the lens, it is always

possible to find a z1 and a z2 such that the equation

1/p = 1/1q = 1/f is valid.

Where

P = L1 + z1

q = L2 + z2

When this is so, P1 and P2 are called the principal planes of the lens.

Now, relating the above statement to matrix formalism, the matrix

transformation for a thick lens between the input and output faces of the

lens has the general form:

x1θ1 = R11R12R21R22 x0θ0          (1)

where as before the det R = l . For a general transformation, R12 is not necessarily equal to O and R11 and R22 are not necessarily equal to l .

The principal planes may be located by the transformation

R11R12R21R22 = 1z201 10-1/f1 1z101 (2)

Using the relation

1z01 1-z01= 1001  = 1 (the unit matrix)

the previous equation may be manipulated into the form

R12-z1R11R11-z2R21-z2R22-z1R21R21R22-z1R21 =   1z201 R11R12R21R22 1-z101 = 10-1/f1  (3)

(Manca ripartizione nella prima parte)

Solving for zl and z2 , we find

Z1 = R22- 1R21

Z2 = R11- 1R21

where z1 and z2 are the location of the principal planes as shown in

Fig. 3. The principal planes of any system may be determined by this method.

Note that R21 = -1/f is not affected by the transformation and that the upper right hand matrix element is zero if det R = 1 . The principal

planes may coincide, may be close together, be far apart; or in many systems,

may be located external to all of the elements comprising the system. An

example of the latter case is a quadrupole pair.

Some examples of principal plane locations for simple systems follow:

A quadrupole singlet:

P1        P2

Figure 4

The principal planes in a single quadrupole are located very close

to each other and very near the center of the lens. As such, a quadrupole

singlet may be considered as a thin lens if the object and image distances

are measured to the center of the lens.

A simple uniform-field wedge magnet:

If the optic axis enters and exits perpendicularly to the pole boundary, the principal planes are at the "center" of the magnet, as shown in Figure 5.

From this, we conclude that a simple wedge bending magnet may be considered

as a 'thin" lens if the object and image distances are measured to the lens

center 0.

A quadrupole pair

For a quadrupole pair, the principal planes are displaced toward and,

usually, beyond the focusing element of the pair, as shown in Figures 6 and 7.

For any lens system, no matter how many elements are involved

object

p		   P1		P2	    q

Figure 8				   image

1/p+ 1/q = 1/f, if p and q are distances measured to the principal

planes. Then the magnification between object and image planes is M = q/p.

Since the quadrupole pair is different in the two planes, (x) and (y),

both situations must be examined. The interesting result turns out to be

that in the x plane, the principal planes are to the right (Figure 6) and

in the y plane, they are to the left (Figure 7). Therefore, in the y plane

the magnification is greater than 1, and in the x plane the magnification

is less than 1.. Typically, for a quadrupole pair the ratio of

My/Mx

may be as high as 20:1 and such cases can be disastrous if not recognized

beforehand. This is a first-order image distortion. For example, if the

source is a circular spot at A , the image at B will appear as a long

thin line.

The situation is different for the

Quadrugole triplet:

zx						zx

A                                                                 B Mx ≅  1

P1 				P2

x plane			Figure 9

Zy						zy

A                                                                 B Mx ≅ 1

P1 				P2

Y plane 			Figure 10

In the symmetric triplet, as shown in Figures 9 and 10, the principal

planes are located symmetrically about the center of the system, although

zx > zy . This is, perhaps, the dominant reason why quadrupole triplets are used. The magnification is approximately equal in both planes; consequently, a circular spot can be imaged through the system with much less

first-order image distortion than is the case for the doublet.

3. Introduction of Momentum Dispersion into the Matrix Formalism

The foregoing discussion and examples dealt only with monoenergetic

first-order effects. First-order dispersion may be taken into account by

introducing a 3 X 3 matrix as follows:

Consider two particles of momentum po and po + Δp passing through

the midplane of a static magnetic field, as illustrated in Figure ll.

Figure ll

Since the scalar momentum of a particle is constant in a static magnetic

field, the transport equation from A to B may be expressed as:

x1θ1δ = R11R12dR21R22d'001 x0θ0δ 	manca quadrettatura nella parte centrale

A

Where

δ = Δp/p0

d = the spatial momentum dispersion

d’= the derivative of the dispersion (the angular momentum dispersion)

and

1 = a carrying tern to generate a square matrix and denote a constant 	momentum.

The determinant of the matrix  R  is equal to l as for the 2 x 2 matrix. However, because of the zeros in the bottom row, the fact that  R = (R11 R22 –R12 R21 ) = 1 only checks the 2 x 2 matrix and not the terms containing d and d'.

Consider now a general system from an object point A to an image point

B.

The above matrix equation is still valid for midplane trajectories.

If A is a source point and if R12 = 0 (i.e., x is independent of θ0), then B is an image point for monoenergetic particles.

Under these circumstances:

R11 = Mx is the x plane magnification

R22 = +1/Mx because det R = l

And

R21 = - 1/1fx

In fact, R21 = - 1/fX for the system between A and B , even if

A and B are not foci.

It is now convenient to develop a more general definition of the matrix

elements Rij and, at the same time, introduce the first-order matrix transformation for the y(non-bend) plane. Consider, again, a general system

where the projection of the central trajectory is allowed to bend in the

x plane but is a straight line in the y plane. The x plane and y plane matrix

transformations may be written as follows:

For the x plane x1 = Rx x0

or    x1θ1δ = cx(t)sx(t)dx(t)c'x(t)s'x(t)d'x(t)001 x0θ0δ  (manca quadrettatura nella parte centrale)

Similarly, for the Y plane Y1 = Ry y0

Or     y1ϕ1 = cy(t)sy(t)c'y(t)s'y(t) y0ϕ0   (manca quadrettatura nella parte centrale)

The c and s functions may be defined in terms of their initial conditions.

Let τ be the distance measured along the

central trajectory. Then:

s(0) = 0 	s’(0) 	= 1  where	s’(τ) = ds(τ)dτ

c(0) = 1	c’(0) = 0		c’(τ) = dc(τ)dτ =-1f

Within an "ideal" magnet, where the bending radius ρ0  is constant, s and

c are sine and cosine or else sinh and cosh functions. Because of

this, the terminology s = a sine-like function and c = a cosine-like

function has been adopted for describing the general case where ρ0 = ρ0(τ) is a function of τ.

By analogy with previous discussions, we observe that whenever s(τ) = O,

we are at an image of point A. Also, at the position where s(τ) = O

is the magnification of point A at that image.

c'(t) = - l/f where f is the focal length of the system between A

and B. The dispersion dx may be derived from the general differential equations of motion of a charged particle in a static magnetic field.(l)

-----------------------------------------------------

(1) See SIAC-75 for a derivation of these equations.

-----------------------------------------------------

The results may always be expressed as a function of sx and cx as follows:

dx(t) = sx(t) otcλ(τ)dα  - cx(t)otsx (τ)dα

and

d’x(t) = s’x(t) otcx(τ)dα  - c’x(t)otsx (τ)dα

where

dα  = dτρo(τ)

is the differential angle of bend of the central trajectory. At an image point [s(t) = o] note that

dx(t) = - c(t)ot s(τ)dα

This approach to the problem may be generalized to include all of the

second-order aberrations of a system. When this is done, it is always

possible to express these aberrations as functions of the first-order

matrix elements cx , sx ,dx ,cy           and sy .

Having developed the above physical concepts and mathematical tools,

we are now in a position to study more complicated systems. As an example,

we consider the general system shown in Figure 14

L1		L2 		L3 		L4

M2

Q1

M1

A                                                  B

Figure 14

L 	= drift elements

= magnetic elements

The matrix formalism states that in the x plane, the transformation

from A to B is given by the following matrix equation.

x1θ1δ = RL4  Rz RL3 Rq1 RL2 R1 RL1 x0θ0δ

(parte centrale scritta in modo impreciso)

As in all matrix calculations, the order of writing down the elements

comprising the system is from right to left. The individual matrix elements

must be derived from the solution of the equation of motion within each element.

If this has been done, then the calculation for the total system is carried

out in the fashion shown by the above equation.

4. Second-Order Matrix Formalism (1) (pag.139)

It is possible to extend the 3 x 3 matrix formalism to solve simultaneous sets of power series by generating a second-order matrix equation as follows:

The "R12" term is obtained by squaring the upper left corner (3 X 3)

matrix so as to obtain second-order equations for x12 , x1θ1, x1 δ, etc.,

as functions of products of the initial first-order variables x0, θ0, and δ.

This is, then, a convenient mathematical formalism for keeping all the

terms desired and dropping those undesired. In the above example, all first-

and second-order terms are retained and all higher-order terms are automatically

dropped by the matrix multiplication.

5. Transformations Involving Many Trajectories

All of the discussion to this point relates to the transformation of

a single trajectory (in addition to the central trajectory) through a static

magnetic system. We wish now to extend the discussion to include "bundles"

of rays. To accomplish this, we take advantage of Liouville's Theorem, which

states that the "phase space" is conserved through the system. While the

Theorem is strictly true to all orders, a convenient mathematical transformation

has only been developed to first-order. A manifestation of Liouville's Theorem is the fact that  R = 1 .

Now, so long as there is no coupling mechanism between the x plane and y plane of a magnetic-optical system (which is the case if the midplane symmetry prevails throughout the system) then, the phase space area in a given plane is also conserved. Consider a bundle of rays represented by the parallelogram, shown in Figure l5(a), representing the phase space distribution of the rays at some initial position. If we now look at the phase space distribution of the same bundle after it has drifted down stream, we observe the θmax  boundary and the x intercept x1 remain unchanged. In other words, the area of the parallelogram is the same, or "phase space area has been conserved."

For mathematical convenience, the parallelogram is rather difficult

to work with and, hence, a phase ellipse is usually used.

The phase ellipse transformation for a drift distance is illustrated

in Figure 16. Figure l6(a) corresponds to a beam which is at its minimum

width (a "waist") and Figure 16(b) shows the same beam after it has drifted

downstream from the waist position. The physical meaning of this is that

particles entering at 9 = O are parallel to the optic axis and, therefore,

cannot change their relative positions with respect to the optic axis; that

is, all particles on the x0 axis act in this manner. Those that enter

at a given angle continue at the same angle.

The phase ellipse transformation for a thin lens is illustrated in

Figure 17. In passing through a thin lens, θ changes and the x dimension

remains constant for a given trajectory.

Stated in other terms, xmax remains constant, as does θi  (the θ intercept). It is apparent, in the given example, that the spot size now becomes, or can become, smaller at the new image because θmax is larger.

This can be related to the physics of the system by saying that the x

magnification is less than unity. This fact is observed directly by comparison

of the x intercept of the ellipse before and after the lens action. It is

interesting to observe that a particle initially at c is transformed to

c' and that particles entering at x = 0 do not change their direction

(θi  is constant). If the particles are now allowed to drift, the ellipse rotates clockwise; when the ellipse if vertical, the spot size is at a

minimum, namely, xmax = x1 , as was illustrated in Figure 16.

Beam Transport Optics - Part II (K. L. Brown)

- Introduction

In Part I, the basic concepts of been transport optics were established.

Starting from the essentials of geometric optics, the methods of matrix

algebra were introduced with the example of calculating the principal planes

of a thick lens. The 3 x 3 matrix for the first- order bean transport

calculations were introduced to take into account the particle moments.

- First Order Transformation Matrix

Figure 1 shows a general region containing a magnetic field.

Figure 1: General Magnetic Field Configuration

The matrix presents a convenient way of writing the family of equations which describe the transformation from surface A to surface B. If x0, θ0  and δ represent the conditions of a ray entering the system at A, then the conditions of the ray at B are X1, θ1  and δ . Here x0 is the distance from the central ray to the ray C, θ0  is the angle between C and the parallel to the central ray and δ is the ratio Δp/p where Δp is the difference between the momentum of C and the momentum of the central ray.

The linear transformation equations are:

x1 = cxx0 + sxθ0 + dxδ

θ1 = cx’x0 + sx’θ0 + dx’δ 					(1)

δ = 0 + 0 + δ

Expressed as a matrix, Eq. (1) are:

x1θ1δ = cxsxdxc'xs'xd'x001 x0θ0δ  				(2)

The equation δ  = δ  expresses the fact that the magnetic field cannot

change the scalar momentum of the particle. The δ terms in the x and θ

equations express the momentum dispersion of the system.

If it happens that A is an object point and B is an image point of

the system, then xl is independent of θ0 , thus sX = O. In this case, cX

is given by cx = x1 /x0 = Mx = the magnification in x plane, (for δ  = 0). if θ0 = δ  = 0, then θ1 = c’x x0 = -x0/f or c’x = -1/f. It must always be true that the determinant of the matrix |R |, is unity. Thus for this

special case of sX = 0, it follows that s’x= l/Mx.

3. Beam Switchyard

As an example of a system which can be calculated with the matrix

method, we next consider the beam switchyard of the two-mile accelerator.

Figure 2 shows the three essential elements, two bending magnets and a

quadrupole lens. In common with many beam transport systems, this one is

designed to be achromatic. Mathematically, this means that the matrix

elements, dx and d’x should be zero, so that there is no x or θ dependence

on the momentum of the particles.

Figure 2: The Essentials of the Beam Switchyard

As a preliminary step, we will find the matrix expression for a bending

magnet when measured from the principal planes. The matrix for a bending

magnet, when measured from the ends of the poles, is given by:

Rbend =   cs1-c-scs001				(3)

(manca quadrettatura

where c = cos α  and s = sin α  and α  is the deflection angle of the central

ray. This expression has been normalized by setting the bending radius

equal to unity. To restore ordinary units it is only necessary to insert

the bending radius wherever a length is needed dimensionally. In this

case, the matrix then becomes

Rbend =   cρsρ(1-c)-s/ρcs001				(4)

(manca quadrettatura)

If the distance from the entrance plane to the first principal plane

is z1 and the distance from the second principal plane to the exit plane

is z2, we can find the values zl and z2 by solving the following matrix

equation:

1-z20010001 cs1-c-scs001 1-z10010001 10dx-1/f1d'x001			(5)

The matrix multiplication need only be done for the 2 X 2 matrices

as outlined. To illustrate matrix multiplication the indicated operations

will be given below in natural stages as follows:

1-z201 c-cz1+   s-s+sz1+   c 11-c-1/f1

(manca quadrettatura)

c+z2s-cz1+  s +sz1z2- cz2  -s+sz1+   c= 10-1/f1				(6)

(manca quadrettatura)

Note that these transformations do not change the focal length expression,

-1/f = -s. In order for two matrices to be equal, each individual element

must be equal to its counterpart in the other matrix. Thus we have

c + z2s = 1

c + z1s = 1					(7)

which when solved for z1 and z2 yeld

z2 = (1-c)/s and z1 = (1-c)/s

If we substitute the trigonometric equivalents, and apply standard identities,

we have z2 = z1 = tan (α/2) which can be seen from Figure 3 to indicate that

the two principal planes are coincident with the symmetry plane in the middle

of the magnet.

Figure 3: The Principal Planes of a Simple Bending Magnet are

Coincident with the Center Plane

The simplified matrix for a bending magnet measured to the principal planes is then:

Rbend =   100-s1s001                                (8)

(manca quadrettatura)

To calculate the transformation matrix for the entire Beam Switchyard

system as shown in Fig. 2, we write the matrices in opposite order from that

in which the beam passes through the elements. That this must be true can

be seen from the way in which one element alone is calculated by

x1θ1δ =R1 x0θ0δ                           (9)

Then for a second element we have

x2θ12δ = R2 x1θ1δ =R2R1x0θ0δ                (10)

and so forth.

If we allow the system to be symmetrical, i.e. s1 = s2 and L1 = L2; the complete series of matrices for Fig. 2 are

RBSY = 100-s1s001 1L0010001 100-1/f10001  1L0010001100-s1s001 		(11)

(manca quadrettatura)

We will show the step—by—step multiplication of the matrices to get

The dx and d’x terms.

RBSY = 1L0-s1-aLs001 100-1/f10001  1-aLLLs-s1s001

(manca quadrettatura)

RBSY = 1LfL2LfLa2LfLa-12s+1f1-La1-Lfs1-La2Lf-La2Lf001         (12)

(manca quadrettatura)

To obtain the required condition that dx = d’x = 0, we set

2-Lf = 0 or f = L2

Then

RBSY = -100-2L1-La-10001				(13)

(manca quadrettatura)

Thus the quadrupole acts as a lens to refocus rays from the center of

the first bending magnet to the center of the last one. For the serious

student, it is a worthwhile exercise to do the BSY problem without the

simplification which resulted from introducing the principal planes.

4. TRANSPORT (pag.150)

As an aid to solving beam transport problems, a computer program

TRANSPORT has been developed at SLAC which takes the greatest amount of

labor out of this work. The program operates in about the way as the BSY example above was calculated, but with some important exceptions. Most

importantly:

1. TRANSPORT has the ability to find the best first-order solution

given a certain set of constraints;

2. TRANSPORT also calculates the transformation of a whole family

of rays as found in a beam by means of the concept of "phase space" which

was introduced in Part I;

3. TRANSPORT can, as an option, calculate the second order effects

on the beam. By second order is meant, for example, terms which depend net

linearly on the displacement xo , but on x02 or x0 θ0 , etc.

To aid in the discussion of TRANSPORT and of the second order terms

we now introduce an abbreviated notation. By writing out the complete equations

for x and y , to second order, we will adequately have displayed the new

notation.

X1 =	(x|x0)x0 + (x |θ0)θ0 +(x|δ )δ

+(x|x02) x02 + (x |x0 θ0)x0 θ0  +(x|x0 δ)x0 δ

+(x|θ02) θ02 + (x |θ0 δ)θ0 δ  +(x|δ2)δ2

+(x|y02) y02 + (x |y0 ϕ0)y0 ϕ0  +(x|ϕ02) ϕ02					(14)

Y1 = 	(y|y0) + (y |ϕ0 )ϕ0

+(y|x0y0) x0y0 + (y|x0 ϕ0 ) x0 ϕ0 + (y|θ0y0) θ0y0 			(15)

+(y|θ0 ϕ0)θ0 ϕ0 + (y|y0 δ)y0 δ + (y|δϕ0) δϕ0

The absence of certain terms which might otherwise be expected in

Eqs. (14) and (15) is due to the fact that horizontal mid-plane symmetry

has been assumed in the derivation. That is, the field on the horizontal

mid-plane is normal to the plane. Thus there can be no (y|x) or (y|θ) term. Similarly, there can only be even powers of y and ϕ , such as and in the (x|y02) and (x|y0ϕ0) in the x equation. Also, note that there is no (y|δ) or (y|δ2) term if there is mid-plane symmetry.

TRANSPORT uses a numerical notation to signify the six basic coordinates:

x  θ  y  ϕ   ℓ   δ

1  2  3  4   5   6							(16)

The ℓ term has not been introduced here before. Its significance is the

preservation of the bunch length of a beam such as the SLAC electron beam.

The first order output from TRANSPORT is a 6 x 6 matrix printout of the

R matrix where the labels are implied by row and column position of the elements.

For example the element appearing at the intersection of row 3 and column 4

Is the coefficient (y|ϕ0) etc.

|  | x0 | θ0 | y0 | ϕ0 | ℓ0 | δ0 |
| --- | --- | --- | --- | --- | --- | --- |
| x | - | - | - | - | - | - |
| θ | - | - | - | - | - | - |
| y | - | - | - | - | - | - |
| ϕ | - | - | - | - | - | - |
| ℓ | - | - |  | - | - | - |
| δ | - | - | - | - | - | - |

The second order terms are labeled by the convention indicated in Eqs. (14), (15) and (16). For example, becomes (x|x02) and becomes 1 11 and (x|θ0δ) becomes 1 26.

5. Second Order Matrix

Normally the matrix method is expected only to apply to the solution

of linear, i.e. first order, equations . However, the method has been extended to include second order terms as discussed in Part I.

For a more extensive discussion of the second—order matrix

formalism, the reader is referred to SLAC report number 75 by K.L. Brown.

FIRST -ORDER R MATRIX FORMALISM FOR TRANSPORT

Section II

Beam transport optics may be reduced to a process of matrix multiplication ( 1,2) . To first-order, this is represented by the matrix equation (using the

notation of SLAc-75) .

x1(t) = j=16Rij xj (0)					(1)

where

x1 = x  x2 = θ  x3 = y  x4 = ϕ  x5 = ℓ   and x6 = δ

The determinant |R| = 1. This is a direct consequence of the basic equation of

motion for a charged particle in a static magnetic field and is a manifestation of

Liouville's theorem of conservation of phase space volume. (See SLAC—75, page 41

for a proof that |R| = 1.)

For static magnetic systems possessing midplane symmetry, the six simultaneous linear equations represented by Eq. (1) may be expanded in matrix form as follows:

xtθtytΦtltδ(t)  = R11R12000R16R21R22000R2600R33R340000R43R4400R51R52001R56000001 x0θ0y0Φ0l0δ0

(2)

(mancano righe verticali parte centrale)

where the transformation is from an initial position τ = 0 to a final position

τ = t measured along the assumed central reference trajectory.

Thus at any specified position in a system, an arbitrary charged particle

is represented by a vector (single column matrix), X, whose components are the

positions, angles, and momentum of the particle with respect to a specified reference trajectory.

i.e. x= xθyϕlδ

where:

x = 	the radial displacement of the arbitrary ray with respect to the

assumed central trajectory.

θ = 	the angle this ray makes in the radial plane with respect to the

assumed central trajectory.

y = 	the transverse displacement of the ray with respect to the assumed

central trajectory.

ϕ = 	the transverse angle of the ray with respect to the assumed central

trajectory.

ℓ  = 	the path length difference between the arbitrary ray and the central

trajectory.

δ = 	ΔP/P is the fractional momentum deviation of the ray from the assumed

central trajectory

The magnetic lens is represented by the square matrix, R, which describes

the action of the magnet on the particle coordinates. Thus the passage of a charged particle through the system may be represented by the matrix equation:

X(1) = R X (0)				(3)

where X(O) is the initial coordinate vector and X(l) is the final coordinate vector of the particle under consideration; R is the transformation matrix for all such particles traversing the system (one particle differing from another only by its initial coordinate vector X(O)) .

The traversing of several magnets and interspersing drift spaces is described by the same basic equation but with R now being the product matrix R = R(n) . . . R(3)R(2)R(l) of the individual matrices of the system elements. TRANSPORT calculates and tabulates the product matrix R representing the system.

The zero elements R13 = R14 = R23 = R24 = R31 = R32 = R41 = R42 = R36 = R46 = 0

in the R matrix are a direct consequence of midplane symmetry. If midplane symmetry is destroyed, these elements will in general become non-zero. The zero elements in column five occur because the variables x, θ, y, ϕ, and δ are independent of the path length difference ℓ. The zero's in row six result from the fact that we have restricted the problem to static magnetic fields, i.e., the scalar momentum is a constant of the motion.

In SLAC report 75 (Ref. 1), a physical significance has been attached to

the non-zero matrix elements in the first four rows in terms of their identification with characteristic first-order trajectories. We include figures showing these characteristic functions as a convenient reference.

We now wish to relate the elements appearing in column six and those in

row five in terms of simple integrals of the characteristic first-order matrix elements cx(t) = R11 and sx(t) = R12. In order to do this, we make use of the Green's function integral, Eq. (43), Section II of SLAC-75, and of the expression for the differential path length in curvilinear coordinates

dT = [(dx)2 + (dy)2 + (1+hx)2 (dt)2 ]½			(4)

used in the derivation of the equation of motion.

First-Order Dispersion (pag.163)

The spatial dispersion dx(t) = R16 of a system at position t is derived

using the Green's function integral, and the driving term f(τ) = h (τ) = 1ρ0(t) for

the dispersion (see Table I of SLAC-75). The result is

dx(t) = R16 = sx(t) 0tcx(τ) h (τ)dτ – cx(t) 0tsx (τ)h(τ)dτ 	 	(5)

where τ  is the variable of integration. Note that h(τ)dτ = dα  is the differential angle of bend of the central trajectory at any point in the system. Thus first-order dispersion is generated only in regions where the central trajectory is deflected (i.e., in dipole elements.) The angular dispersion is obtained by direct differentiation of dx(t) with respect to t, the curvilinear distance along the central trajectory.

dx'(t) = R26 = s'x(t) 0tcx(τ) h (τ)dτ – c’x(t) 0tsx (τ)h(τ)dτ 	 (6)

where

c’x(t) = R21 = s’x(t) =R22

First-Order Path Length

The first-order path length difference is obtained by expanding and integrating Eq. (4) and retaining only the first-order term, i.e.,

ℓ – ℓ0 = (T-t) = 0txτhτdτ+ higher order terms				(6)

from which

ℓ = x0 0tcxτh τ+ θ0 0tsx (τ) h(τ) dτ +ℓ0 +δ  0tdx(τ) h(τ)dτ

= R51 x0 + R52θ0 + ℓ0 + R56 δ 							(7)

Inspection of Eqs. (5), (6), and (7) yields the following useful theorems:

Achromaticity: A system is defined as being achromatic if R16 = R26= 0, i.e., if dx(t) = d’x(t) = 0. Therefore it follows from (5) and (6) that the

necessary and sufficient conditions for achromaticity are that

0tsx(τ) h(τ)dτ  = 0tcx(τ) h(τ)dτ = 0						(8)

By comparing Eq. (7) with Eq. (8), we note that if a system is achromatic, all

particles of the same momentum will have equal (first-order) path lengths through

the system.

Isochronicity: It is somewhat unfortunate that this word has been used

in the literature to mean equal path lengths since equal path lengths only imply

equal transit times for highly relativistic particles. Nevertheless, from Eq. 7,

the necessary and sufficient conditions that the first-order path length of all

particles (in dependent of their initial momenta) will be the same through a system are R51 = R52 = R56 = 0, i,e., if

0tcx(τ) h(τ)dτ  = 0tsx(τ) h(τ)dτ = 0tdx(τ) h(τ)dτ = 0			(9)

First -Order Imaging (pag.164)

First-order point-to-point imaging in the x plane occurs when x(t) is

independent of the initial angle θ0 . This can only be so when

sx(t) = R12 =0.					(10)

Similarly first-order point-to-point imaging occurs in the y plane when

sy(t) = R34 =0.					(11)

First-order parallel-to-point imaging occurs in the x(t) plane when is independent

of the initial particle position xo. This will occur only if

cx(t) = R11 =0.					(12)

and correspondingly in the y plane, parallel-to-pointimaging occurs when

cy(t) = R33 =0.					(13)

A parallel ray entering a system exits parallel to the central trajectory if

c’x(t) = R21 =0.					(14)

in the x plane; and if

c’y(t) = R43 =0.					(15)

in the y plane.

Point-to—parallel imaging occurs in the x plane if

s’x(t) = R22 =0.					(16)

and in the y plane if

s’y(t) = R44 =0.					(17)

Focal Lengths

A simple ray diagram of a "thick" lens demonstrates that R21 and R43 have

the following physical interpretations

c’x(t) = R21 = -1fx  and c’y(t) = R43 = -1fy				(18)

where fx and fy are the system focal lengths in the x and y planes respectively.

Zero Dispersion

For point-to-point imaging, using Eq’s.(5) and (10), the necessary and

sufficient condition for zero dispersion at an image is

dx(t) = R16 = 0tsx(τ) h(τ)dτ = 0			(19)

For parallel-to-point imaging, (i.e., cx(t)= O),the condition for zero dispersion

at the image is

dx(t) = R16 = 0tcx(τ) h(τ)dτ = 0			(20)

Magnification

For monoenergetic point-to-point imaging in the x-plane, the magnification

is given by

Mx = x(t)x0 = R11 = cx(t)

And the y plane by

My = R33 = cy(t)						(21)

where a negative number means an inverted image.

First-Order Momentum Resolution

For point-to-point imaging the first-order momentum resolving power Rl

(not to be confused with the matrix R) is the ratio of the momentum dispersion to

the total image size. Thus if 2xO is the total source size then

R1 = PΔP = R162x0R11 = dx(t)2x0cx  (t)

For point-to—point. imaging sx(t) = 0. Using Eq. (5), the dispersion at an image

is

dx(t)= -cx(t)  0tsx(τ) h(τ)dτ  			(22)

from which the first-order momentum resolving power Rl becomes

2x0R1 = dx(t)cx(t) = 0tsxτh τdτ = R52		(23)

Equation (23) for the first-order resolving power of a system may be

expressed in a number of useful forms. If we consider a ray (particle) originating

at the source with X0 = O and δ = ΔPP = O and lying in the midplane. (i.e., a mono-

energetic point source), the first-order equation representing the midplane displacement, x of this trajectory is

x(t) = sx(t) θ0 						(24)

We may then rewrite Equation (23) as follows:

2x0R1 = 0tsx(τ) h(τ)dτ = 1θ00tx(τ) h(τ)dτ = (l-l0)θ0= R52		(25)

or we may. also write it in the form

R1 =12x0θ0 0tBxtdτBρ0 = 12x0θ01Bρ00tBdA					(26)

where BdA  is the magnetic flux inclosed between the central trajectory and the

ray represented by Eq. (24), and Bρ  is the magnetic rigidity (momentum) of the

central trajectory. Please note, however, that if the ray crosses the central

trajectory or the sign of B changes, this changes the sign of the integration.

Some important observations may be made from Eq's. (25) and (26).

l) Resolving particles of different momentum requires that a path

length difference must exist between the central trajectory and the trajectory

defined by Eq. (22). The greater the path length difference, the greater the

resolving power.

2) From Eq. (24), we may define resolving power as the magnetic flux

inclosed per unit phase space area per unit momentum (Bρ) of the central ray.

First-Order σ  Matrix (Phase Ellipse) Formalism for TRANSPORT

In accelerators and beam transport systems, the behavior of an individual

particle is often of less concern than is the behavior of a bundle of particles

(the BEAM) of which an individual particle is a member. An extension of the matrix

algebra of Eq. (3) provides a convenient means for defining and manipulating this

BEAM. TRANSPORT assumes that the bundle of rays constituting a BEAM may correctly

be represented in coordinate phase-space by an ellipsoid whose coordinates are the

position, angle, and momentum coordinates of the arbitrary rays in the beam about

an assumed central trajectory. TRANSPORT is a matrix calculation that truncates

the problem to either first- or second-order in a Taylors expansion about the

central trajectory. Particles in a BEAM are assumed to lie within the boundaries

of the ellipsoid with each point within the ellipsoid representing a possible ray.

The sum total of all phase points, the phase space volume, is commonly referred

to as the "phase space" occupied by the BEAM. The validity and interpretation of

this phase ellipse formalism must be ascertained for each system being designed.

However, in general, for charged particle beams in, or emanating, from accelerators, the first-order phase ellipse formalism of TRANSPORT is a reasonable representation of physical reality; but for other applications, such as charged particle spectrometers, caution is in order in its use and interpretation.

The equation of an n-dimensional ellipsoid may be written in matrix

form as follows:

X(0)T σ (0)-1 X(0) = 1					(27)

Where X(0)T is the transpose of the coordinate vector X(O), and σ(0) is a real,

positive definite, symmetric matrix.

The volume of the n-dimensional ellipsoid defined by sigma is

πn 2Γ\n2+1 (det σ)1/2 , the area of the projection in one plane is A = π(det σ)1/2.

This is the "phase space" occupied by the beam.

As a particle passes through a system of magnets, it undergoes the matrix

transformation of Eq. (3). Combining this transformation with the equation of the

initial ellipsoid, and using the identity RR-1 = I (the unity matrix), it follows

that:

X(0)T(RTRT-1) σ (0)-1 (R-1R) X(0) =1

from which:

(RX(0))T (Rσ (0)RT)-1 (RX(0)) =1				(28)

The equation of the ellipsoid representing the "BEAM" at the end of

the system is thus:

X(l)T σ (1)-1 X(1) - 1 					(29)

where the equation for the sigma matrix at the end may be related to that at the

beginning by:

σ (1) = Rσ (0)RT 						(30)

In addition to calculating the product matrix R, TRANSPORT also computes

the signs "BEAM" matrix at the end of each physical element via Eq. (30).

All of the important physical parameters of the BEAM ellipsoid may be

expressed as functions of the matrix elements of the sigma matrix at the location

in question. In particular the square roots of the diagonal elements  σii'sare

the projection of the ellipse upon the coordinate axes and thus represent the

maximum extent of the BEAM in the various coordinate directions. The correlation

between components (the orientation of the ellipse) is determined by the off-diagonal terms (the σij 's). An illustration of this is given below for a 2-dimensional ellipse.

Description of the Sigma BEAM Matrix

Consider a 2-dimensional (x,θ) plane projection of the general 6~dimensional ellipsoid . Let

σ σ11σ21σ21σ22

be a real, positive definite, symmetric matrix; the inverse of which is

σ-1 = 1ϵ2 σ22-σ21-σ21σ11

where ϵ2 is the determinant of σ.

The 2-dimensional coordinate vector (column matrix) and its transpose

are:

X = xθ and XT = (x θ)

The expansion of the matrix equation XT σ-1 X = l is the equation of the ellipse

σ22 x2 – 2σ21 xθ  + σll θ2 = ϵ2 = det σ 		 (31)

The (x,θ) plane BEAM ellipse represented by Eq. (31) is shown in the following

figure along with the physical meaning of the sigma matrix elements.

The area of the ellipse is given by:

A = π (det σ)1/2 = πxmax θint = πxint θmax 		(32)

A Two Dimensional BEAM Phase Ellipse

The correlation between x and θ  (the orientation of the ellipse) depends

upon the off-diagonal term σ21. This correlation is defined as

r21 = r12 = σ21σ11σ22

So defined r always falls in the range

-1≤ r ≤ 1

The correlation, r, measures the tilt of the ellipse and the intersection of the

ellipse with the coordinate axes.

Since the det R = l for all static magnetic beam transport elements, it

follows that the determinant of σ(1) and σ(0) are identical under the transforma-

tion of Eq. (30) . Hence the "phase space" area is an invariant under the transformation of Eq. (30) . This is a statement of Liouville's Theorem for the magnetostatic fields employed and results from the fact that the det R = l.

It is perhaps worthwhile noting that this 2-dimensional representation

of the BEAM matrix has a one to one correspondence with the Courant-Snyder treatment of the theory of the Alternating Gradient Synchrotron* as follows:

σ11σ21σ21σ22 = ϵ β-α-αγ					(33)

----------------------------------------------------------------------------

* E. D. Courant and H. S. Snyder, "Theory of the Alternating Gradient

Synchrotron", Annals of Physics 3, pp 1-118 (1958).

----------------------------------------------------------------------------

The Phase Ellipse Beam Matrix used by TRANSPORT

For static magnetic systems possessing midplane symmetry, the (x,θ)

plane and (y,ϕ) plane trajectories are decoupled in first-order, i.e. there is

no mixing of phase space between the two planes. However for mathematical simplicity and to allow for the possibility of more general systems, the sigma BEAM matrix used in a TRANSPORT calculation has the following general 6-dimensional construction.

|  | x | θ | y | ϕ | ℓ | δ |
| --- | --- | --- | --- | --- | --- | --- |
| x | σ(11) |  |  |  |  |  |
| θ | σ(21) | σ(22) |  |  |  |  |
| y | σ(31) | σ(32) | σ(33) |  |  |  |
| ϕ | σ(41) | σ(42) | σ(43) | σ(44) |  |  |
| ℓ | σ(51) | σ(52) | σ(53) | σ(54) | σ(55) |  |
| δ | σ(61) | σ(62) | σ(63) | σ(64) | σ(65) | σ(66) |

The matrix is symmetric so that only a triangle of elements is needed.

In the printed output this matrix has a somewhat different format for

ease of interpretation:

|  |  |  | x | θ | y | ϕ | ℓ |
| --- | --- | --- | --- | --- | --- | --- | --- |
| x | σ(11) | CM |  |  |  |  |  |
| θ | σ(22) | MR | r(21) |  |  |  |  |
| y | σ(33) | CM | r(31) | r(32) |  |  |  |
| ϕ | σ(44) | MR | r(41) | r(42) | r(43) |  |  |
| ℓ | σ(55) | CM | r(51) | r(52) | r(53) | r(54) |  |
| δ | σ(66) | PC | r(61) | r(62) | r(63) | r(64) | r(65) |

Where: 			r(ij) = σ(ij)ijσij σ(ij)1/2				(34)

As a result of the fact that the matrix is position definite, the r(ij)’s

satisfy the relation

|r(ij)|≤ 1						(35)

The physical meaning of the σ(ij)'s is as follows:

σ(11) = xmax = The maximum (half)-width of the beam envelope in the

x plane at the point of the printout.

σ(22) = θmax = The maximum (half)-angular divergence of the beam envelope

in the x plane.

σ(33) = ymax = The maximum (half)-height. of the beam envelope.

σ(44) = ϕmax = The maximum (half)-angular divergence of the beam envelope

in the y plane.

σ(55) = ℓmax = l/2 the longitudinal extent of the bunch of particles .

σ(66) = δ  = The half-width 1/2 (ΔP/P) of the momentum interval being

transmitted by the system.

The units appearing next; to the σ(11)'s in the TRANSPORT printout sheet

are the units chosen for coordinates x, θ, y, ϕ, ℓ and δ = ΔP/P respectively.

To the immediate left of the listing of the beam envelope size in a

TRANSPORT printout, there appears a column of numbers whose values will normally

be zero. These numbers are the coordinates of the centroid of the beam phase ellipse (with respect to the initially assumed central trajectory of the system). They may become non-zero under one of three circumstances:

1) When the misalignment (Type Code 8.) is used.

2) When a Beam Centroid shift (Type Code 7.) is used.

or 	3) When a 2nd—order calculation (Type Code 17.) is used.

Physical Interpretation of Various Projections of the 2-dimensional BEAM Ellipse

Consider again Eq. (30) σ(1) = R σ(0) RT and expand it in it's most

general form for the 2-dimensional (x, θ) plane case.

σ(1) = R11R12R21R22 σ11(0)σ21(0)σ21(0)σ22(0) R11R21R12R22 σ11(1)σ21(1)σ21(1)σ22(1)

the result is:

σ(1) = R112σ110+2R11  R12σ21 0+R122σ22(0)R11R21σ110+(R11R22R12R21)σ210+R12R22σ22(0)R212σ110+2R21R22σ210+R222σ22(0)

In the special case when the initial ellipse is erect i.e., σ2l(O)=O, σ(1) reduces to:R112σ110+R122σ22(0)R11R21σ110+R12R22σ22(0)R212σ110+R222σ22(0)

Similar results are, of course, obtained for the (y,ϕ) plane.

If an arbitrary beam transport system is reduced to the most elementary

first-order form of representing it as an initial drift distance, followed by a

lens action between two principal planes, and a final drift distance; then we observe that for the 2-dimensional representation there are only two basic phase

ellipse transformations of interest.

(1) An arbitrary DRIFT distance and

(2) A LENS action

Each of these elementary cases are illustrated on Fig.8 for both a

parallelogram as well as ellipse phase space transformations. Note that a DRIFT

followed by a LENS action is not necessarily equal to a LENS action followed by

a DRIFT; i.e., the matrices do not necessarily commute.

The phase ellipse transformations for a DRIFT and for a LENS action

(between principal planes) as shown in Fig. (8 ) may be readily calculated using

the results of Eq. (37).

The 2-dimensional R matrix representing a drift of distance L is:

R(Drift) = 1L01					(38)

Substituting into Eq. (37) we find

σ(1) = σ110+L2σ22(0)Lσ22(0)Lσ22(0)σ22(0)  = σ111σ21(1)σ21(1)σ22(1)				(39)

Attaching the physical meaning to the matrix elements yields the following interpretations:		 σ11(1) = σ11(0) +L2 σ22(0)

or

(x12)max = (x02)max + L2(θ012)max				(40)

similary 			σ22(1) = σ22(0)

or				(θ12)max = (θ02)max 					(41)

Note that this transformation assumes that the initial phase ellipse is erect,

i.e., σ21(0) = 0.

The 2-dimensional R matrix for a lens actions (between principal planes)

is

R(Lens) = 10-1f1					(42)

Substitution into Eq. (37) yields

σ(2) =  σ110-σ11(9)fσ11(1)fσ110f2+ σ22(0) =  σ112σ21(2)σ21(2)σ22(2)				(43)

Again attaching physical meaning to the matrix elements we have:

σ11(2) = σ11(0)

or					(x22)max = (x02)max				(44)

and					σ22(2)= σ11(0)f2 + σ22(0)

or					(θ22)max = 1f2 (x02)max + (θ02)max		(45)

Note the change in sign of the σ21 elements for the Drift and the Lens actions

indicating the different sense of orientation of the resulting ellipses as

illustrated in Fig. 8.

The Upright Ellipse:

A case of particular interest in any 2-dimensional phase ellipse projection

(e.g., the (x,θ) plane or the (y, ϕ) plane is when the off-diagonal correlation

matrix elements are equal to zero; i.e., an erect ellipse. In a field-free region

this corresponds to a so-called "waist" in the BEAM as illustrated in Fig. 9.

It is important to understand correctly the meaning of a waist: for an

existing beam, it is the location of the minimum beam size in a given region of

the system. Although the waist is the minimum beam size in any given beam, the

minimum beam size achievable at a fixed target position by varying the focal

strength of the preceding lens system is not the same as the above defined wait.

See Fig. 10. In a field-free region, the minimum beam spot size achievable at a

fixed target position will occur when the preceding lens system is adjusted such

that a waist precedes the target position. Only in the limit of zero phase space

area do these quantities occur at the same location. A useful criterion that

determines the physical proximity of these quantities is the following: if the

system has been adjusted for the smallest. spot size at a fixed position and if the size of the beam at the principal planes of the optical system is large compared to its size at the waist, or at the minimum spot size, then the location of these quantities, the waist and the minimum, will closely coincide; if, on the other hand, the size of the beam does not change substantially throughout the system, then the locations of a waist and the minimum beam size may (and usually will) differ substantially. The numerical proximity of these two quantities will be discussed in greater detail later in the report.

In a field free region (i.e., a Drift), the distance to a waist from any

location may be readily calculated if the σ matrix at the location is known. Using

Eq. (36) and the R matrix for a Drift (Eq. 38) we have for the (x,θ) plane:

σ21(1) = σ21(0) + L σ22(0) = 0 (specifying that σ(1) shall be at a waist)

or the distance to the waist is:

L = -σ21(0)σ22(0) = -r21 σ11σ22 					(46)

Similarly for the (y, ϕ) plane the distance to a waist is:

L = -σ43(0)σ44(0) = -r43 σ33σ44 					(47)

Relationship between a Waist and a Parallel-to-Point Image

A parallel-to—point image in the (x,θ) plane occurs when Rll = O. The

R matrix corresponding to this is

R =0R12R21R22 = 0sxc'xs'x = 0fx-1fxs'x   				(48)

(manca suddivisione centrale nelle 3 matrici)

Since = |R|= 1, R12R2l for this situation.

If we assume an erect ellipse σ(0) as the beginning of the system, the

final beam matrix σ(1) is given by substitution of Eq. (48) into Eq. (37) as

follows:

σ(1) = R122σ22(0)R12R22σ22(0)R12R22σ22(0)    R212σ110+R222σ22(0)				(49)

(manca suddivisione centrale

for parallel-to-point imaging.

Several conclusions may be extracted from this result: the first observation is that a waist and a parallel-to-point image will coincide if R11=R22=0.  This is equivalent to requiring that the object and image distances (measured to the principal planes) are both equal to the focal length f of the system.

The distance to a waist in this example is:

L = -σ21(1)σ22(1) = R12R22σ22(0)R212σ110+R222σ22(0) = - sxs'xσ22(0)c'x2σ110+s'x2σ220 				(50)

If sxs'x=  R12R22 = 0, a waist and a parallel-to-point image coincide.

If sxs'x=  R12R22 > 0, the waist precedes the image; and if If sxs'x=  R12R22 < 0, the waist follows the image; unless σ22(0) = 0 (zero phase space area) in which

case a waist. and an image always coincide.

The size of the beam at the image is:

x12max = σ11(1) = R122 σ22 (0)= f2 (θ02)max 					(51)

independent of the source size x0 and of the object distance.

The size of the beam at the waist is:

(size at waits)2 = |σ0|σ22(1) = σ110σ220R212σ110+ R222σ220						(52)

If R22 = s’X = O, the two sizes are equal as expected, otherwise the

size at the waist is always smaller.

Relationship between a Waist and a Point-to-Point Image

A point—to—point first-order image in the (x,θ) plane occurs when

R12 = sx = O. The R matrix representing this case is:

R = =R110R21R22 = cx0c'xs'x = M0-1fx1M   			(53)

(manca suddivisione centrale nelle 3 matrici)

where |R| = 1 = R1lR22, and M is the magnification.

If we again assume an erect ellipse σ(0) as the beginning, the final

beam matrix σ(1) is given by Eq. (37) as:

σ(1) = R112σ11(0)R11R21σ11(0)R11R21σ11(0)    R212σ110+R222σ22(0)				(54)

(manca suddivisione centrale)

for point-to-point imaging.

Our first observation is that except for a zero source size, an image and a waist will coincide only if R12 = R21 = O Clearly this is not possible

with a single lens; at least two lenses are needed. Such an optical situation is

as follows :

The distance to a waist is

L = -σ21(1)σ22(1) = - R11R21σ11(0)R212σ110+R222σ22(0)				(55)

So if R11R21 = cxc’x = 0, a waist and a point-to-point image coincide.

If cxc’x > 0, the waist precedes the image and if cxc’x < 0, the waist follows the image.

The size of the bean at the image

x12max = σ11(1) = R112σ11 (0)= (Mx0)max 					(56)

and the size of the bean at the waist is:

(size at waits)2 = |σ0|σ22(1) = σ110σ220R212σ110+ R222σ220					(57)

Thus if R21 = 0, the two sizes are equal since |R|= R11R22 =1.

Otherwise the size at the waist is smaller than the image size

Relationship between a Waist and the Smallest Spot Size Achievable at a Fixed Target. Position

Consider the following general situation:

Assume that the size of the beam σ11lens at the principal planes of the lens

system being adjusted is held constant (i.e., that no other preceding optical

elements of the system are being varied); and that the remainder of the system

may be represented by a general matrix R which is also held constant. The focal

length f is then varied until a minimum spot size σ11minis achieved at the

target location. The sigma beam matrix at the target position then has the

following unique form independent of the orientation of the initial beam ellipse

at the lens .

thus

σ11(min) =R122|σ|σ11(lens)

or

xmin = R12|σ|1/2x(lens)				(59)

If the position of the waist and the minimum bean size both fall within the the same field-free region, then the distance to the waist from the target is:

Z= -σ21  (min)σ22(min)   = - R12R22R222+ σ11(lens)σ11(min)= -R12R22σ11(waist)σ11(lens)		(60)

So if sxs’x = R12R22 = 0, the waist and the minimum spot size  coincide.

If R12R22 > 0, the waist precedes the target; and if R12R22 < 0, the waist occurs after the target position.

If the waist and the target positions fall within the same field-free

region, the following simple relationship exists between the beam size at the

lens σ11lens,  at the waist σ11waist,  and at the target.

1σ11  (waist)=1σ11(min)+R222σ11(lens) 				(61)

If now the lens system is readjusted to form a waist at the target position as

shown by the dotted lines in Fig. 10, the relative size of this waist and the

minimum spot size achieved by the previous lens setting is:

σ11(minat target)σ11(waist at target)=1-R222σ11(waist at target)σ11(lens)			(62)

Again we observe that the two quantities approach each other if the size of the

beam at the lens is large compared to the beam size at the target.

There are several cases of special interest that may be derived from

the above equations:

1) If R22 = O at the target position, then a minimum spot size at the

target is also a waist. This corresponds to point-to—parallel imaging from the

principal planes of the variable lens system to the target position. Beyond the

last lens in the field-free region preceding the target, R12 = a constant if

R22 = 0; thus we conclude from Eq. (59) that in this field-free region, the

minimum spot; size achievable at a target is a waist and is independent of the

target position. Such a system is a "Zoom" lens.

2) If there are no lenses beyond the variable lens system, i.e., R

is an entirely field—free region (a drift), then R is of the form:

R=1L01=R11R12R21R22

In this situation R12R22 = L is a positive quantity, consequently the waist always

precedes a minimum spot size at a target. A case of particular interest is when

the minimum spot size achievable is equal to the initial beam size at the lens. It

then follows from Eq. (60) that Z. -L/2, i.e., a waist occurs midway between the

lens and the target. From Eq. (61), the ratio of the size of the beam at the lens

and at the waist is:

x(lens)x(waist)=σ11(lens)σ11waist= 2  			(63)

Combining this result with Eq.(59),

L =R12= xlensx(min)σ12=(lens)xwaistθ(waist)

or

L =  2 x(waist)θ(waist)					(64)

where L is the longest distance a beam can drift without exceeding its initial

size at the lens.

Imaging from an Erect Ellipse to an Erect Ellipse

The general sigma matrix for imaging from an erect ellipse to an erect

ellipse may be derived by inspection from Eq. (36) by setting σ21(1) = σ2l(O) = 0.

The result is:

σ(1) = R112σ 110+ R122σ220                                            00                                                  R112σ 110+ R122σ220 = σ111            00              σ22(1)   			(65)

For symmetric magnetic systems Rll R22. Using this property and the fact that

R= 1, it follows that Rl2R2l = R112- 1.  So for symmetric magnetic systems

Eq- (65) reduces to:

σ(1) =        -R12σR211/2                      00                                    -R12σR211/2  = σ111            00              σ22(1)   				(66)

The above equations may be used to calculate the optimum design parameters for

periodic beam transport systems .

Example No. 1:

Consider a unit-cell of a periodic focusing array consisting of focusing

elements only as indicated below.

The R matrix for the unit-cell, i.e., from the principal planes of the first lens

to the principal planes of the second lens, is

R = R11                     2l-1f+l2f2                 R11  				(67)

If now we require that the beam envelope possess symmetry coincident with the lens

symmetry i.e., that erect ellipses occur at me principal planes of each lens and

a waist midway in between, and furthermore that the beam size at the second lens

be kept; to a minimum and equal to the beam size at the first lens: then substituting Eq.(67) into Eq. (66) and setting σ11(1) to be a minimum yields:

σ11(1) = σ11(0)

f = ℓ = 12σ11(0)σ22(0)= σ11(w)σ22(w)  					(68)

where σ11(0) and σ22(0) are measured at the principal planes of the first lens.

and finally

x(max)x(min)= σ11(0)σ11(w) = 2						(69)

Note that the ratio of the maximum to the minimum beam size (Eq. 69) is independent of the phase space area and of the lens spacing.

Example No. 2:

If the unit cell is a FODO array as follows:

The R matrix for the unit cell (from the principal planes of the first lens to the

principal planes of the third lens) is:

R= R11                                 l2+lf- l4f22-lf                 R11				(70)

If we now impose the symmetry requirements that erect ellipses occur at

the principal planes of each lens; and that the beam size σ11(1)  at lens 3 be

a minimum and equal to the beam size σ11(0)  " at lens 1, then it follows that:

σ11(1)  = σ11(0)

lf=41+ 5≅ 1.236							(71)

σ11(0)σ11(w)=-3+ 51- 5 >≅4.23

or

x(max)x(min)= σ11(0)σ11(w)≅2.055  			(72)

and finaly

ℓ ≅0.3003 σ11(0)σ22(0)						(73)

where σ11(0) and σ22(0) are measured at the principal planes of the first

lens in the FODO array.

For a FODO quadrupole array where the field strength is held constant

for all elements rather than the focal lengths, the results are somewhat

different than those above. This case may be readily calculated via TRANSPORT

using the above results as initial guesses in the calculation.

Relationship between a First-Order Point-to—Point Image and the Minimum Spot Size Achievable at a Fixed Target Position

This problem is not as easy to explore as were the preceding ones because

the question arises "the first-order image of what?” If, however, we restrict

the discussion to a thick or thin lens system that does not have intermediate

images between the source and the image under consideration, then the following

comments are applicable.

The ratio of the minimum beam size to the size of a first—order image

at a fixed target position may be calculated using Eq.'s (56) and (59). From

Eq.(59) we have:

σ11min= L2σσ11(lens)

and from Eq. (56) the size of a first-order image at hue position is:

σ11image = Mσ11(obiect) = Lpσ11(obiect)

where M is the magnification of the first-order image, p is the object distance

measured to the principal planes, and L is the distance to the target measured from the principal planes.

The ratio of sizes is

σ11(min)σ11(lst order image) = p2σσ11objectσ11(lens)  			(74)

Using Eq.(36), we may write

σ11lens=σ11object+ 2p σ21object+p2σ22 (object) 			(75)

and since

σ=σ11object σ22object- σ222(object)

it follows that the first-order image will coincide with the smallest spot size only if the orientation of the initial beam ellipse at the object is such that

p σ21(object) = - σ11 (object)				(76)

or if σll(object) = O 1.e., for a point source.

For an erect ellipse at the source and the lens adjusted to provide a

minimum spot size at the target, it can be shown that the first-order image will

always follow the target position (the minimum spot size) by a distance

Z=L M σ11(object)σ11(lens) = L Mx02x2(lens)			(77)

where L is the distance to the target position from the principal planes of the lens system, x0 is the source size, and M is the magnification of the first—order image.

Again we observe that the ratio of the beam size at the source and the beam size at the "lens" is the criterion determining the proximity of these two quantities.

Orientation of the Major Axes of a Phase Space Ellipse

The matrix equation for a coordinate rotation as shown in Fig. 11 is

x1θ1= cosα-sinαsinαcosα x0θ0				(78)

or

X1 = M X0

The equation of an ellipse in either set of coordinates is

XT σ-1 X = 1 where X = xθ; XT = (x θ)

and the transformation from σ(0) to σ(1) is

σ(1) = M σ(0) MT

provided M= 1, which it does.

If we assume a general ellipse for σ(0) and an "erect" ellipse for σ(1),

i.e.,

σ(0) = σ1110   σ21(0) σ210   σ22(0) and σ(1)  = σ111            00              σ22(1)

It follows from Eq.(72) that:

σ21(1) = 0 = M11M21 σ11(0) + (M11M22 +M21M12) σ12(0) + M12M22 σ22(0)

from which

Tan 2α = 2σ21(0)σ220-σ 11(0) 					(80)

or using the definition

r21 = σ21σ11σ22

an alternate form of expressing the ellipse orientation is

Tan 2α =2r21σ11σ22σ22-σ11  = 2r21σ22σ11 - σ11σ22    				(81)

Clearly α is dependent upon the units chosen for σ1 1 and σ22 except in the obvious case of α = O; i.e., an erect ellipse.

SECOND -ORDER ABERRATIONS

Section III

TRANSPORT has the capability of calculating the second-order matrix

elements (aberration coefficients) of any static-magnetic beam transport system

composed of combinations of bending magnets, quadrupoles, solenoids, sextupoles

and interspersed drift spaces. It is assumed that mid-plane symmetry prevails for

any given magnetic element in a system (except for solenoids) but not necessarily

for the system as a whole. The notation used in a TRANSPORT printout is described

in reference 1 (SLAC-75) beginning on page 46?. The subscript notation is the same

as that used for first-order where the subscript 1 means x, 2 means θ, 3 means y,

4 means ϕ, 5 means ℓ, and 6 means δ.

The symbol Rij has been used to signify a first-order matrix element and

the symbol Tijk will be used to signify a second-order matrix element. Thus we

may write the second-order Taylor expansion representing the deviation of an arbitrary trajectory from the central trajectory as:

xi(t) = j=16Rijxj(0) + j=16k=16Tijkxj0xk(0)

where x1 = x  x2 = θ x3 = y x4 = ϕ  x5 = ℓ  and x6 = δ  denotes the subscript

notation. In an actual computer printout, the TiJk 's are abbreviated as (i jk); for example T126 = (x|θ0 δ) would appear in a printout as (l 26) followed by the

computed value of the aberration coefficient for the system being designed.

In order to modify the magnitude of any given aberration coefficient, it

is necessary to introduce multipole component(s) of the magnetic field of order

equal to or less than the order of the aberration. Thus sextupole, quadrupole and

dipole components of the field may all be used to modify any given second-order

aberration. But, in practice, the second—order aberrations are usually minimized

by only introducing sextupole components so as not to disturb the first-order

optics of the system. It should always be kept in mind, however, that it may be

beneficial to go back and change the first-order solution (optical mode) so as to

provide a more favorable situation for correcting aberrations; a wise selection

of the first-order optical mode may in many instances be the deciding factor between the success or failure of a design.

For a fixed location of a sextupole component, the partial derivative

of any second-order aberration coefficient Tijk with respect to the strength S2

of a given sextupole component is a constant. i.e.,

δ Tijkδ S2= a constant = the coupling coefficient of S2 to Tijk .

T

hus minimizing a selected group of aberrations is a straight forward problem

of solving a set of simultaneous linear equations once the coupling coefficients

are known .

The strengths of the sextupole components may be determined directly by TRANSPORT. The user may either constrain certain second order matrix elements to certain values , or may minimize the net second-order contributions to a given component of the beam ellipsoid

Second-Order Phase Ellipsoid Formalism

It will be noted by the user, that a second~order TRANSPORT calculation

modifies the phase-ellipsoid printout. In a second-order run, TRANSPORT calculates

and prints out the second-moments of the phase space distribution function in the

σii columns. In addition, it also calculates and prints out the new coordinates

of the centroid (first—moment) of the phase space distribution function and tabulates this result to the left of the σii  columns in the same manner as it does for a magnet misalignment run.

Caution should be used in the use and interpretation of the second-order

phase ellipsoid results especially if it is known or suspected that the phase space distribution resulting from a second-order run is not symmetrical about the beam centroid. To be certain of the situation in any given design, it would be wise to calculate the actual distribution function by using the Monte—Carlo computer program TURTLE1

The actual method used in TRANSPORT by which the second-order terms are included in the beam ellipse is described in the following report. The reader should bear in mind that the derivation is based on a gaussian initial beam distribution.

For any other initial distribution the second order effects on

the beam ellipsoid should be regarded only as an approximation.

-----------------------------------------------------------------------

1 D. C. Carey, "TURTLE, A Computer Program for Simulating Charged Particle Beam Transport Systems", N.A.L. Report No. 64, Fermi National Accelerator Laboratory, Batavia, Illinois (1971)

-----------------------------------------------------------------------

Second Order Contributions to Beam Dimensions

David C. Carey

May, 1972

I. Introduction

The phase space region occupied by an aggregate of charged

particles in a beam line is often represented by a higher

dimensional ellipsoid. Given no further information, one

might interpret such an ellipsoid as an envelope inside of which

particles are distributed uniformly, or as giving the scale

dimensions of a gaussian distribution. The latter case has

the advantage that is easily adapted to include higher order

effects of the beam line. In either case the parameters of

the ellipsoid are simply related to the first and second moments

and therefore the width of the distribution in any coordinate.

In first order an ellipsoid at any point in a beam line is

transformed into another ellipsoid at any other location in a

beam line. In second and higher orders a transformation from

one location in a beam line to another hill cause the ellipsoid

to become distorted. One can still, however, calculate the

first and second moments of the distribution, and thereby

obtain a measure of its dimensions in any coordinate.

Below we elaborate on the methods for calculating the

ellipsoid parameters at any point in the beam line. Much

of the first order theory can be found in the work of Brown

and Howry.l It is included here for completeness.

II. The Ellipsoid Formalism

The position and motion of a particle in a beam line may

be represented via a six—dimensiona]_ vector.

x =xθyϕlδ					(1)

The coordinates x and y represent respectively the

horizontal and vertical displacements at the position of the

particle, θ and ϕ, the angles with the axis of the beam line

in the same planes. The quantity ℓ represents the longitudinal

position of the particle relative to a particle traveling on

the magnetic axis of the system with the central momentum

designed for the system. The remaining quantity δ = Δpp

gives the fractional deviation of the momentum of the particle

from the central design momentum of the system.

An ellipsoidal hypersurface in this six—dimensional space

may be represented by the equation:

xTσ-1 = l 					(2)

where σ-1 is a symmetric positive definite matrix. We represent

this matrix as an inverse for reasons which will become apparent

later. At this stage the center of the ellipsoid is assumed

to lie at the origin of the coordinate system. The ellipsoid

may be taken to be the envelope of a uniform distribution, or

the scale in a gaussian distribution, giving a particle density:

ρ = C exp (-½ xTσ-1x)				(3)

For any symmetric matrix there exists a coordinate

system in which that matrix is diagonal and an orthogonal

transformation to that coordinate system.2 Let us represent

the orthogonal transformation by the matrix 0, so that:

xi = 	j0ijxj~					(4)

where xj~ are the coordinates in the frame where the transform of σ-1 and therefore that of σ are diagonal. Calling the matrix σ transformed to the new frame σ~ we now have:

σij=kl0ikσkl~0jl 					(5)

and equation (1) becomes

x~Tσ~-1x~=1              			(1a)

Specializing to the gaussian distribution, it is now an easy matter to calculate the second moments in the new frame since the coordipates are decoupled.

We arrive at:

xi~xj~=σij~=δijσij~       				(6)

The second moments in the old frame are now:

xixj= kl0ik0jlxk~xl~=kl0ik0jlxk~xl~= kl0ik0jlσ kl~= σij		(7)

Therefore in this case the elements of the matrix σ give the second moments of the distribution in the original coordinate system. The density function, properly normalized, now becomes:

ρ= N0detσ 2π3 exp -½xTσ-1x			(8)

whore NO is the total number of particles. Since the matrix 0

is orthogonal the determinants of σ and σ~ are equal.

The elements of the matrix σ may be put in more convenient form for interpretation. The square roots of the diagonal elements may be taken as giving the half widths xo of the distribution in a given coordinate while the off-diagonal elements may be related to the correlations rij, so

x0i= σii

rij=σ ij/σ iiσ jj				(9)

Since, for any positive definite symmetric matrix σ, we have:2

σ iiσ jj- σij2> =0				(10)

the correlations must all obey the inequality

rij< 1					(11)

If the ellipsoid is interpreted as describing the envelope of a uniform distribution, then the xoi represent the maximum extents of the beam in the given coordinates.

III. The Effect of a Beam Line

A. First Order

If we now let xj(1) be the coordinates of a ray at the initial point in a beam line, and xj(2) the coordinates at some later point, the two are related by the equation:

xj(2)= jRijxj(1)				(12)

If we continue to assume a distribution centered at the origin the first moments at both initial and final point will be zero.

The second moments will now be given by:

σij(2)= xi(2)xj(2)= ikRikRjlxk(1)xl(1) 			(13)

=klRikRjlσkl(1)

or more concisely

σ(2) = Rσ (1)RT					(14)

To first order an ellipsoid at the initial point will transform into an ellipsoid at the final point, so that the equation:

x(2)T- (σ(2))-1x(2) = 1				(15)

will give the envelope of the particle distribution at the later point.

B. Second Order

In second order the transformation on the coordinates

effected by the beam line is given by:

xi(2)= jRijxj(1)+jkTijk xj(1)xk(1) 			(16)

We employ here a symmetric T matrix whose off—diagonal elements are half

those of the T matrix used by Brown. The first and second moments of the distribution at the final point are now given by

xi(2)= jRijxj(1)+jkTijk xj(1)xk(1) 						(17)

xi(2)xj(2)= kljRikRjl xk(1)xl(1)

+  klmRikTjlm+ TiklRjm xk(1)xl(1)xm(1)

+klmnTiklTjmnxk(1)xl(1)xm(1)xn(1)

For a symmetric, on—axis initial distribution, the first and

third moments vanish. The problem now reduces to determining

the fourth moments of the initial distribution.

As an extension of previous notation we now denote the

fourth moments of the distribution about the initial point by σijkl(1).

We consider the coordinate system in which the matrix of second moments σij

is diagonalized, denoting the moments in

this frame by σ~. Then from equation (7) we have:

σij= kl0ik0jlσkl~ 				(18)

= k0ik0jkσkk~

We continue to specialize to a gaussian distribution so that

the fourth moments will be directly derivable from the second

moments. In the diagonal frame the coordinates separate, and

the fourth moments are easily calculated. The only ones which

are non-zero are σiijj  ~, σijij~, or σijji~, for i≠ j, and σiiii~ with

σiijj  ~= σii~σjj~  							(19)

σijij  ~= σii~σjj~

σijji  ~= σii~σjj~

σiiii  ~= 3 σii~σii~

so that in general:

σijkl  ~= δij δkl σii~σkk~+δik δjl σii~σjj~+ δil δjk σii~σjj~   		(20)

Now if under the transformation O, the fourth moments transform

as:

σijkl  ~= mnop0im0jn0ko0lpσmnop~            			(21)

then from equation (la) we finally arrive at:

σijkl  ~= σij~σkl~+σik~σjl~+ σil~σjk~        				(22)

Substituting into equation (17) we determine that:

xi(2)= jkTijkσjk(1)  				(23)

xi(2)xj(2)= klRikRjlσkl(1)+ klTiklσkl(1) mnTjmnσmn(1)

+ 2 lmkTiklσkm(1)nTjmnσln(1)

Note that, because of the symmetry properties of both T and σ

that the two expressions in parentheses in the last term of the

second equation represent the same array. From a practical

standpoint this means that it needs to be calculated only once.

We see from equation (23) that the centroid of the distribution at the final point no longer coincides with the beam axis.

Letting  σ(2) represent the matrix of second moments about the

new centroid we now have:

σij(2)= xi(2)xj(2)- xi(2)xj(2)   			(24)

= klRikσkl(1)

+ 2 lmkTiklσkm(1)nTjmnσln(1)

IV. Off-Axis Initial Distribution

Now consider a gaussian distribution whose center does not

coincide with the beam axis. Letting the coordinates of the

centroid by xi(1), we have for the coordinates of a ray:

xi(1)= + xi(1)ϵi(1)  					(25)

We let the matrix σ represent the moments of the distribution

about its centroid so that:

ϵi(1)ϵj(1)= σij(1)				(26)

ϵi(1)ϵj(1)ϵk(1)ϵl(1)= σijkl(1)

Equation (17) continues to hold for the moments of the distribution

about the beam axis, vhile equation (22) holds for the moments

about the centroid. We must therefore express one set of moments

in terms of the other.

Using equations (22), (25), and (26) and applying the first

part of equation (24) to the initial distribution, the initial

third and fourth moments are given in terms of the initial

first and second moments as follows:

xi(1)xj(1)xk(1) = xi(1)xj(1)xk(1) + xj(1)xi(1)xk(1)

+ xk(1)xi(1)xj(1) - 2 xi(1)xj(1)xk(1)

xi(1)xj(1)xk(1)xl(1) =  xi(1)xj(1)xk(1)xl(1) +xi(1)xk(1)xj(1)xl(1)

+ xi(1)xl(1)xj(1)xk(1)  - 2 xi(1)xj(1)xk(1)xl(1)			(27)

Substituting into equation (17) and rearranging terms we arrive

at the following expressions for the first and second moments

of the distribution at the final point.

xi(2)= jRijxj(1)+jkTijk xj(1)xk(1) 					(28)

xi(2)xj(2)= klRikRjlxk(1)x2(1)+ xi(2)xj(2) - 2 xi(2)xj(2)

+ 2 lmRil xm1+ kTikl xk1xm1Rjm xl1+nTjmnxl1xn1  –

kRik xk(1) mRjm xm(1)

where xi(2)= kRik xk(1)+kl Tikl xk(1)xl(1)

is the image of the original centroid.

We may now again use equations (9) and (24) to relate this

matrix of second moments to the final beam half widths and

correlations.

Reference

1. Karl L. Brown , Sam K. Howry , SLAC Report No . 91 (1970) .

2. F. R. Gantmachcr, The Theory of Matrices , Chelsea Publishing Co., New York (1959).

A Systematic Procedure for Designing High Resolving Power Beam Transport Systems or Charged Particle Spectrometers

The following is a report submitted to the Third International Magnet

Symposium held in Hamburg, Germany - May 1970. It is a general description of a

suggested procedure for designing systems to any order and includes the derivation

of the coupling coefficient of an nth-order multipole to any nth-order aberration

coefficient. The report also derives the multipole strengths for the three

techniques for introducing multipole components into a system: namely, 1) pure

multipole fields, 2) non-uniform fields, and 3) contoured entrance or exit boundaries of magnets .

The notation used in this report is identical to TRANSPORT notation

except for the following:

Replace x’ and y’ in the report by σ  and ϕ respectively to convert to

TRANSPORT notation

REPORT SUBMITTED TO THE THIRD INTERNATIONAL mam sYMPosIUM

HELD IN HAMBURG, GERMANY-May 1970

by

Karl L. Brown

Stanford Linear Accelerator Center

Stanford, California

Summary

By extrapolating the systematics of the general first- and second-order theory of beam transport optics (1, 2,3) to include higher order multipole terms, it has been possible to evolve a simple, step by step, procedure for the design of high resolving power static-magnetic beam transport systems . The choice of the appropriate dipole and quadrupole elements for a given system may be determined once the resolving power, solid angle, momentum range and detector system of the instrument have been specified. The partial derivative of any nth-order aberration coefficient with respect to an nth-order multipole component located anywhere in the system has been derived. From this "coupling coefficient", the strength and the optimum location of multipole element(s) to correct or modify a given aberration or group of aberrations is uniquely determined.

I . Introduction

Within the last two decades, significant advances have been made in the understanding of charged particle optics. Perhaps the first major contribution was the development of the theory of the Alternating Gradient Synchrotron (A.G.S.) by Courant, Livingston, and Snyder(4) which led to the first-order matrix algebra formulation of beam-transport optics. Subsequent to this a second-order matrix algebra was developed by Brown, Belbeoch, and Bounin(5); followed by the development at SLAC of the digital computer program called TRANSPORT(6) that is widely used today in many laboratories for solving first- and second-order static-magnetic beam transport problems. In principle, the second-order matrix formalism my be extended to any order, but in practice this approach has proved to be too cumbersome. Thus beyond second-order it has been more efficient to use computer ray-tracing programs which integrate the basic differential equation of motion

of the charged particles through the known or assumed magnetic fields. The fundamental difficulty with ray-tracing has been the required computational time to complete a design involving the minimization of many higher-order aberrations.

In this report, we will outline a systematic procedure for the design of high-resolution systems based upon the extrapolation of the first- and second-order theory (1, 2,3) to include higher-order multipole components. A general equation has been derived for the coupling coefficient of an nth-order multipole to any given nth-order aberration coefficient. As will be shown later, these coupling coefficients are a function only of the characteristic first—order trajectories (matrix elements) introduced and defined in References l and 2.

Given this information, a systematic procedure for designing high resolution beam transport systems is as follows:

- Find a satisfactory first-order solution to the problem using TRANSPORT or its equivalent.

- Calculate and make the necessary corrections to the second-order aberrations by introducing sextupole components into the system. The "best" locations and strengths of the sextupole components required may be selected via the coupling coefficients for the aberrations to be minimized.

- Calculate and make the necessary corrections (via ray-tracing) to the third-order aberrations by introducing octupole components into the system. (Note that an nth-order multipole couples with terms of order n or higher but not with terms of order lower than n. Thus an octupole component will not disturb the first- and second-order solutions already found from steps 1 and 2.)

- Repeat the above procedure up to the multipole order desired or needed to achieve the design objectives.

If the design requires a solution to nth-order and m multipoles at each order are necessary to minimize the aberrations, the number of computer runs previously needed to complete a design was at least (n+m)2. Having a knowledge of the coupling coefficients, after the first-order design has been selected, now (in principle) reduces the number of computer runs required to n. Since ray-tracing is very time consuming, this is indeed a significant saving.

II. Theory*

---------------------------------------------------------------

* The notation used in this report follows that used in Ref. 1 unless otherwise indicated.

---------------------------------------------------------------

The following results are applicable to static-magnetic charged particle

optical systems possessing median plane symmetry. As in Ref. 1, we shall use a right-handed curvilinear coordinate system (x, y, t) where x and y are the trans-

verse coordinates. x is the outward normal distance in the median plane away

from the central trajectory, y is the perpendicular distance from the median

plane, t is the distance along the central trajectory, and is the curvature of the central trajectory.

The existence of the median plane requires that the scalar potential ϕ be an odd function of y, i.e., ϕ(x,y,t) = - ϕ(x,y,t). The most general form of ϕ  may therefore, be expressed as follows:

ϕ(x,y,t) = ϕx,y,t= m=0∞n=0∞A2m+1,nxnn! y2n+12m+1!					(1)

where the coefficients A2m+1,n are functions of t.

In this coordinate system, the differential line element dT is given

by

dT2 = dx2 + dy2 + (1+hx)2 dt2

The Laplace equation has the form

∇2ϕ= 11+hxδδx 1+hxδϕδx+ δ2ϕδy2+ 11+hx δδt 11+hx δϕδt=0 			(2)

Substitution of (l) into (2) gives the following recursion formula for the

where prime means 3? , and where it is understood that all coefficients

-A2m+3,n        = A2m+1,n''+nhA2n+1,n-1''-nh'A'2m+1,n-1+A2m+1,n+2          (3)

+3n+1hA2m+1,n+1+n3n-1h2A2m+1,n+ nn-12h3A2m+1,n-1		+3nhA2m+3,n-1+ 3nn-1 h2A2m+3,n-2 + nn-1n-2h3A2m+3,n-3

where prime means ddt ,and where it is understood that all coefficients A with one or more negative subscripts are zero. This recursion formula expresses all

the coefficients in terms of the midplane field By(x, 0,t):

where

A1,n=  δnByδxnx=0y=0  = functions of t							(4)

Since ϕ  is an odd function of y, on the median plane we have Bx = Bt = O. The normal (in x direction) derivatives By on the reference curve defines By over the entire median plane, hence the magnetic field B  over the whole space. The components of the field are expressed in terms of ϕ  explicitly by B = ∇ϕ or

Bx= δϕδx= m=0∞n=0∞A2m+1,n+1 xnn!  y2m+12m+1!

By= δϕδy= m=0∞n=0∞A2m+1,n xnn!  y2m2m!

Bt=11+hx δϕδx=11+hx m=0∞n=0∞A2m+1,n xnn!  y2m+12m+1! 				(5)

The expression for the magnetic field on the midplane is

Byx,  o,t = n=0∞A1,nxnn!   									(6)

At this point we deviate from the notation and formalism of Ref. 1 and introduce Kn(t),the multipole strength per unit length; and sn’, the total multipole strength of a static-magnetic field.

We rewrite equation (6) as

By(x,o,t) = Bρ n=0∞Kn(t) xn 								(7)

Where Bρ = Bh = Poe is the magnetic rigidity of a particle of momentum Po and

charge e along the central trajectory; from which

Knt=1Bρ 1n!A1,nt= 1Bρ1n!δnByδxnx=y=0 					(8)

We define Sn as

Sn=0LKntdt   										(9)

Sn so defined is the strength of the nth-order multipole component of a field

over the interval of integration.

Multipole Strengths for Pure Multipole Fields

Consider the scalar potential of an nth-order [2(n+1)pole] pure multipole

element:

ϕ = B0rn+1n+1an sinn+1 θ 				(10)

where

x = r cosθ   and y = r sinθ

Bo is the field at the pole and a is the radial distance to the pole from the central trajectory.

Expanding ϕ as a function of x and y and differentiating, we have

By= δϕδy= Boan xn+ ……..

From which

Kn=Boan1Bρ

and

Sn=BoanLBρ  							(11)

Where L is the length of the multipole element.

For a dipole and the dipole strength is

So= Lρ=α (The angle of bend of the central trajectory)

For a. quadrupole and

S1=BoaLBρ

For a sextupole and

S2=Boa2LBρ

etc. for higher-order multipoles .

Multipole Strengths for a Non-Uniform Field Expansion

From the midplane field expansion of a. non—uniform magnetic field

and evaluated over the length L of the central trajectory is:

By(x,o,t) = By(o,o,t) 1-nhx+βhx2+yhx3+ …….. 			(12)

K0 = h,  K1 = -nh2,  K2 = βh3,  etc.

and Sn evaluated over the length L of the central trajectory is:

S0 = hL =α  as before,

S1 = -nh2L,  and S2 = βh3L, etc.

Multipole Strengths for a Contoured Entrance or Exit. Boundary of a Magnet

A third method of introducing multipole components is via a curved entrance

or exit boundary of a magnet. To calculate the multipole strengths in this case,

we integrate equation (7), holding x constant, as follows:

oLByx,o,tdt=BρxnoLKntdt=BρSnxn			(13)

To relate this to the field boundary, we assume By to be a constant inside the effective field boundary and zero outside (i.e., we ignore the finite extent

of the fringing field) . In this sharps -cutoff approximation, the field boundary

Z = Z(x) is:

Z = 1By oLByx,o,tdt=1hSnxn = x tan β + S2h x2                     (14)

where h = 1ρ and β = The “quadrupole strength”

The radius of curvature of the boundary is related to the sextupole strength as follows:

1R=Z''1+z'23/2=2S2h sec3β

or

S2= hsec3β2R=The "sextupole strength"

From equation (13), we note that a positive multipole component of the field

increases the Bdt for a positive x; thus a positive sextupole is represented

by a concave surface of the entrance or exit boundary.

The Descrigtion of the Trajectories as a Taylor's Expansion

The deviation of an arbitrary trajectory from the central trajectory is

described by expressing x and y as functions of t. The expressions will also

contain xo, yo, x’o, y’o and δ, where the subscript o indicates that the quantity is evaluated at t = o. The prime (') denotes the derivative with respect to t, and δ =  is the fractional momentum deviation of the ray from that of

the central trajectory. These five initial boundary values will have the value

zero for the central trajectory itself. an and y are expressed as a five-fold

Taylor expansion using these initial boundary values. The expansions are

written:

xt = x|x0κy0λx'0μy'0υ δχx0κy0λx'0μy'0υ δχ

yt = y|x0κy0λx'0μy'0υ δχx0κy0λx'0μy'0υ δχ					(15)

Here, the parentheses are symbols for the Taylor coefficients; the first part

of the symbol identifies the coordinate represented by the expansion, and the

second indicates the term in question. These coefficients are functions of t to be determined. The symbol indicates summation over zero and all positive integer values of the exponents Κ, λ, μ, ν, χ ;. The constant term is zero,

and the terms that would indicate a coupling between the coordinates x and y

are also zero; this results from the midplane symmetry. Thus we have

(x|1) = (y|1) = 0

(x|y0) = (y|x0) = 0

(x|y’0) = (y|x’0) = 0 								(16)

Here, the first line is a consequence of choosing the central trajectory as

the reference axis, while the second and third lines follow directly from

considerations of median plane symmetry.

Since they will appear often in the formalism, it is convenient to

introduce the following abbreviations for the first-order Taylor coefficients:

(x|x0) = cx(t)	(x|x’0) = sx(t)		(x|δ) = dx(t)

(y|y0) = cy(t)	(y|y’0) = sx(t)						(17)

When the transverse position of an arbitrary trajectory at position t is

written as a first-order Taylor's expansion as a function of the initial

boundary conditions, the above five quantities are just the coefficients

appearing in the expansion for the transverse coordinates x and y as follows:

x(t) = cx(t)x0 + sx(t)x’0 + dx(t)δ  + higher-order terms

and

y(t) = cy(t)y0 + sx(t)y’0 + higher-order terms.

III. Solution of the Equations of Motion

The general differential equation of motion of a charged particle in a

static-magnetic field valid to all orders in x and y and their derivatives as

derived in Ref. 1, equation (5) is:

xx''- h1+hx –x'T'2x'x''+ y'y''+ 1+hxhx'+h'x

+y y''-y'T'2 x'x''+y'y''+ 1+hxhx'+h'x

+t 2hx'+h'x-1+hxT'2 x'x''+y'y''+1+hxhx'+h'x

=eP T'T '   X B  =eP T’x y'Bt- 1+hxBy+ y  1+hxBx- x'Bx+ t x'By –y'Bx (18)

If this equation is solved to nth-order for the Taylor's coefficients

of equation (15), it will be observed that the result has the remarkably

simple form:

xi|x0κy0λx'0μy'0υ β χ= ±n!κ!λ!μ!ν!χ!0tGi  t,τcxκ τcyλ τsxμτsyν(τ)dxχτKnτdτ

+ terms containing K0’ ……….Kn-1					(19)

where the variable of integration is τ and n = (κ + λ + μ + ν + χ ).

The xi have the following meaning:

x1 = x(t)	x2 = x’(t)	x3 = y(t)	x4 = y’(t)

cx’ cy’ sx’ sy’ and dx are defined by equation (17) and in general are function of the variable of integration τ over the interval of integration. Kn is defined by equation (8) and in general is also a function of τ .

The Gi's are Green's fimctions where:

G1t,τ= xt|x'τ= sxtcxτ - cxtsxτ

G2t,τ= x't|x'τ= s'xtcxτ – c'xtsxτ

G3t,τ= yt|y'τ= sytcyτ - cytsyτ

G4t,τ= y't|y'τ= s'ytcyτ – c'ytsyτ                 (20)

Note that the Gi‘s are Just first-order Taylor's coefficients measured from the location (τ) of the multipole component to the end of the system (t) .

Thus we see that the coupling coefficient to an nth-order multipole is a function only of the first-order matrix elements cx, cy, sx, sy, dx and their derivatives with respect to t.

From median-plane symmetry considerations, the allowed aberrations are those with and/or y’ appearing an even number of times in the Taylor coefficient. For example x|x02, (x|y0y'0) and y|y02y'0 are allowed aberrations;

whereas x|y0, x|x02y'0 or  y|y02 are not allowed and are therefore equal to zero.

The minus sign is used when y and/or y’ appear 0, 4, 8, 12 …. times and the plus sign is used when y and/or y’ appear 2, 6, l0 …. times. For example for the coefficients x|x02 and y|y03, the minus sign is applicable; whereas for the coefficients x|y02  and  y'|y03y'02 the plus sign is applicable.

Equation (19) is derived by observing in the pattern of the solution of the differential equation that an nth-order aberration term containing the nth-order multipole strength factor Kn cannot include multipole strength factors of lower order than n; or stated physically, an nth-order multipole cannot couple to aberrations (terms) of order lower than n. This fact allows the recursion formula equation (3) to be reduced to the simple form

A2m+3,n = -A2m+1,n+2 				(21)

in so far as it applies to the derivation of nth-order terms containing only Kn. As a consequence, the scalar potential for deriving these terms assumes the simplied form

ϕx,y,t= m=0∞n=0∞-1m A1,2m+nxnn!y2m+12m+1!	 					(22)

From which, it follows that

Bxx,y,t= P0em=0∞n=0∞-1m K2m+n+12m+n+1!2m+1!xny2m+1

Byx,y,t= P0em=0∞n=0∞-1m K2m+n2m+n!n!  2m!!xny2m 				(23)

For terms containing only Kn, the basic differential equations assume the

form:

x''+ ………= -ePBy

y''+ ………= -ePBx  									(24)

Substituting the Taylor's expansion of equation (15) and solving for the nth-order terms using a conventional Green's function solution (see Ref. 1) yields equation (19) above.

IV. Interpretation and Use of Equation (19)

For most practical cases of interest, Kn will be a constant over the interval of integration. In this event we may define the coupling coefficient

of an nth—order multipole to an nth—order aberration as the partial derivative

of equation (19) with respect to the Kn in question as follows:

δxi|x0κy0λx'0μy'0υ δ χδKn= ±n!κ!λ!μ!ν!χ! 0LGi  cxκ cyλ sxμsyνdxχdτ 				(25)

where now the interval of integration is over the multipole length L represented

by Kn. For a distributed multipole component (such as in a non—uniform field bending magnet), equation (25) is used.

In many cases where a curved entrance or exit pole contour is used or a

short multipole magnet is used such that the characteristic first-order functions cx, cy, sx, sy, and dX are essentially constants over the interval of integration (the length of one multipole), then the coupling coefficient is best

defined as the partial derivative of equation (19) with respect to Sn as follows:

δxi|x0κy0λx'0μy'0υ δ χδSn= ±n!κ!λ!μ!ν!χ! G1cxκ cyλ sxμsyνdxχ 					(26)

Examples

Assume a situation where the end of the system is a point-to-point image

or the origin (i.e., sx(t)=0) then using equation 26, the coupling coefficients

of a sextupole of strength S2 to various second order aberration coefficients

are:

δx |x'0 δδS2= cxtSx2dx

δx |x'02δS2= cxtSx3

δx |y'0 y'0δS2= 2cxtcysxsy   								(27)

etc. Where the Green's function used in these examples is

G1 = sx(t)cx - cx(t)sx = - cx(t)sx (since sx(t) = 0 for point-to-point 								imaging)

The aberration and cx(t) are evaluated at the end of the system. cx(t) is equal to the magnification Mx in the examples given. The remaining coefficients cv, sx, sy and dx are evaluated at the location of the sextupole S2. The above results are in agreement with Table VII of Ref. 1.

To illustrate a more complex example, consider the fourth-order aberration coefficient (y |y02 y'0 δ) and assume parallel-to—point imaging in the y coordinate (i.e., cy(t) = O). The appropriate Green's function is:

G3 = sy(t)cy – cy(t)sy = - sy(t)cy

and the coupling coefficient to a fourth-order multipole of strength S4 is:

δy |y02 y'0 δδS4  = 4!2! sy(t)cy3sydx 								(28)

where again the aberration coefficient  y |y02 y'0 δ and sy(t) are evaluated at the end of the optical system and cy, sy, and dx are evaluated at the

location of the fourth-order multipole S4.

V. A Systematic Procedure for Designing High Resolution Systems First-Order Considerations

In many respects, the determination of a satisfactory first-order magnetic

optical design is more difficult to achieve than is the subsequent higher-order

design. This is true not only because the basic equipment configuration is

dominated by first-order optical considerations but also because the choice of

the first-order optics affects the magnitude of all higher-order aberrations

and the ease with which these aberrations may be minimized by introducing multipole components into the design.

The dominating design parameters that must be clearly specified in order

to evolve a first-order design are the momentum resolving power; the spatial resolution of the particle detector system to be used (this determines the momentum dispersion required); the required phase space acceptance (the solid

angle, the source size, and the momentum range) of the instrument, and the

first-order imaging requirements in both the x and y coordinates.

Given the above specifications (assuming they are self-consistent), the optical mode and physical configuration of the instrument may be determined.

Often, more than one theoretical solution exists; in which case the choice is usually resolved by practical or economic considerations. In other cases,

no solution is evident and the basic specifications must be modified accordingly. In any event, the following equations and discussion are applicable to

the solution of the problem.

l) First-Order Resolving Power

A general equation for the first-order resolving power has been derived in References (l,2, and 3) . For point-to-point imaging the first-order

momentum resolving power R1 is defined as the ratio of the momentum dispersion at the image plane to the total image size. Thus if 2x0 is the total source

size then from Reference 1 we have:

R1=PΔP=dxt2x0cxt=12x00tsxτhτdτ    						(29)

Note that h(τ )dτ = dα  is the differential angle of bend of the central trajectory of the optical system.

Equation (29) may be expressed in a number of useful forms. If we consider

a particle originating at the source with xo = 0 and 5 = 0 and δ  = ΔPP0  and lying 1n the midplane (i.e., a monoenergetic point source), the first—order equation of

its trajectory is

x(τ) = sx(τ)x’0					(30)

We may then rewrite equation (29) as follows:

R1=12xox'ootxτhτdτ=l-lo2xox'o							(31)

where (l-lo) is the path length difference between the trajectory ‘described by equation (30) and the central trajectory. Or we may also write equatien (31) in the form

R1=12xox'ootBxτdτBρ=12xox'o1BρotBdA 					(32)

where BdA is the magnetic flux enclosed between the central trajectory and the trajectory described by equation (30), and Bρ  is the magnetic rigidity of the central trajectory. Please note, however, that if the trajectory of equation (30) crosses the central trajectory or the sign of B changes, this changes the sign of the integration. From equation (32) we may define resolving power as the magnetic flux enclosed per unit phase space area (2xox’o), per unit momentum (Bρ ) of the central ray.

In any given design, one or more of the ebove equations may be used as a guide toward achieving the required resolving power. One of the design decisions that must be made is the appropriate choice of the dipole magnet parameters (width and length) to achieve the required BdA. From first-order considerations, this choice is dominated primarily by practical and economic factors. However, a study of the nature of the origin of aberrations (see for example Ref. 1) suggests that it is advisable to keep the amplitude of sx small. In order to simultaneously satisfy this requirement and meet the required resolving power R1, we see from equation (29) that the total angle of bend α of the central trajectory should be chosen as large as is practical. Also, in general, the focal plane angle tends to be more normal to the optic axis for larger α - a property usually desired in most designs.

2) Dispersion

From Reference 1, 2, or 3; for point-to-point imaging (sx(t)=o) the

dispersion at the image plane is

dxt=-cxt0tsxτhτdτ 								(33)

where cx(t) is the magnification at the image plane.

The dispersion and hence the magnification in the design of a spectrometer is dominated almost entirely by a compromise between the spatial resolution of the particle detectors used at the image plane and the momentum range to be covered by the instrument; or in the case of a momentum defining (analyzing) system, by the acceptable momentum-defining slit spacings.

3) The Selection of the Optical Mode

By optical mode, we mean the type of imaging (e.g., point-to-point

or parallel-to-point, etc.) required at the image plane in both the x and y

coordinates, and the number of intermediate images imposed between the source

and image planes. The imaging requirements at the image plane are usually

dominated by the physics to be performed by the instrument and the nature of the particle detectors used. However often (especially at low energies) the

imaging in the y plane may be unimportant as far as the physics requirements are

concerned which in turn provides some additional flexibility in the optics design.

A study of the coupling coefficients to the aberration coefficients

(equation l9) shows the not surprising result, that multipoles located at

intermediate images in a system do not couple to aberrations in the plane in

which the intermediate image occurs. Hence it often proves beneficial to

intentionally create an intermediate image in the y plane of an optical system

so as to achieve Some degree of "orthogonality" in the minimizing of x and y

aberrations .

The considerations of l), 2), and 3) above are the determining factors

in the selection of the first-order solution of a system design.

The optical mode and dispersion of the system are determined to a great

extent by the choice of the quadrupole components chosen to achieve the first

order imaging although it is clear that the dipole elements also influence the

first-order imaging to a greater or lesser extent depending upon the total

angle of bend of the system.

4) Aberrations and their Correction

A study of the source of second- and higher—order aberrations (see

for example Ref. 1) suggests that it is advisable to maintain the characteristic

first-order functions cx, sx, dx and cy, sy and their derivatives as small as is feasible through the magnetic elements of a system when choosing the first

order design. This procedure will tend to reduce the initial size of the

aberrations and hence simplify the problem of minimizing them by the addition

of multipole components to the system design.

The procedure for minimizing aberrations has already been outlined in the

Introduction and as such will not be repeated here. The "key" to the minimization procedure is the coupling coefficient given by the integral expression in

equation (19) . The "best" location for the correcting multipole is where the

coupling coefficient has its maximum value.

The preferred method of introducing the multipole components, i.e., via

pure multipoles, contoured entrance or exit boundaries, or non-uniform fields

is a combination of practical and economical considerations and, of course,

personal taste and experience. All three methods have been used with pure

multipoles dominating the situation for higher energy physics and the other two

methods dominating medium and low-energy physics applications. All three

techniques should be considered in any given design situation to be certain

that an important economic or practical advantage has not been ignored.

REFERENCES

- "A First— and Second-Order Matrix Theory for the Design of Beam Transport

Systems and Charged Particle Spectrometers" SLAC Report No. 75 by K. L.

Brown or Advances in Particle Physics Interscience 1 71-134 (1967).

(2) 	"A General First- and Second-Order Theory of Beam Transport Optics", Proc.

of the International Symposium on Magnet Technology (1965); p. 141.

(3) 	"Some First- and Second-Order Magnetic Optics Theorems Applicable to the

Design of Beam Transport Systems and Charged Particle Spectrometers",

Proc. of the International Symposium on Magnet Technology (1967); p.40.

(4) 	E. D. Courant and H. S. Snyder, "Theory of the Alternating Gradient

Synchrotron", Annals of Physics 3, pp. 1-48 (1958).

(5)	K- L. Brown (with R. Belbeoch and P. Bounin), "First- and Second-Order

Magnetic Optics Matrix Equations for the Midplane of Unifom—Field Wedge 	Magnets, Rev. Sci. Instr. 35, 481 (l964).

(6)	K. L. Brown, B. K. Kear, S. K. Howry, "TRANSPORT/360 A Computer Program 	for Designing Beam Transport Systems", SLAC Report No. 91, Stanford Linear

Accelerator Center, Stanford University, Stanford, California (1970)

THE EFFECT OF BEAM LINE MAGNET MISALIGNMENTS

Karl L. Brown and Frank Rothacker

Stanford Linear Accelerator Center, Stanford, California

David C. Carey

Fermi National Accelerator Laboratory, Batavia, Illinois*

and

Christoph Iselin

CERN, Geneva, Switzerland

---------------------------------------------------------------

*Operated by Universities Research Association Inc. under contract

with the Energy Research and Development Administration

---------------------------------------------------------------

The misalignment of a magnet in a beam line will cause

an alteration of the beam envelope at any later point in that

beam line. The position of a misaligned magnet may be described

in terms of six coordinates, three translational and three

rotational. The effect of a misalignment on a single particle

trajectory is derived to first order, including bilinear terms.

A bilinear effect is one which affects the beam line focusing

characteristics, but not the central ray, such as the effect of

rotating a quadrupole about its axis. The effect on the beam

envelope is calculated, both for a known magnet displacement and

for an uncertain magnet position. The formalism has been included

in the computer program 'IRANSPORT.l

I. Introduction

The effects of magnet misalignments are an important

consideration at every stage of beam line design, installation,

and operation. The selection of the optical mode, determination

of surveying accuracy requirements, and the choice of correcting

elements are all dependent on misalignment information.

Two types of misalignment information are typically needed.

To assess the general effect of misalignments in the design stage,

one needs to know the change in beam position and beam line

transmission characteristics due to uncertainties in the position

of each magnetic element in each separate coordinate. Secondly,

to provide for correcting elements, one needs to know the effect

on the beam of specific misalignments. I

In the following we derive a method of determining the

effect of magnet misalignments on a particle beam. We first

define a reference system in which to express misalignments.

Then we determine the effect of a misalignment on individual

particle trajectories. Finally we express the effect on the beam

envelope which describes the ensemble of particles comprising the

beam.

II. Particle Trajectory Coordinates

To specify the position and direction of a particle at any

instant in time, we employ a coordinate system defined with respect

to the beam line reference trajectory.2 The z axis is taken to

point along the reference trajectory; the x axis points to the

left, and the y axis points up. The position and direction of

the particle trajectory can then be given by a vector with six

components

X= xx'yy'lδ 				(1)

The quantities x, x’, y, and y’ are respectively the horizontal

displacement and slope, and vertical displacement and slope of

the ray with respect to the central reference trajectory. ℓ  is

the longitudinal separation of the ray from a ray which enters

the beam line at the same time as the given ray and travels

along the central trajectory. δ  is the fractional momentum

deviation of the particle from the design momentum of the beam

line.

When a charged particle passes through a perfectly aligned

magnet, the transformatiqn may be described to first order by the matrix equation2

X(1) = R X(0) 				(2)

The sets of six coordinates X(0) and X(l) give the particle

position and direction at the entrance and exit faces of the

magnet respectively.

When a magnet is misaligned, the central trajectory of the

magnet is no longer continuous with the central trajectory of

the beam line (see figure 1 below). In particular, at both

the entrance and exit faces, the reference coordinate system

external to the magnet no longer coincides with the reference

coordinate system internal to the magnet (see figure 2 below).

The misaligned and aligned reference coordinate systems are

related by a translation of origin plus a rotation of axes.

We continue to use X(O) and X(l) to denote respectively

the entrance and exit face ray coordinates in the aligned

coordinate systems. We use a subscript f to denote the ray

coordinates Xf(O) and Xf(l) expressed in the misaligned

reference coordinate systems. To first order the ray coordinates

in the misaligned coordinate systems may be expressed in terms

of those in the aligned coordinate systems by an affine

transformation

Xf(0) = S0 X(0) – D0						(3)

Xf(1) = S1 X(1) – D1						(4)

The symbols SO and S1 represent six by six matrices, whose

form will be derived below. The two six-vectors Do and D1

are translations in the six dimensional space of particle

coordinates. The three vectors D0 and D1 formed from the

displacement coordinates (x, y, and z) of DO and D1 give the

displacement due to the misalignment of the origins of the

reference coordinate systems. These two three-vectors are

shown in figure 1.

III. Magnet Misalignment Coordinates

The alignment of a rigid magnet has six degrees of freedom,

three translational and three rotational. These are conveniently

represented by the six quantities

m= δxθxδyθyδzθz 				(5)

where δx, δy, δz are the displacements in the x, y, and z

directions, and θx, θy, and θz are the rotations about the

x, y, and z axes respectively. The origin of the xyz coordinate

system, called the pivot, is the point about which the

misalignments are measured. If the pivot point is located at

some point on the reference trajectory, the x, y, and z axes

of the alignment coordinate system are taken to coincide with

the x, y, and z axes of the beam line reference coordinate

system.

The misalignments form a mathematical group, which is the

Euclidean group in three dimensions. This group is non—commutative

and the order in which the misalignments are imposed is important

if terms of higher order than linear are included. In practice,

however, misalignment values are sufficiently small so that a

first—order approach is justified. For these reasons, we consider

only those terms which are of first order in the misalignment

parameters.

IV. Transformation of Particle Trajectory Coordinates

We now temporarily delete the indices 0 and 1 indicating

the entrance and exit magnet faces respectively, and consider

the effect of a misalignment at a single magnet face. Later

we will combine the results from the two faces to obtain the

net effect of a misalignment.

When the components of the misalignment vector m are

small, we may expand the matrix S and the centroid displacement

D in the misalignment parameters. Retaining only first-order

terms we have

D = Am

S = I + Bm 					(6)

The six by six matrix A represents a transformation from the

misalignment parameters to the particle coordinates. I is the

identity matrix, and B represents a set of six matrices, one

for each of the misalignment parameters. A single six by six

matrix Bm is obtained by multiplying each of the six matrices

by its corresponding misalignment parameter and summing the

results. In terms of the misalignment parameters, the particle

coordinates in the misaligned reference coordinate system now

take the form

Xf = X – Am +BXm				(7)

To derive the forms of the matrices A and B, we consider

separately the effect of each of the various misalignment

parameters on each of the ray coordinates. First we derive

the effect on the ray coordinates of the various misalignments

as expressed in the coordinate system of the aligned magnet

face. Then we will express the misalignment of the magnet

face in terms of the misalignment parameters about the pivot

point.

A rigid translation of the magnet face will change the

x, y, and z coordinates of a ray by the amount of the displacement.

The z translation will also introduce a short drift distance

(positive or negative length) at the magnet face, and will

contribute to B via the transformation matrix of that drift

space.

To determine the effect of a rotational misalignment we

form from the ray angles x’ ( = dx/dz) and y’ ( = dy/dz) and

the number 1 ( = dz/dz) , a three—vector (x',y',l) giving the

ray direction. We let θx , θy, θz be the three rotational components of the misalignment vector. Then, including only first-order effects, this three-vector is transformed as

x'y'1=1θz-θy-θz1θxθy-θx1x'y'1								(8)

(mancano f in fondo prima parentesi e segni dei vettori su θ )

In the misaligned coordinate system the ray angles become

x’f = x’ – θy + θz y’

y’f = y’ + θx – θz x’				(9)

(mancano segni dei vettori su θ )

Thus coordinate rotations about the aligned magnet face x and

y axes only shift the ray angles. A rotation about the z axis

mixes x‘ and y'.

If we let m represent the misalignment parameters relative

to the aligned magnet face coordinate system, and A and B

be the corresponding matrices, then equation (6) holds using the

barred quantities. Using the results derived above, the matrices

A and B are now given by

A =1000000001000010000-10000000010000000					(10)

B125 = B345 = 1

B136 = B246 = - B316 = - B426 = -1 					(11)

(mancano segni dei vettori su B )

The first two indices for B correspond to the ray coordinates

and the third corresponds to the misalignment parameters. All

other elements of B are zero.

In order to express the quantities m in terms of m, the

misalignment parameters at the pivot point, we need two items.

First is the orthogonal matrix 0 giving the three translational

coordinates at the magnet face in terms of those at the pivot

point

xf = 0xp 					(12)

Also needed is the three-vector P which gives the position of

the origin of the aligned magnet face coordinate system, in

the coordinate system of the pivot.

We now define two three—vectors which give the translational

mX and rotational mθ parts of the misalignment vector m. We

also do the same for m. Then the contribution of mx to mx

is given by equation (11) , so that

mx = 0 mx  					(13)

The contribution of mx to mθ  is zero, since parallel translations

do not affect angles.

The displacement of a point due to a rotation about the

pivot is given by the vector product of the rotation vector and

the position vector of the point. Therefore the displacements

of the magnet face mx due to a rotation at the pivot are given

by

mx = 0mθ  ⤬   P					(14)

The orthogonal transformation indicated by the matrix 0 gives

the misalignment parameters in the magnet face coordinate

system. Finally the transformation of rotational misalignment

parameters is again given by equation (11) , so that

mx = 0 mx 					(15)

V. Evaluation of the Relevant Matrices

We choose the pivot to be the origin of the aligned magnet

entrance face coordinate system. Therefore we have

A0 = A

B0 = B						(16)

For the exit face, the matrix 0 transforms from the

aligned entrance face coordinate system to the aligned exit

face coordinate system. The vector P gives the position of

the origin of the aligned exit face coordinate system in the

aligned entrance face coordinate system. In figure 1 it is

the vector which reaches from A to B.

For the exit face of a bending magnet we therefore have

0=cosα0sinα010-sinα0cosα 								(17)

P=-ρ(1cosα0ρsinα  								(18)

where ρ is the radius of curvature of the central trajectory

and α is the total bend angle. We then derive for the matrices A1 and B1

A1 =cosα00sinαsinα00001000-sinα100-ρ1-cosα0-cosα000-sinα-sinα00-ρ1-cosαcosα0000000		(19)

B(1)121 = B(1)341 = -sinα

B(1)124 = B(1)344 = -ρ(1-cosα)

B(1)125 = B(1)345 = cosα

B(1)132 = B(1)242 = -B(1)312 = -B(1)422 = sinα

B(1)136 = B(1)246 = -B(1)316 = -B(1)426 = -cosα				(20)

All other elements of B(l) are zero.

To calculate Al and B1 for a quadrupole, we take the

limit α +0 with αρ  = L, the length of the magnet being held

fixed. Then we have

A1 =100L000001000-L10000-10000000010000000					(21)

and B1 = B as given above.

VI. Effect on the Beam Envelope

To first order, the coordinates at the misaligned magnet

exit face are related to those at the misaligned entrance face

by a transfer matrix, so that

Xf (1) = R X (0)						(22)

or

X(1) – A1m + B1X(1)m = R[X(0) –A0m + B0X(0)m]				(23)

If we solve for X(l) and discard all terms in m of order

higher than first, we then derive

X(1) = R X(0) + [A1 – RA0]m +[RB0-B1R] X(0)m				(24)

For later use we define two new matrices F and G given by

F = A1 – RA0

G = RB0-B1R						(25)

so that

X(1) = RX(0) + Fm +GX(0)m					(26)

An ensemble of particles in a beam line is often

represented as a six-dimensional ellipsoid. The equation of

this ellipsoid may be written in matrix form as follows:

XTσ-1X = 1 							(27)

where XT is the transpose of the coordinate vector X, and σ

is a real, positive definite, symmetric matrix. The square

roots of the diagonal terms of the sigma matrix are a measure

of the beam size in each coordinate. If the centroid of this

ellipsoid does not fall on the central trajectory, then one

needs to specify this centroid position also. The sigma matrix

then gives the beam dimensions as measured about the centroid.

The beam envelope entering a misaligned magnet may be

described in terms of the position in the aligned coordinate

system of the beam centroid and the sigma matrix. For a known

misalignment m, the centroid is transformed as in equation (25).

The sigma matrix is transformed by

σ(1) = Rσ (0)RT + Gσ (0)mRT + Rσ (0)mTGT + Gσ (0)mmTGT 		(28)

where the superscript T indicates a transpose.

For an uncertainty in position we define a covariance

matrix  mmT measuring the distribution of possible magnet

positions. The sigma matrix, which represents the beam

envelope entering the magnet may contain contributions from

both the original beam and from the uncertainty in positions

of previous magnets. We assume there is no correlation of

errors of positioning between any two magnets. The beam

centroid is unaffected by an uncertainty in position. The

transformed sigma matrix becomes

σ(0) = Rρ (0)RT + F mmTFT + Gσ (0) mmT GT 		(29)

If the original sigma matrix is zero, then the resultant

sigma matrix represents the uncertainty in the beam centroid

upon leaving the magnet. If the original sigma matrix encloses

a region of _phase space, then the resultant sigma matrix

represents the envelope of possible particle trajectories,

including both the undisturbed sigma matrix and the effects of

the misalignment.

VII .

This model for misalignments has been implemented in the

computer program TRANSPORT1.An arbitrary misalignment m may

be imposed on any magnet or section of the beam line.

Misalignments may also be nested. The effect of all misalignments

may then be added into the sigma matrix and thereby be traced

through the system. Alternatively, the effects of separate

components of the misalignment vector on individual magnets

may be stored in a table. This table is traced through the

beam line and may be compared with the unperturbed sigma

matrix at any later point. Details of implementation are

described in the TRANSPORT manual.

Figure Captions

Figure l

Perfectly aligned and misaligned bending magnets.

With the misaligned magnet the beam line reference

trajectory is no longer continuous with that inside

the magnet. The displacements of the origins of the

entrance and exit face reference coordinate systems

are shown as D0 and D1 respectively.

Figure 2

Magnet entrance and exit face coordinate systems.

The misalignment causes both a translation and a

rotation of the reference coordinate system.

FIRST-ORDER PARAMETER OPTIMIZATION AND COVARIANCE

Section V

To optimize the selected parameters, TRANSPORT uses the method of non

linear least squares, differential correction, as good description of which is –found in Ref. (1) below pages, 390-393. A useful by-product of this method ‘is the covariance matrix C, printed by the program at the successful conclusion of any run involving parameter fitting. In many applications C may be used to estimate tolerances on the fitted parameters subject to the specified tolerances (i.e., the standard deviations) of the constraints.

The covariance matrix C is symmetric. This admits a geometrical interpretation as an ellipsoid, and is printed in the same suggestive format as is the beam ellipsoid σ, except that in this case the dimension is equal to N, the number of parameters varied. The center of the ellipsoid is at

λ0 = λ10,λ20… λN0,  					(1)

the N values found by TRANSPORT to be the best estimate of the varied parameters.

The equation of the covariance ellipsoid is

(λ – λ0) C-1 (λ –λ0)T ≤1   				(2)

where (C11)1/2 , the first diagonal element printed, measures the maximum extent of

the ellipsoid along the Al axis (the first varied parameter) in the same sense that (σ11)l/2 measures the maximum extent of the beam ellipsoid along the x axis. The off diagonal elements are normalized so that they are ≤ l in magnitude, in analogy with the rij of the beam matrix, and can be interpreted as measures of the orientation of the covariance ellipsoid.

(7) 	SOLMITZ, Analysis of Experiments in Particle Physics,

Ann. Rev. Nuc. Sci., Vol 14, 1964.

K. Halbach, "A Program for Inversion of System Analysis and its Application 	to the Design of Magnets", Second International Conference on Magnet 	Technology, Oxford, 1967.

The best, estimates (or optimized values) of the varied parameters λ are

precisely those that minimize the quantity:

x2= K=1Mξ K-fKλ1…λNSKK2 							(3)

where:

M = number of constraints

fK = a function selected by the code digits (i,j) on the constraint 	definition (Type Code 10.). For example, I = -l, j = 1 means that the 	transform matrix element R11 is to be constrained.

ξK = EK = the desired value of the selected function.

SKK = the desired accuracy of fit (i.e., the standard deviation).

In our notation this minimum is expressed by:

xmin2= K=1MξK-fKλ10……..λN0SKK2 							(4)

xmin2 is printed at the successful conclusion of any run involving parameter fitting.

Whether or not the optimization (xmin2, λ0) is ‘acceptable’ depends on each application and must be evaluated by the user. Values of xmin2 ~ (M-N) are sometimes (but not necessarily) regarded as 'good' . In particular if M-N, then an exact; solution, xmin2 = 0 should be found by TRANSPORT.

If the resulting fit is acceptable, then the following interpretation may

be put on the covariance matrix C: Let the parameters be changed to values λ  near

the optimum λ0 , such that they stay within the ellipsoid defined by:

(λ – λ0) C-1 (λ –λ0)T ≤1

Then the resulting deviation of the specified constrained quantities is

bounded by:

K=1MξK-fKλSKK2≤ xmin2+ 1 				(5)

This interpretation is strictly true if the constrained functions fK are linear in the parameters λ . In the non-linear case, it is an approximation valid only in

some neighborhood of λ0.

are linear in

Example:

On the following page is an example of a TRANSPORT data deck and the resulting covariance fit of a first-order run. We have ask for a point-to-point image in both the x and y planes by varying the fields of the quadrupole triplet. The following definitions and solution are applicable:

B1 = λ1 , B2 = λ2 , B10 = λ10 ,  B20 = λ20 , f1 = R12 , f2 = R34 , ξ1 = 0 , ξ2 = 0

S11 = 0.005 , S22 = 0.005 and N = 2.

From the data deck and the TRANSPORT printout* shown on the following

page, we learn that:

xmin2N=0.8

B10 = -7.4096 Kg,  B20 = 6.1577 Kg

C11  = 0.079

Coviance matrix information

r12 = -0.883 	C22  = 0.038

The ellipsoid (B - Bo) C-1 (B – B0)T ≤1   can then be constructed as

shown

--------------------------------------

*Note: the printout format is:

COVARIANCE (FIT xmin2/N

C11

Example:

Interpretation:

So long as Bl and B2 fall inside the shaded. area (this is the tolerance

requirement), then the ellipsoid representing the corresponding deviations of the

matrix elements R12 and R34 is using Eq. (5):

0-R120.0052+0-R340.0052≤xmin2+ 1.0=2.6

or

R122+R342≤2.6(0.005)2

Note that it is not enough to prescribe tolerances ΔB1<.079   and ΔB2<.038, since there is unshaved area inside the rectangle defined by these values. The strongly tilted covariance ellipse (i.e., r12~ 1) suggests that the triplet

power supplies should be designed so that any drift in magnetic field B1 causes a compensating drift in the magnetic field B2 so as to stay inside the shaded area

shown in Figure l.

For a more complete description of the mathematical basis of TRANSPORT,

refer to SLAC—75 l‘), and to other references listed at the end of this

manual

See the Appendix of this report, or the Appendix of Ref. 5, for a

derivation of these statements.